# 3GPP TSG-RAN WG4 Meeting #112                                R4-2411895

# Maastricht, Netherlands, 19th-23th Aug, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on system parameters for LP-WUS/WUR

# Agenda Item:   8.22.2.1

# Document for:  Approval

- Introduction

In this contribution, we provided out view on system parameters for LP-WUS/WUR.

- Discussion

## 2.1 Channel Raster

In RAN4#111 meeting, there is no consensus achieved on channel raster for LP-WUR[1]. RAN4 needs to further check whether channel raster is needed for LP-WUR.

Issue 1-1-7: Channel raster for LP-WUR Agreements: RAN4 further check whether no channel raster is needed for LP-WUR.

Whether channel raster is needed depends on whether LP-WUR needs to search LP-WUS. In general scenarios, it depends on whether the frequency location of LP-WUS is indicated by NW. If not, channel raster is needed for LP-WUR to search the LP-WUS in LO. We need to wait for RAN1’s conclusion on whether NW will indicate the frequency location of LP-WUS. In other special scenarios, such as that MR has been sleeping for a long time and has been out of synchronization with NW, LP-WUR may also need to re-search the LP-WUS by itself. In this case, channel raster is needed. We should further check in which scenarios that LP-WUR needs to search LP-WUS.

Proposal 1: If LP-WUR needs to search LP-WUS by itself, channel raster is needed. Further check in which scenarios LP-WUR needs to search LP-WUS.

## 2.2 Metric

Regarding metric for LP-WUR requirements, MDR and FAR are two commonly used metrics and are under discussion[1].

Issue 1-1-11: Metric for LP_WUR requirements Agreements: - MDR is used as the metric for LP_WUR Rx RF requirements - FFS MDR values. - FAR will be defined as demodulation requirements - FFS FAR values.

For the values of MDR and FAR, we prefer 1% MDR as the metric for LP-WUR Rx RF requirements and 0.1% FAR as the metric for LP-WUR demodulation requirements.

Proposal 2: Set 1% MDR as the metric for LP-WUR Rx RF requirements and 0.1% FAR as the metric for LP-WUR demodulation requirements.

- Conclusion

Proposals:

Proposal 1: If LP-WUR needs to search LP-WUS by itself, channel raster is needed. Further check in which scenarios LP-WUR needs to search LP-WUS.

Proposal 2: Set 1% MDR as the metric for LP-WUR Rx RF requirements and 0.1% FAR as the metric for LP-WUR demodulation requirements.

- Reference

- R4-2410569 WF for [111][136] NR_LPWUS_UERF, RAN4#111, Fukuoka, Japan, May 2024.