# 3GPP TSG-RAN WG4 Meeting #112                                R4-2411896

# Maastricht, Netherlands, 19th-23th Aug, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on REFSENS, ASCS, ACS for LP-WUR

# Agenda Item:   8.22.2.2

# Document for:  Approval

- Introduction

In RAN4#111 meeting, RAN4 achieved an agreement on the simulation assumption of LP-WUS LLS for companies to do some simulation work[1]. In this contribution, we provided some preliminary simulation results and discussed the issues left.

- Discussion

## 2.1 ASCS and ACS Evaluation

### 2.1.1 Performance Metric

In last meeting, RAN4 discussed the performance metric (MDR/BLER value) for LLS simulation that apply to ACS/ASCS and REFSENS as follows[1].

Issue 2-1-2: Performance metric (MDR/BLER value) for LLS simulation (apply to ACS/ASCS and REFSENS) Agreement: - For LLS simulation (apply to ACS/ASCS and REFSENS), the metric includes - 1% MDR/BLER as baseline and 5% MDR/BLER as optional - The following false alarm rate can be considered - 1% - 5% - Providing the information whether the false alarm rate is considered or not - Further down-select the performance metric for the requirements and testing

Usually 1% MDR/BLER as performance metric is common understanding that is used in both RAN1 and RAN4. We understand that 5% comes from 95% throughput for traditional NR transmission. However, LP-WUS is quite different from traditional NR PDSCH/PDCCH in signal design, data size and receiver. In this case, it may be unreasonable to consider the same metric value as NR PDSCH/PDCCH.

Proposal 1: 1% BLER/MDR is preferred, and 5% BLER/MDR may be too high for LP-WUS.

Another issue is whether FAR should be considered in LLS. In our understanding, in practical environment, for a given UE, false alarm happens when the UE thinks it detects the WUS for itself or its subgroup but actually the WUS is not transmitted at all. This wrong WUS may come from AWGN or a WUS for any other UE/subgroups. However, in LLS, WUS for other UE/subgroup can not be simulated, where the wrong WUS can only come from AWGN. In this case, FAR is highly related to the bit length of the WUS (including both information bits and CRC bits). For example, if WUS consists of 8 information bits and 8 CRC bits, then FAR equals to 1/2^16 theoretically. If such a low FAR need to be simulated, an incredibly massive simulation processes are required, which is unacceptable.

Therefore, we don’t think it is necessary to consider FAR in LLS. As for practical FAR test, a more complex electromagnetic environment with all kinds of interference exiting should be considered and FAR requirement should be further checked.

Proposal 2: There is no need to consider FAR in LLS because it is low enough, but for practical test, FAR should be further checked.

### 2.1.2 ADC Assumption

In last meeting, companies are encouraged to provide simulation results with both 8-bit ADC and 4-bit ADC. More ADC bits means more consumption. Based on our simulation results in Fig 1, there is little performance improvement for 8-bit ADC compared to 4-bit ADC, and the performance is near to ideal ADC. Therefore, we suggest that 4-bit ADC is enough for LLS.

Fig. 1  The effect of ADC bit on BLER.

Proposal 3: 4-bit ADC is enough for LP-WUS LLS.

### 2.1.3 ASCS and Guard RB

According to the definition of ASCS proposed in last meeting, ASCS is defined as the ratio of the receive filter attenuation on the assigned LP-WUR channel frequency to the receive filter attenuation on the adjacent NR subcarrier[1]. The related agreements are as follows.

Issue 2-3-1: Align the definition of ASCS requirements Agreement: - RAN4 can use the following ASCS definition as starting point. - Adjacent SubCarrier Selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its assigned channel frequency in the presence of adjacent subcarrier NR signal(s) at a given frequency offset (guard RB) between LP-WUS and NR. ASCS is the ratio of the receive filter attenuation on the assigned LP-WUR channel frequency to the receive filter attenuation on the adjacent NR subcarrier. Issue 2-1-8: For ASCS, the BWinterferer for ASCS evaluation Agreement: - For ASCS evaluation, only 10MHz and 20MHz NR CBWs are assumed and the BWinterferer is - All RBs between WUS edge to channel edge Issue 2-3-3: Required number of guard RB for ASCS Agreement: - RAN4 further discuss required number of guard RB for ASCS based on LLS simulation outcome next meeting.

According to WF, we share some simulation results on the guard RB evaluation for ASCS as follows under 5th filter order and WUS location at center. Fig. 2 show the impacts of guard RBs on BLER for CBW=10MHz and CBW=20MHz, respectively.

Fig. 2  LP-WUR performance in ASCS case for CBW=10MHz and CBW=20MHz

We can find that with 1 guard RB, there is little improvement on the required SNR to reach 1% BLER for given waveform compared to 0 guard RB.

Observation 1: With 1 guard RB, there is little improvement on the required SNR to reach 1% BLER for given waveform compared to 0 guard RB.

Proposal 4: Based on our simulation results, we propose no guard RB is needed for ASCS.

Moreover, we need to determine ASCS requirements for LP-WUR. With the agreed definition of ASCS, if WUS is located at the center of NR carrier with NR RBs on both sides for ASCS evaluation, the filter attenuation of all the NR RBs should be considered. In this case, for different CBW, the numbers of NR RBs to measure filter attenuation are different. For example, 6RB+5RB for CBW=10MHz and 19RB+19RB for CBW=20MH, as shown in Fig. 3.

Fig. 3  Filter attenuation for LP-WUS and NR signal under different CBW

Besides, WUS location will also influence ASCS. Different from WUS at center, for the case that WUS is located at the edge of the NR carrier, the filter attenuation for all the NR RBs in one side should be considered. For example, 19RB+19RB for WUS at center and 39RB for WUS at edge as shown in Fig. 4. It is obvious that the attenuation is different between these two cases with different LP-WUS locations.

Fig. 4  Filter attenuation for LP-WUS with different location within NR carrier under CBW=20MHz

We show the ASCS under different guard RB numbers, filter orders and residual frequency error for CBW=10MHz and 20MHz in Table 1 and Table 2, respectively. In Table 3, we show the ASCS results for the case that WUS is located at edge of the carrier for CBW=20MHz.

Table 1 ASCS results for CBW=10MHz (WUS at center)

| Filter order | Waveform |  | OOK-1 |  | OOK-4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | M = 1 |  | M = 2 |  | M = 4 |  |
|  | Guard RB CBW | Residual Frequency Error | 0RB | 1RB | 0RB | 1RB | 0RB | 1RB |
| 5th | 10MHz(24RB) SCS = 30kHz | 0 ppm | -11.60 | -18.05 | -11.54 | -18.54 | -11.52 | -19.03 |
|  |  | 10 ppm | -11.59 | -18.03 | -11.52 | -18.52 | -11.49 | -19.01 |
|  |  | 20 ppm | -11.56 | -17.98 | -11.47 | -18.47 | -11.43 | -18.96 |
| 3rd | 10MHz(24RB) SCS = 30kHz | 0 ppm | -8.94 | -12.15 | -8.90 | -12.38 | -8.90 | -12.63 |
|  |  | 10 ppm | -8.94 | -12.15 | -8.89 | -12.37 | -8.89 | -12.62 |
|  |  | 20 ppm | -8.93 | -12.13 | -8.87 | -12.36 | -8.86 | -12.60 |

Table 2 ASCS results for CBW=20MHz (WUS at center)

| Filter order | Waveform |  | OOK-1 |  | OOK-4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | M = 1 |  | M = 2 |  | M = 4 |  |
|  | Guard RB CBW | Residual Frequency Error | 0RB | 1RB | 0RB | 1RB | 0RB | 1RB |
| 5th | 20MHz(51RB) SCS = 30kHz | 0 ppm | -16.51 | -23.60 | -16.42 | -24.12 | -16.43 | -24.85 |
|  |  | 10 ppm | -16.50 | -23.58 | -16.39 | -24.10 | -16.40 | -24.83 |
|  |  | 20 ppm | -16.45 | -23.55 | -16.37 | -24.07 | -16.38 | -24.80 |
| 3rd | 20MHz(51RB) SCS = 30kHz | 0 ppm | -13.74 | -17.49 | -13.67 | -17.82 | -13.70 | -18.13 |
|  |  | 10 ppm | -13.74 | -17.49 | -13.66 | -17.81 | -13.69 | -18.13 |
|  |  | 20 ppm | -13.72 | -17.48 | -13.65 | -17.80 | -13.68 | -18.12 |

Table 3 ASCS results for CBW=20MHz (WUS at edge)

| Filter order | Waveform |  | OOK-1 |  | OOK-4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | M = 1 |  | M = 2 |  | M = 4 |  |
|  | Guard RB CBW | Residual Frequency Error | 0RB | 1RB | 0RB | 1RB | 0RB | 1RB |
| 5th | 20MHz(51RB) SCS = 30kHz | 0 ppm | -19.55 | -26.78 | -19.39 | -27.35 | -19.37 | -28.13 |
|  |  | 10 ppm | -19.14 | -26.34 | -18.97 | -26.91 | -18.95 | -27.70 |
|  |  | 20 ppm | -18.87 | -26.07 | -18.75 | -26.63 | -18.74 | -27.41 |
| 3rd | 20MHz(51RB) SCS = 30kHz | 0 ppm | -16.77 | -20.69 | -16.67 | -20.99 | -16.69 | -21.41 |
|  |  | 10 ppm | -16.54 | -20.46 | -16.43 | -20.77 | -16.46 | -21.20 |
|  |  | 20 ppm | -16.39 | -20.32 | -16.31 | -20.61 | -16.34 | -21.04 |

Based on our simulation results, we find that 3rd filter order has much less suppression on sub-carrier interference than 5th filter order, and ASCS is related to several factors, including filter orders, the number of guard RBs and WUS location. Comparing Table 1 and Table 2, ASCS value highly depends on NR CBW and should be further studied for all available CBW after the number of guard RBs for ASCS is determined. Comparing Table 2 and Table 3, ASCS value for the case that WUS is at edge of the NR carrier is about 2~3 dB better than that for WUS at center. Considering filter suppression characteristic, WUS at center can be seen as the worst case to determine ASCS requirements.

Observation 2: 3rd filter order has much less suppression on sub-carrier interference than 5th filter order.

Observation 3: Based on the definition of ASCS, the receive filter attenuation on the adjacent NR subcarrier depends on CBW, WUS location and filter characteristic.

As shown in Table 4, ASCS results are different under different CBW, taking 20ppm frequency error, 0 guard RB, 30kHz SCS and waveform OOK-4 with M=4 as example.

Table 4 ASCS results for different CBW (WUS at center)

| CBW(MHz) | SCS(kHz) | NRB | ASCS(dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Filter order = 3 | Filter order = 5 |
| 5 | 30 | 11 | - | - |
| 10 | 30 | 24 | -8.86 | -11.43 |
| 15 | 30 | 38 | -11.96 | -14.66 |
| 20 | 30 | 51 | -13.68 | -16.38 |
| 25 | 30 | 65 | -14.98 | -17.65 |
| 30 | 30 | 78 | -15.92 | -18.61 |
| 35 | 30 | 92 | -16.71 | -19.45 |
| 40 | 30 | 106 | -17.41 | -20.12 |
| 45 | 30 | 119 | -17.95 | -20.65 |
| 50 | 30 | 133 | -18.49 | -21.20 |
| 60 | 30 | 162 | -19.41 | -22.11 |
| 70 | 30 | 189 | -20.13 | -22.82 |
| 80 | 30 | 217 | -20.77 | -23.48 |
| 90 | 30 | 245 | -21.31 | -24.01 |
| 100 | 30 | 273 | -21.80 | -24.49 |

In this case, we should determine ASCS requirements for all available CBW under the worst case that WUS is located at center the NR carrier.

Proposal 5: Determine ASCS requirements for all available CBW under the worst case that WUS is located at center of the NR carrier.

### 2.1.4 ACS and Guard RB

According to [1], ACS is defined as the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s) as starting point as follows.

Issue 2-4-1: Whether to update the ACS definition for LP-WUS Agreement: - RAN4 consider the following clarified ACS requirements definition for LP-WUS as starting point. - Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR LP-WUS signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the center frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

This definition is reused from legacy NR ACS requirements, where the receive filter attenuation on the assigned channel frequency considers all the different CBW for NR transmission. Different from legacy ACS requirements for NR that depends on CBW, there is no need to consider CBW for ACS of LP-WUR because LP-WUS only occupies part (11RB for 30kHz SCS) of the CBW and the filter pass bandwidth also matches fixed bandwidth (3.96MHz) for LP-WUS, as shown in Fig 5, no matter how wide the CBW is.

Fig. 5  Filter attenuation for LP-WUS and adjacent carrier interference

Observation 4: There is no need to consider different CBW for ACS of LP-WUR because LP-WUS occupies fixed 3.96MHz and the filter pass bandwidth also matches the fixed 3.96MHz for LP-WUS.

Therefore, to distinguish ACS for LP-WUR and legacy ACS for NR, we prefer to change “assigned channel frequency” to “assigned LP-WUR channel frequency” just like that in the definition of ASCS.

Proposal 6: Change “assigned channel frequency” to “assigned LP-WUR channel frequency” in the definition of ACS as follows:

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive a LP-WUS signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the center frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned LP-WUR channel frequency to the receive filter attenuation on the adjacent channel(s).

For LLS of ACS and guard RB, although REFSENS for LP-WUR has not been decided yet, we provide some preliminary simulation results setting the EPRE ratio between adjacent channel interference and LP-WUS as 31.5dB in Fig 6.

Fig. 6  LP-WUR performance in ACS case

Observation 5: To achieve 1% BLER, 2 guard RBs are necessary for OOK-4 waveform and it is relatively easy for OOK-1 waveform to achieve 1% BLER with adjacent channel interference.

Proposal 7: For ACS, 2 guard RBs are needed and guard RBs can be blank or filled with NR signal.

We show the ACS results under different filter orders, guard RBs and waveform in Table 5. 3rd filter order has much less suppression on adjacent channel interference than 5th filter order. As the number of guard RBs affects ACS results a lot, we prefer to further study ACS after the number of guard RBs for ACS is determined.

Table 5 ACS results with 5MHz adjacent carrier interference (WUS at edge)

| Filter order | Waveform | OOK-1 |  |  |  | OOK-4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | M = 1 |  |  |  | M = 2 |  |  |  | M = 4 |  |  |  |
|  | Residual Frequency error Guard RB | 1RB | 2RB | 3RB | 4RB | 1RB | 2RB | 3RB | 4RB | 1RB | 2RB | 3RB | 4RB |
| 5th | 0 ppm | -36.64 | -40.47 | -44.01 | -47.24 | -36.58 | -40.39 | -43.95 | -47.20 | -36.74 | -40.56 | -44.10 | -47.33 |
|  | 10 ppm | -36.49 | -40.33 | -43.89 | -47.12 | -36.43 | -40.25 | -43.81 | -47.07 | -36.59 | -40.43 | -43.97 | -47.21 |
|  | 20 ppm | -36.47 | -40.30 | -43.86 | -47.10 | -36.39 | -40.21 | -43.77 | -47.03 | -36.53 | -40.37 | -43.91 | -47.16 |
| 3rd | 0 ppm | -23.33 | -25.49 | -27.49 | -29.34 | -23.26 | -25.42 | -27.44 | -29.30 | -23.38 | -25.54 | -27.54 | -29.39 |
|  | 10 ppm | -23.25 | -25.41 | -27.43 | -29.27 | -23.18 | -25.34 | -27.36 | -29.22 | -23.30 | -25.46 | -27.47 | -29.31 |
|  | 20 ppm | -23.23 | -25.39 | -27.41 | -29.25 | -23.15 | -25.31 | -27.33 | -29.20 | -23.27 | -25.43 | -27.43 | -29.28 |

Observation 6: 3rd filter order has much less suppression on adjacent channel interference than 5th filter order.

Proposal 8: Determine ACS for LP-WUR after the number of guard RBs is determined.

## 2.2 REFSENS

For REFSENS requirements, RAN4 reached some agreements in last meeting as follows[1].

Issue 2-2-3: SNR value (not requirement) for REFSENS Agreement: - Stick to last meeting agreements: Wait for RAN1 SNR progress Issue 2-2-4: NF Gap between LR and MR for REFSENS (assume MR as 9dB) Agreement: - RAN4 further discuss potential NF for OOK-based and OFDM-based receiver. - Linked to resolution of Issue 2-2-8 Issue 2-2-6: IM value for REFSENS Agreement: - RAN4 further discuss potential IM value for OOK-based and OFDM-based receiver.

Usually, REFSENS for NR UE is expressed as

REFSENS(dBm) = kT0 + 10log10(NRB*12*SCS) + NF + SNR + IM

where some other related parameters are not included for LP-WUS, such as diversity gain, because compared to the main receiver, LP-WUR architecture is a separate Rx chain without the corresponding diversity Rx chain, i.e. 1Rx architecture, so there is no need to consider diversity gain. Therefore, to derive REFSENS, we need to decide NF and IM (for SNR, it is agreed to wait for RAN1 SNR progress).

### 2.2.1 Noise Figure

Regarding the NF values, in RAN1#116bis meeting, they achieved a conclusion to consider all three values +2dB, +5dB, +8dB on top of NF of MR (7dB), as assumptions to derive the SNR to achieve the coverage of PUSCH for Message 3[3].

(RAN1) Conclusion: For calibration purposes, companies are encouraged to report the SNR to achieve the coverage of PUSCH for message3, at least with the following assumptions: - Carrier frequency: 2.6 GHz - The number of Tx chains: 1 - MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE - Transmit antenna gain correction factors for WUS: up to company report - Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately

In RAN4, we can follow the assumptions in RAN1, i.e., +2 dB and +5 dB on top of NF of MR (9dB) for OFDM-based receiver and OOK-based receiver, respectively (+8 dB is used for high NF architectures, such as RF ED architecture).

Proposal 9: NF Gap between LR and MR for REFSENS could be +2dB and +5dB for OFDM-based receiver and OOK-based receiver, respectively.

### 2.2.2 IM Value

For NR MR, IM is ~2.5dB considering implementation issues. For LR, the architecture is simpler than and power consumption is less than MR, we think 2 dB is okay for LR implementation.

Proposal 10: 2dB IM is okay for LR.

### 2.2.3 Baseline Architecture

In last meeting, many proposals on the baseline architecture are provided from companies as follows and RAN4 needs to further discuss and decide baseline architecture(s) for OOK-based receiver and OFDM-based receiver[1].

Issue 2-2-8: Whether a baseline architecture is needed for LP-WUS receiver Agreement: - RAN4 will further discuss and decide baseline architecture(s) for OOK-based receiver and OFDM-based receiver.

First of all, considering that the reported noise figure is in the range of 12~22 dB for architecture with RF envelope detection, which would cause terrible sensitivity/coverage. Also considering it cannot support multiple band operation, it is proposed to exclude RF ED architecture for OOK-based LP-WUR. For the left two OOK-based receiver architecture, heterodyne architecture with IF envelope detection and homodyne/zero-IF architecture with baseband envelope detection, zero-IF architecture is preferred as baseline because of its less power consumption and complexity.

Proposal 11: Exclude RF envelope detection architecture for LP-WUR.

Proposal 12: Use zero-IF architecture as baseline architecture for OOK-based LP-WUR.

For OFDM-based LP-WUR, time-domain correlation and frequency-domain correlation are still under discussion in RAN1. Besides, RAN1 has not decided how to overlay OFDM sequences on the OOK patterns and whether the OFDM sequences will carrier information yet. In this case, it is too early to consider OFDM-based LR and determine some requirements in RAN4. We suggest to wait for RAN1’s conclusions.

Proposal 13: To wait for RAN1’s conclusion on the OFDM-based LP-WUR.

- Conclusion

Observations:

Observation 1: With 1 guard RB, there is little improvement on the required SNR to reach 1% BLER for given waveform compared to 0 guard RB.

Observation 2: 3rd filter order has much less suppression on sub-carrier interference than 5th filter order.

Observation 3: Based on the definition of ASCS, the receive filter attenuation on the adjacent NR subcarrier depends on CBW, WUS location and filter characteristic.

Observation 4: There is no need to consider different CBW for ACS of LP-WUR because LP-WUS occupies fixed 3.96MHz and the filter pass bandwidth also matches the fixed 3.96MHz for LP-WUS.

Observation 5: To achieve 1% BLER, 2 guard RBs are necessary for OOK-4 waveform and it is relatively easy for OOK-1 waveform to achieve 1% BLER with adjacent channel interference.

Observation 6: 3rd filter order has much less suppression on adjacent channel interference than 5th filter order.

Proposals:

Proposal 1: 1% BLER/MDR is preferred, and 5% BLER/MDR may be too high for LP-WUS.

Proposal 2: There is no need to consider FAR in LLS because it is low enough, but for practical test, FAR should be further checked.

Proposal 3: 4-bit ADC is enough for LP-WUS LLS.

Proposal 4: Based on our simulation results, we propose no guard RB is needed for ASCS.

Proposal 5: Determine ASCS requirements for all available CBW under the worst case that WUS is located at center of the NR carrier.

Proposal 6: Change “assigned channel frequency” to “assigned LP-WUR channel frequency” in the definition of ACS as follows:

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive a LP-WUS signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the center frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned LP-WUR channel frequency to the receive filter attenuation on the adjacent channel(s).

Proposal 7: For ACS, 2 guard RBs are needed and guard RBs can be blank or filled with NR signal.

Proposal 8: Determine ACS for LP-WUR after the number of guard RBs is determined.

Proposal 9: NF Gap between LR and MR for REFSENS could be +2dB and +5dB for OFDM-based receiver and OOK-based receiver, respectively.

Proposal 10: 2dB IM is okay for LR.

Proposal 11: Exclude RF envelope detection architecture for LP-WUR.

Proposal 12: Use zero-IF architecture as baseline architecture for OOK-based LP-WUR.

Proposal 13: To wait for RAN1’s conclusion on the OFDM-based LP-WUR.

- Reference

- R4-2410569 WF for [111][136] NR_LPWUS_UERF, RAN4#111, Fukuoka, Japan, May 2024.

- 38.869, v18.0.0

- RAN1 Chairman notes, RAN1#116bis, Changsha, China, Apr. 2024.