# 3GPP TSG-RAN WG4 Meeting #112                                R4-2411897

# Maastricht, Netherlands, 19th-23th Aug, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on receiver characteristics for LP-WUR

# Agenda Item:   8.22.2.3

# Document for:  Approval

- Introduction

In this contribution, we provided our view on receiver characteristics for LP-WUR.

- Discussion

## 2.1 IBB and OBB

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies specified for IBB and OBB in TS38.101-1. The left issues in last meeting on IBB and OBB for LP-WUR are listed as follows[1].

Issue 3-1-1: IBB and OBB assumption Agreement: - RAN4 further check and discuss whether same interference level of IBB and OBB as MR is assumed for LP-WUR. - With assumption that Rx dynamic range for LR may not be as good as MR Issue 3-1-2: IBB and OBB requirements Agreement: - RAN4 further discuss detailed test condition for IBB and OBB, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level. - FFS IBB and OBB requirements

LP-WUR an MR may share the same antenna and baseband processor, which means that LP-WUR and MR will suffer the same level of interference. In our understanding, same interference level of IBB and OBB as MR can be assumed for LP-WUR.

Proposal 1: Same interference level of IBB and OBB as MR can be assumed for LP-WUR.

However, it is known that the performance metrics of LP-WUR and MR are different. For MR, the performance metric is throughput and it shall be ≥95% of the maximum throughput of the reference measurement channels with specified assumptions, including interferer BW, interferer transmission power, interferer frequency offset, etc.[2]. While for LP-WUR, the performance metric is BLER/MDR, which shall be ≤X% but the value has not been decided yet. Besides, the architecture of LP-WUR is simpler than MR and receiving mechanism based on envelope detection and time/frequency-domain correlation is quite different from MR.

With all the concerns above, we do not think we can determine the test conditions for IBB and OBB easily. It needs to be evaluated carefully after related basic issues are finished, including LP-WUS design, performance metrics, guard RB requirements, REFSENS evaluation, etc..

Proposal 2: Postpone the discussion on the test conditions for IBB and OBB until related basic issues are finished, including LP-WUS design, performance metrics, guard RB requirements, REFSENS evaluation, etc..

## 2.2 Intermodulation requirements

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal. The main issue for intermodulation requirements for LP-WUR is the detailed test condition[1].

Issue 3-1-4: Intermodulation requirements Agreement: - RAN4 further check and discuss detailed test condition for Intermodulation requirements, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level. - FFS Intermodulation requirements

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively in TS38.101-1[2]. The throughput shall be ≥95% of the maximum throughput of the reference measurement channels under these assumptions. Similar to the discussion in Section 2.1, the discussion on detailed test condition for intermodulation requirements should be postponed.

Proposal 3: Postpone the discussion on the test conditions for intermodulation requirements until related basic issues are finished, including LP-WUS design, performance metrics, guard RB requirements, REFSENS evaluation, etc..

## 2.3 Spurious response

Spurious response is also used to evaluate the receiver capability when there is presence of an unwanted CW interfering signal on the frequency where out-of-band blocking limit is not met. Spurious response requirement is needed for LP-WUR.

Proposal 4: Spurious response requirement is needed for LP-WUR.

- Conclusion

Proposals:

Proposal 1: Same interference level of IBB and OBB as MR can be assumed for LP-WUR.

Proposal 2: Postpone the discussion on the test conditions for IBB and OBB until related basic issues are finished, including LP-WUS design, performance metrics, guard RB requirements, REFSENS evaluation, etc..

Proposal 3: Postpone the discussion on the test conditions for intermodulation requirements until related basic issues are finished, including LP-WUS design, performance metrics, guard RB requirements, REFSENS evaluation, etc..

Proposal 4: Spurious response requirement is needed for LP-WUR.

- Reference

- R4-2410569 WF for [111][136] NR_LPWUS_UERF, RAN4#111, Fukuoka, Japan, May 2024.

- TS38.101-1, v18.6.0