# 3GPP TSG-RAN WG4 Meeting #112                                R4-2411899

# Maastricht, Netherlands, 19th-23th Aug, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Systems parameters and RF requirements of n87 and n88

# Agenda Item:   7.15.2

# Document for:  Approval

- Introduction

In RAN#104 meeting, a Rel-19 WI on introduction of new NR FDD bands n87 and n88 was approved in [1].

The core objectives of this WI are: - Standardization of two new NR FDD bands n87 and n88 in the 410 MHz range based on - E-UTRA band 87 (UL 410 - 415, DL 420 – 425 MHz) - E-UTRA band 88 (UL 412 - 417, DL 422 – 427 MHz) - During the work evaluate potential impacts of NR. If any impact on performance or any changes w.r.t. existing requirements of band 87 and 88. are found, address those during the work. - Specify band numbering (i.e. n87 and n88) system parameters and RF characteristics for 3 and 5 MHz channel bandwidth of the new bands. - 15 kHz SCS for 3MHz and 5MHz channel bandwidth - Only symmetric bandwidths are supported. - Address potential BS and UE co-existence issues, if any. - Specify the UE RF requirements for the new bands based on 2 Rx operation. - Update the related technical specifications to include support for the new band Note: These 2 new NR bands will be introduced in a REL-independent way starting from REL-15.

Standardization of these two new NR bands is based on E-UTRA band 87 and band 88. In this contribution, we provide some system parameters and RF requirements of these two new bands that need to be included to Rel-19 specification.

- Discussion

## 2.1 System parameters

The inclusion of bands n87 and n88 within the 5G NR operating bands will allow bands n87 and n88 to deploy advanced 5G NR services and applications in additional low frequency spectrum and contribute to extend the range of deployment[1]. In this case, NR bands n87 and n88 should be included in Rel-19 spec in order to guide 5G NR devices to support these two new bands.

Introduction of NR bands n87 and n88 will influence the following system parameters in existing specification.

- Operating band

- As LTE refarming bands, n87 and n88 should be defined in operating bands in FR1 as in Table 2-1.

Table 2-1  NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n87 | 410 MHz – 415 MHz | 420 MHz – 425 MHz | FDD |
| n88 | 412 MHz – 417 MHz | 422 MHz – 427 MHz | FDD |

- Channel bandwidth

- According to the objectives in WI, we can require the channel bandwidth for bands n87 and n88 in Table 2-2.

Table 2-2  Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n87 | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n88 | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Channel raster

- Applicable NR-ARFCN for new operating bands n87 and n88 are listed in Table 2-3 and Table 2-4.

Table 2-3  Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n87 | 100 | 82000 – <20> – 83000 | 84000 – <20> – 85000 |
| n88 | 100 | 82400 – <20> – 83400 | 84400 – <20> – 85400 |

Table 2-4  Applicable NR-ARFCN per operating band for enhanced channel raster

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| n87 | 10 | 82000 – <2> – 83000 | 84000 – <2> – 85000 |  |
| n88 | 10 | 82400 – <2> – 83400 | 84400 – <2> – 85400 |  |

- Synchronization raster

- Only 15kHz SS block SCS is supported in bands n87 and n88, and applicable SS raster for both 5MHz channel bandwidth and 3MHz channel bandwidth are listed in Table 2-4 and Table 2-5, respectively.

- For 5MHz channel bandwidth:

Table 2-5  Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n87 | 15 kHz | Case A | 1055 – <1> – 1057 |
| n88 | 15 kHz | Case A | 1061 – <1> – 1062 |

- For 3MHz channel bandwidth:

Table 2-6  Applicable SS raster entries per operating band for 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n87 | 15 kHz | Case A | 28743 – <1> – 28754 |
| n88 | 15 kHz | Case A | 28752 – <1> – 28764 |

- TX–RX frequency separation

- The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for bands n87 and n88 is derived from uplink operating band and downlink operating band, which is same as that in LTE.

Table 2-7  UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n87 | 10 MHz |
| n88 | 10 MHz |

Proposal 1: Approve Table 2-1 to Table 2-7 on system parameters for new Rel-19 specification.

## 2.2 RF requirements

- Maximum output power

- Generally PC3 should be supported as in Table 2-8.

Table 2-8  UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n87 |  |  |  |  |  |  | 23 | ±2 |
| n88 |  |  |  |  |  |  | 23 | ±2 |

- REFSENS

For REFSENS, we prefer to reuse the REFSENS for E-UTRA band 87 and 88.

Table 2-9  Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n87 | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n88 | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |

Proposal 2: Approve Table 2-8 and Table 2-9 on RF requirements for new Rel-19 specification.

- Blocking characteristics

For IBB and OBB which are band specific, we need to add n87 and n88 into FDL_high < 2700 MHz and FUL_high < 2700 MHz tables. For NBB which is also band specific, we need to add n87 and n88 into the NBB table.

Proposal 3: Add n87, n88 into FDL_high < 2700 MHz and FUL_high < 2700 MHz tables for IBB and OBB, and add n87, n88 into NBB table for new Rel-19 specification.

- Conclusion

Proposal 1: Approve Table 2-1 to Table 2-7 on system parameters for new Rel-19 specification.

Proposal 2: Approve Table 2-8 and Table 2-9 on RF requirements for new Rel-19 specification.

Proposal 3: Add n87, n88 into FDL_high < 2700 MHz and FUL_high < 2700 MHz tables for IBB and OBB, and add n87, n88 into NBB table for new Rel-19 specification.

- Reference

- RP-241663 New WID on introduction of NR bands n87 and n88.