# 3GPP TSG-RAN WG4 Meeting #112                                R4-2411900

# Maastricht, Netherlands, 19th-23th Aug, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Systems parameters and RF requirements of n68

# Agenda Item:   7.16.2

# Document for:  Approval

- Introduction

In RAN#104 meeting, a Rel-19 WI on introduction of new NR FDD bands n68 was approved in [1].

The core objectives of this WI are: - Specify a new NR FDD operating band n68 (the uplink band for this new band is 30 MHz, 698-728 MHz; the downlink band is 30 MHz, 753-783 MHz; the duplex gap is 25 MHz, 728-753 MHz) to support subcarrier spacing of 15 kHz for 5 MHz channel bandwidth, and subcarrier spacing of 15 kHz and 30 kHz for 10 and 15 MHz channel bandwidths. - Only symmetric bandwidths are supported. - Specify system parameters and RF characteristics of the new band. - Address potential BS and UE co-existence issues, if any. - Update the related technical specifications to include support for the new band - Specify the UE RF requirements for the new bands based on 2 Rx operation. Note: This new NR band will be introduced in a REL-independent way starting from REL-15.

Standardization of NR band n68 is based on E-UTRA band 68. In this contribution, we provide some system parameters and RF requirements of this new band that need to be included to Rel-19 specification.

- Discussion

## 2.1 System parameters

The inclusion of band n68 within the 5G NR operating bands will allow band n68 to deploy advanced 5G NR services and applications in additional low frequency spectrum and contribute to extend the range of deployment[1]. In this case, band n68 should be included in Rel-19 spec in order to guide 5G NR devices to support this new NR band.

Introduction of NR bands n68 will influence the following system parameters in existing specification.

- Operating band

- As LTE refarming bands, n68 should be defined in operating bands in FR1 as in Table 2-1.

Table 2-1  NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n68 | 698 MHz – 728 MHz | 753 MHz – 783 MHz | FDD |

- Channel bandwidth

- According to the objectives in WI, we can require the channel bandwidth for band n68 in Table 2-2.

Table 2-2  Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n68 | 15 |  | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Channel raster

- Applicable NR-ARFCN for new operating bands n68 is listed in Table 2-3 and Table 2-4.

Table 2-3  Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n68 | 100 | 139600 – <20> – 145600 | 150600 – <20> – 156600 |

Table 2-4  Applicable NR-ARFCN per operating band for enhanced channel raster

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| n68 | 10 | 139600 – <2> – 145600 | 150600 – <2> – 156600 |  |

- Synchronization raster

- The applicable SS raster for n68 is listed in Table 2-4.

Table 2-5  Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n68 | 15 kHz | Case A | 1888 – <1> – 1951 |

- TX–RX frequency separation

Table 2-6  UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n68 | 55 MHz |

Proposal 1: Approve Table 2-1 to Table 2-6 on system parameters for new Rel-19 specification.

## 2.2 RF requirements

- Maximum output power

- Generally PC3 should be supported as in Table 2-7.

Table 2-7  UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n68 |  |  |  |  |  |  | 23 | ±2 |

- REFSENS

The proposed REFSENS is listed in Table 2-8.

Table 2-8  Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n68 | 15 |  | -98.5 | -95.3 | -93.5 |  |  |  |  |  |  |  |
|  | 30 |  |  | -95.6 | -93.6 |  |  |  |  |  |  |  |

Proposal 2: Approve Table 2-7 and Table 2-8 on RF requirements for new Rel-19 specification.

- Blocking characteristics

For IBB and OBB which are band specific, we need to add n68 into FDL_high < 2700 MHz and FUL_high < 2700 MHz tables. For NBB which is also band specific, we need to add n68 into the NBB table.

Proposal 3: Add n68 into FDL_high < 2700 MHz and FUL_high < 2700 MHz tables for IBB and OBB, and add n68 into NBB table for new Rel-19 specification.

- Conclusion

Proposal 1: Approve Table 2-1 to Table 2-6 on system parameters for new Rel-19 specification.

Proposal 2: Approve Table 2-7 and Table 2-8 on RF requirements for new Rel-19 specification.

Proposal 3: Add n68 into FDL_high < 2700 MHz and FUL_high < 2700 MHz tables for IBB and OBB, and add n68 into NBB table for new Rel-19 specification.

- Reference

- RP-241664 New WID on introduction of NR band n68.