# 3GPP TSG-RAN WG4 Meeting #112                                R4-2411901

# Maastricht, Netherlands, 19th-23th Aug, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Systems parameters and RF requirements of 1.4GHz band

# Agenda Item:   7.13.2

# Document for:  Approval

- Introduction

In RAN#104 meeting, a Rel-19 WI on introduction of 1.4GHz band was approved in [1]. The WI proposed a FDD band spanning 1390 MHz - 1395 MHz for UL and 1432 - 1435 MHz for DL as follows.

The objective of this work item is to: - Specify a new NR FDD operating band for the US 1.4 GHz allocation spanning 1390-1395 MHz for UL and 1432-1435 MHz for DL. - Specify band numbering and RF characteristics of the 1.4 GHz band. - Specify 3 MHz channel BW for DL and UL in the 1.4 GHz band, with 15 KHz SCS. - Note: This NR band will be introduced in a REL-independent way starting from REL-15.

In this contribution, we provide some system parameters and RF requirements of this new band that need to be included to Rel-19 specification.

- Discussion

## 2.1 System parameters

- Band numbering

First of all, we need to specify band numbering for this new 1.4GHz band in order that it can be introduced in Rel-19 specification.

Proposal 1: The new NR 1.4GHz band can be numbered as n110.

- Operating band

- According to the WI, operating bands in FR1 as in Table 2-1.

Table 2-1  NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| [n110] | 1390 MHz – 1395 MHz | 1432 MHz – 1435 MHz | FDD |

- Channel bandwidth

3MHz channel bandwidth and 15kHz SCS can be supported for this 1.4GHz band.

Table 2-2  Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| [n110] | 15 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Channel raster

- Applicable NR-ARFCN for new 1.4GHz band listed in Table 2-3.

Table 2-3  Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n110] | 100 | 278000 – <20> – 279000 | 286400 – <20> – 287000 |

Table 2-4  Applicable NR-ARFCN per operating band for enhanced channel raster

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| [n110] | 10 | 278000 – <2> – 279000 | 286400 – <2> – 287000 |  |

- Synchronization raster

- Only 15kHz SS block SCS is supported in this new 1.4GHz band, and applicable SS raster for 3MHz channel bandwidth are listed in Table 2-4 .

- For 3MHz channel bandwidth:

Table 2-5  Applicable SS raster entries per operating band for 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n110] | 15 kHz | Case A | 33802 – <1> – 33804 |

- TX–RX frequency separation

- The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for this new 1.4GHz band is derived from uplink operating band and downlink operating band, which is same as that in LTE.

Table 2-6  UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [n110] | 42 MHz |

Proposal 2: Approve Table 2-1 to Table 2-6 on system parameters for new Rel-19 specification.

## 2.2 RF requirements

- Maximum output power

- Generally PC3 should be supported as in Table 2-7.

Table 2-7  UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [n110] |  |  |  |  |  |  | 23 | ±2 |

- REFSENS

Consider define -102.2 dBm REFSENS requirement for this new 1.4GHz band.

Table 2-8  Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| [n110] | 15 | -102.2 |  |  |  |  |  |  |  |  |  |  |

Proposal 3: Approve Table 2-7 and Table 2-8 on RF requirements for new Rel-19 specification.

- Blocking characteristics

For IBB and OBB which are band specific, this new band should be contained in FDL_high < 2700 MHz and FUL_high < 2700 MHz tables. For NBB which is also band specific, this new band should be contained in the NBB table.

Proposal 4: Add this new 1.4GHz band into FDL_high < 2700 MHz and FUL_high < 2700 MHz tables for IBB and OBB, and add this new 1.4GHz band into NBB table for new Rel-19 specification.

- Conclusion

Proposal 1: The new NR 1.4GHz band can be numbered as n110.

Proposal 2: Approve Table 2-1 to Table 2-6 on system parameters for new Rel-19 specification.

Proposal 3: Approve Table 2-7 and Table 2-8 on RF requirements for new Rel-19 specification.

Proposal 4: Add this new 1.4GHz band into FDL_high < 2700 MHz and FUL_high < 2700 MHz tables for IBB and OBB, and add this new 1.4GHz band into NBB table for new Rel-19 specification.

- Reference

- RP-241648 Introduction of the 1.4GHz band.