3GPP TSG-RAN WG4 Meeting#112                              R4-2411930

Maastricht , NL, 19th – 23th Aug, 2024

Agenda item:	8.10.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirements for ATG BS in Rel-19

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID on Enhancements for Air-to-ground network for NR has been approved and it’s expected to start the normative work from this April meeting. During last RAN4 meeting, we have reached good progress for RF requirement for Rel-19 ATG BS supporting CA. In this contribution, we would like to share some further views on the remaining issue.

Issue 1-1: TAE requirements for ATG BS supporting intra-band contiguous CA and inter-band CA - Way Forward - To reuse the existing NR BS TAE requirements - Issue 1-2: How to handle CA TAE requirements for ATG BS in TS38.104 - Way Forward - Option 1: Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3 - Option 2: Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3, then add the following in 6.5.3.2 and 9.6.3.2, - For ATG BS, for intra-band contiguous carrier aggregation, without MIMO, TAE shall not exceed 260ns. - For ATG BS, inter-band carrier aggregation, without MIMO, TAE shall not exceed 3µs. - Option 3: Find a place to add a general clarification that for ATG, intra-band contiguous CA and inter-band CA are supported but DL MIMO is not supported. - Issue 1-3: RF requirements other than TAE for ATG BS supporting intra-band contiguous CA and inter-band CA - Online agreements: - To reuse the same requirements as defined for the legacy NR BS RF CA requirements. - WF reached at RAN4#111 meeting - Issue 1-1: Whether to preclude MIMO for TAE requirements for ATG BS supports CA based on the current WID? - Way Forward： - FFS on whether to preclude MIMO for TAE requirements for ATG BS supports CA. - Issue 1-2: How to handle CA TAE requirements for ATG BS in TS38.104 based on the current WID? - Way Forward： - FFS on the wordings on the TAE requirements for ATG BS CA in the spec.

Discussion

In the last RAN4 meeting, we have reached the consensus on RF requirement for ATG BS supporting intra-band contiguous CA and inter-band CA. Regarding how to capture it in the specification, at least from our understanding, option 1, 2 and 3 is not exclusive for each other.

Firstly, the note for TAE requirement for single carrier operation without MIMO of ATG BS in Rel-18 is not applicable anymore in Rel-19 specification.

Secondly, it’s clearly captured in the WID that ATG BS is supposed to support the intra-band contiguous CA and inter-band DL CA without MIMO transmission. If we just reuse the existing TAE requirement for ATG BS, this will cause some confusions since the wording of TAE requirements in the specification includes both with or without MIMO. Therefore option 2 is somehow preferred and make the specification more clear.

Thirdly, as we discussed during the last RAN4 meeting, for other RF requirement needs to be reused for ATG BS e.g. aggregated measurement bandwidth for OFF power or OBW and CACLR requirement etc. In other words, the option 3 is also essential for other requirements.

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: n79 - FR1+FR1 inter-band CA - Example band combination: n3+n39 - Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4] - Specify signaling and UE capabilities if necessary [RAN2].

Observation 1: According to the current WID, MIMO is mentioned for BS, instead it is for UE.

However, according to the previous discussions, operator would like to request to keep the MIMO, and would like to revise the WID in the RAN plenary. If the WID is revised later to inlcude the MIMO, then in this case there is no problem to include the MIMO as there are no technical issue from BS RF requirements perspective. Before that, we think MIMO should not be included to facilitate the discussion.

Based on the above considerations, we propose:

Proposal 1: MIMO is not included based on the current WID, leave DL MIMO as implementation.

Proposal 2: Whether to include MIMO feature for ATG BS CA depends on the further revised WID.

Conclusions

In this contribution, we shared some further views on the RF requirements for ATG BS supporting CA in Rel-19 and proposals are made as following:

Observation 1: According to the current WID, MIMO is mentioned for BS, instead it is for UE.

Proposal 1: MIMO is not included based on the current WID, leave DL MIMO as implementation.

Proposal 2: Whether to include MIMO feature for ATG BS CA depends on the further revised WID.

References

- RP-240839,New WID on Enhancements for Air-to-ground network for NR, Approved.

- R4-2321028,CR for TS 38.104 on adding RF requirements for ATG BS, ZTE, Agreed

- R4-2406089, Way Forward for [110bis][312] NR_ATG_enh, Approved.

- R4-2409957, Way Forward for [111][312] NR_ATG_enh, Approved.