# 3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411932

# Maastricht, Netherlands, Aug.19th – 23th, 2024

# Source:	ZTE Corporation

# Title:	TP for TR38.719-01-01_CA_n104C

# Agenda Item:	7.3.2

# Document for:	Approval

- Introduction

CA_n104C was requested and included in the new R19 basket WID[1], Hence, we provide a TP to TR38.719-01-01 to introduce intra-band UL CA_n104C.

- Reference

- [1] RP-241674, New WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Ericsson, ZTE, Huawei

- Text Proposal

----- Start of TP -----

  - 5.x			CA_2DL_n104C_2UL_n104C

    - 5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n104C | CA_n104C | 20, 30, 40, 50 | 60, 70, 80, 90, 100 |  |  |  | 200 | 0 |
|  |  | 60, 70, 80, 90 | 60, 70, 80, 90, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | See n104 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  |  | 200 | 4 and 5 |

    - 5.x.2	UE co-existence studies

As stated in clause Table 6.2.3.1-1A in clause 6.2.3.1 in  the current spec, additional emission requirements and associated network signalling for Band n104 are not defined in this version of the specification but may be forthcoming in the future. Thus for PC3 CA_n104C, there is no need to define the additional emission requirements at the current stage but may be needed in the future.

Table 5.x.2-1 shows the spurious emissions requirements for UE co-existence for intra-band contiguous CA n104C.

Table 5.x.2-1: Requirements for uplink intra-band contiguous carrier aggregation

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n104 | E-UTRA Band 1, 3, 7, 8, 20 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |

----- End of TP -----