# 3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411937

# Maastricht, Netherlands, Aug.19th – 23th, 2024

# Source:	ZTE Corporation, Skyworks Solutions, Inc.

# Title:	TP for TR38.719-03-01_CA_n28A-n41A/C-n79A/C

# Agenda Item:	7.3.4

# Document for:	Approval

- Introduction

CA_n28A-n41A/C-n79A/C was requested and included in the new R19 basket WID[1], Hence, we provide a TP to TR38.719-02-01 to introduce CA_n28A-n41A/C-n79A/C. Noted that the fallbacks of CA_n28-n41, CA_n28-n79 and CA_n41-n79 have already completed.

- Reference

- [1] RP-241674, New WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Ericsson, ZTE, Huawei

- [2] TR38.719-03-01,Rel-19 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2)

- Text Proposal

----- Start of TP -----

  - 5.x				CA_n28-n41-n79

    - 5.x.1		Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1		Operating bands for CA

Table 5.x.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n28 | 703 MHz – 748 MHz | 703 MHz – 748MHz | FDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

      - 5.x.1.2		Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier6 |  | NR Band |  | Channel bandwidth (MHz) (NOTE 3) |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n41A-n79C |  | CA_n79C CA_n28A-n41A CA_n28A-n79A CA_n28A-n79C CA_n41A-n79A CA_n41A-n79C |  | n28 |  | n28 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS 4 and 5 |  |  |  |
| CA_n28A-n41C-n79A |  | CA_n41C CA_n41A-n79A CA_n28A-n41A CA_n28A-n79A |  | n28 |  | n28 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS 4 and 5 |  |  |  |
|  |  |  |  | n79 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |  |
| CA_n28A-n41C-n79C |  | CA_n41C CA_n79C CA_n28A-n41A CA_n28A-n79A CA_n28A-n79C CA_n41A-n79A |  | n28 |  | n28 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS 4 and 5 |  |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS 4 and 5 |  |  |  |

      - 5.x.1.3		∆TIB,c and ∆RIB,c values

∆TIB,c and ∆RIB,c values have already been defined in the current spec.

    - 5.x.2		Specific for 2 bands UL CA

      - 5.x.2.1		UE co-existence studies

The co-existence studies of fallback configurations of 2UL band with one CC per band can be applied.

        - 5.x.2.1.1			Co-existence studies for 2UL band with 1CC per band

For CA_n28A-n41A/C-n79A/C, the two UL bands with one CC per band IMD interference analysis have already been analysed in the fallback.

        - 5.x.2.1.2			Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.x.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n41 interference analysis for CA_n28A-n79C with n79C transmitting with a 200MHz maximum instantaneous bandwidth.

Table 5.x.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis.

| Band / CA1 | n28 |  | CA_n79C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 703 -- 748 |  | 4400 -- 5000 |  |
| fDL | 758 -- 803 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -- 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+ max2CCBW |
| IMD3 (MHz) | 503 -- 703 |  | 748 -- 948 |  |
| Analysis | There are no triple beat products caused by UL CA_n28A-n79C falls into band n41. |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.x.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n28 interference analysis for CA_n41A-n79C with n79C transmitting with a 200MHz maximum instantaneous bandwidth.

Table 5.x.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis.

| Band / CA1 | n41 |  | CA_n79C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 2496 -- 2690 |  | 4400 -- 5000 |  |
| fDL | 2496 -- 2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -- 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+ max2CCBW |
| IMD3 (MHz) | 2296 -- 2496 |  | 2690 -- 2890 |  |
| Analysis | There are no triple beat products caused by UL CA_n41A-n79C falls into band n28. |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

      - 5.x.2.2		REFSENS requirements

There is no need to define additional REFSEN requirements (i.e. MSD) for CA_n28A-n41A/C-n79C due to there are no additional triple beat co-existence issue.

----- End of TP -----