3GPP TSG RAN WG4 meeting #112												R4-2411946

Maastricht, Netherlands, Aug 19-23, 2024

Source:			Nokia

Title:			NR band n68 system parameters

Agenda item:	7.16.2

Document for:	Approval

# Introduction

The core objectives of this WI are:

- Specify a new NR FDD operating band n68 (the uplink band for this new band is 30 MHz, 698-728 MHz; the downlink band is 30 MHz, 753-783 MHz; the duplex gap is 25 MHz, 728-753 MHz) to support subcarrier spacing of 15 kHz for 5 MHz channel bandwidth, and subcarrier spacing of 15 kHz and 30 kHz for 10 and 15 MHz channel bandwidths.

  - Only symmetric bandwidths are supported.

- Specify system parameters and RF characteristics of the new band.

- Address potential BS and UE co-existence issues, if any.

- Update the related technical specifications to include support for the new band

- Specify the UE RF requirements for the new bands based on 2 Rx operation.

Note: This new NR band will be introduced in a REL-independent way starting from REL-15.

RAN#104 has approved the WI to introduce NR band n68 (UL 698-728 MHz, DL 753-783 MHz) [1].

In this contribution we discuss the system parameters for n68.

# Discussion

The frequency range of NR band n68 is the same as E-UTRA band 68. The operating band definition shall be specified in the following Table 1.

Table 1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n68 | 698 MHz – 728 MHz | 753 MHz – 783 MHz | FDD |

UL MIMO shall be applicable according to Table 2.

Table 2: NR operating bands for UL MIMO in FR1

NR operating band

n68

Subcarrier spacing of 15 kHz for 5 MHz channel bandwidth and subcarrier spacing of 15 kHz and 30 kHz for 10 and 15 MHz channel bandwidths are supported [1], therefore the channel bandwidth shall be defined as in Table 3. The channel bandwidth up to 15 MHz is specified for E-UTRA band 68; so is for NR band n68.

Table 3 Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n68 | 15 |  | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The following NR-ARFCN shall be applicable for the 100 kHz channel raster.

Table 4: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n68 | 100 | 139600 – <20> – 145600 | 150600 – <20> – 156600 |

Enhanced channel raster is defined in Table 5. Enhanced channel raster is required for the odd/even PRB number issue when UE specific channel bandwidth is configured in a wider BS channel bandwidth. As the maximum channel bandwidth for n68 is 15 MHz, there may not be a use case to configure a UE specific channel bandwidth. Nevertheless, a wider channel bandwidth may be introduced later in the NR and may cause the odd/even number issue in future. We understand that Enhanced channel raster should be defined by default for all FR1 bands with 100 kHz raster, unless the band has no such risk. For example if the maximum frequency range of the band is as small as 5 MHz and only 5 MHz channel bandwidth is defined, then we do not need to introduce Enhanced channel raster for such band. As NR band n68 is 2x30 MHz, this exception shall not apply and therefore Enhanced channel raster shall be introduced.

Table 5: Applicable NR-ARFCN per operating band for enhanced channel raster

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n68 | 10 | 139600 – <2> – 145600 | 150600 – <2> – 156600 |

Enhanced channel raster is defined in Table 5. Enhanced channel raster is required for the odd/even PRB number issue when UE specific channel bandwidth is configured in larger BS channel bandwidth. As the maximum channel bandwidth for n68 is 15 MHz and is supported by all UEs, there may not be a use case to configure a UE specific channel bandwidth. Nevertheless, a wider channel bandwidth may be introduced later in the NR and may cause the odd/even number issue in future. In our view, Enhanced channel raster should be defined by default for all FR1 bands with 100 kHz raster, unless the band has no such risk of odd/even number issue. For example if the maximum frequency range of the band is as small as 5 MHz and only 5 MHz channel bandwidth is defined, then we do not need to introduce Enhanced channel raster for such band. As NR band n68 is 2x30 MHz, this exception shall not apply and therefore Enhanced channel raster shall be introduced.

Sync raster is calculated so that SS block is contained in the band with the minimum guardband 242.5 kHz garanteed. GSNC range from 1888 to 1951 can be used for n68.

Table 6: Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n68 | 15 kHz | Case A | 1888 – <1> – 1951 |

TX-RX frequency separation is defined in Table 7.

Table 7: UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n68 | 55 MHz |

# Conclusion

In this contribution, system parameters for NR band n68 are discussed.

It is proposed to prepare a CR draft to TS 38.101-1 next meeting according the discussion above.

# Reference

[1] RP-241664	New WID on introduction of NR band n68, Ericsson, RAN#104