3GPP TSG RAN WG4 meeting #112												R4-2411947

Maastricht, Netherlands, Aug 19-23, 2024

Source:			Nokia

Title:			Discussion on UE 40MHz channel bandwidth for NR band n28

Agenda item:	7.20.3

Document for:	Approval

# Introduction

The objectives of this WI are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28 as optional features, including the following aspects:

- Introduce requirements for PC2 and those associated with following bullets

  - Both 1Tx and 2Tx PC2 are supported

  - UL-MIMO is supported.

  - Specify A-MPR requirements for PC2

- Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2.

  - Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz.

- Release independence of UE 40MHz Channel bandwidth to Rel-15 is anticipated.

- No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks. In the event of new regulatory requirements, this assumption can be revisited.

RAN#104 has agreed the WI to introduce Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28 [1]. The objectives of the WI are quoted in the following.

In this contribution we discuss the channel raster issues for supporting 40 MHz channel bandwidth.

# Discussion

The WI objective specifies the channel bandwidth location in UL 703-743 MHz and DL 758-798MHz. It is not required to support any arbitrary 40 MHz channel location within NR band n28.

BS 40 MHz channel bandwidth has been already introduced as TEI16 feature [2,3]. It is dedicated to the channel raster entry at 723.04 MHz for UL and 778.04 MHz for DL.

Observation 1: BS 40 MHz channel bandwidth at 723.04 MHz for UL and 778.04 MHz for DL has been already introduced as TEI16 feature.

This channel raster entry for BS 40 MHz channel bandwidth (signaled in SIB1) supports 30 MHz UE specific channel bandwidth centered at 718 MHz for UL and 763 MHz for DL. This UE channel bandwidth location is aligned with the lower band edge of n28 with the minimum guard band 592.5 kHz. At the same time, 30 MHz UE specific channel bandwidth is PRB-grid aligned with BS 40 MHz channel bandwidth centered at 723.04 MHz for UL and 778.04 MHz for DL. There is a 40 kHz frequency shift from the legacy channel raster of 100 kHz.

For the legacy UEs that can support only 100 kHz raster and up to 30 MHz channel bandwidth, 40 MHz BS channel bandwidth (signaled in SIB1) should be configured at this raster entry (UL 723.04MHz and DL 778.04MHz). This is because the legacy 30 MHz UEs cannot to camp on or operate in this cell if 40 MHz is configured at 100 kHz raster and UE channel bandwidth is off the 100 kHz channel raster. No use case so far exists to place 40 MHz channel other than the BS channel raster at UL 723.04MHz and DL 778.04MHz.

Proposal 1: 40 MHz channel bandwidth for both BS and UE is only at the raster entry, 723.04 MHz for UL and DL 778.04 MHz for DL.

The new UEs that support 40 MHz must support the 40 kHz shifted channel raster for 40 MHz channel bandwidth. Therefore, the support of Enhanced channel raster 10 kHz is required. Enhanced channel raster is early implementable from Rel-16 [4]. It is not necessary to specify the specific UE channel raster entry in UE specifications.

Observation 2: It is not necessary to specify the specific UE channel raster entry in UE specifications.

Proposal 2: UE supporting 40 MHz channel bandwidth in band n28 shall support Enhanced channel raster.

# Conclusion

In this contribution

Observation 1: BS 40 MHz channel bandwidth at 723.04 MHz for UL and 778.04 MHz for DL has been already introduced as TEI16 feature.

Proposal 1: 40 MHz channel bandwidth for both BS and UE is only at the raster entry, 723.04 MHz for UL and DL 778.04 MHz for DL.

Observation 2: It is not necessary to specify the specific UE channel raster entry in UE specifications.

Proposal 2: UE supporting 40 MHz channel bandwidth in band n28 shall support Enhanced channel raster.

# Reference

[1] RP-241660 New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC, CBN, RAN#104

[2] RP-223523 CR on R16 TS 38.104 to add channel raster exception for band n28, CMCC, RAN#98e

[3] RP-223524 CR on R17 TS 38.104 to add channel raster exception for band n28, CMCC, RAN#98e

[4] R4-2410712 Rel-18 RAN4 UE feature list for NR (version 5), CMCC, RAN4#111