3GPP TSG-RAN WG4 Meeting #112	R4-2411948

Maastricht, Netherlands, Aug 19-23, 2024

Source:			Nokia

Title:			IMT parameters: General system parameter and UE aspects

Agenda item:	8.2.5

Document for:	Approval

# Introduction

ITU-R WP 5D has requested that external organizations provide IMT parameters for 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz for the sharing studies towards WRC-27 [1]. RAN has agreed SI FS_4400_15350MHz_NR and tasked RAN4 to work on these parameters [2].

RAN4 started the work in RAN4#110-bis.. In the last RAN4#111, the wayforward [9] was agreed for 7125-8400 MHz and 14.8-15.35 GHz while LS reply on 4400-4800 MHz was agree [11].

In this contribution, we discuss open issues on system parameter and UE aspects for 7125-8400 MHz and 14800-15350 MHz.

# Discussion

Typical channel bandwidth

Agreement:

- Mention 100MHz as typical bandwidth in the LS

  - FFS whether to mention wider bandwidths may be possible in future in the LS

Describe other possibilities for bandwidth in the TR, mentioning that additional specification effort needed for >100MHz

The agreement about the typical channel bandwidth in RAN4#111 [9] is in the following for 7125-8400 MHz.

The required channel bandwidth for IMT-2030 depends on the required data rate and achievable spectrum efficiency. The values of 50, 100 or 200 Gbps are mentioned in IMT-2030 Framework [3] as possible examples for the peak data rates. This is 2.5 to 10 times greater than IMT-2020 peak data rate (10 Gbps for Uplink and 20 Gbps in Downlink) [4]. IMT-2030 Framework targets 1.5 times higher averages spectrum efficiency [3]. As it is not easy to significantly improve the peak spectrum efficiency for IMT-2030, much wider total bandwidth than IMT-2020 would be needed to achieve the target peak data rate.

For the frequency range 7125-8400 MHz, we agreed to assume NR band n104 (100 MHz) as baseline and is for further study if we also mention wider bandwidth(s) (e.g. 200 MHz) [7,9]. It is recommended to mention also a wider bandwidth than NR in the reply LS to WP5D to indicate that a wider channel bandwidth than NR is considered in IMT-2030.

Proposal 1: In addition to 100 MHz, 200 MHz is also mentioned as a typical maximum channel bandwidth for 7125 – 8400 MHz in LS.

The following bandwidth options were discussed in RAN4#111 [10] for 14800-15350 MHz.

Issue 3-10: Bandwidth

- Proposals

    - Option 1: 200-400MHz (Nokia)

    - Option 2: 200MHz (Qualcomm, Vivo)

    - Option 3: 100MHz (Ericsson, Apple (possibly also larger))

      - Consider smaller BW for UL with several UEs (e.g. 1-3 UEs) (Ericsson)

    - Option 4: 400MHz (Huawei)

For the same reason as above, we think it is important to assume a wider channel bandwidth than IMT-2020 to achieve higher peak data rate for IMT-2030. Therefore, we propose to consider typical maximum channel bandwidth 200-400 MHz for the frequency range for 14800 – 15350 MHz.

Proposal 2: 200-400 MHz is considered as typical maximum channel bandwidth for 14800 – 15350 MHz.

Typical signal bandwidth

Issue 2-4: Typical signal bandwidth

Option 1: 273 RB, 30k SCS as 38.104

Option 2: Quote formula of RBs * SCS without number of RBs

Option 3: Directly quote the signal bandwidth in MHz

The following options for typical signal bandwidth were discussed in RAN4#111 [9] for 7125-8400 MHz.

The spectrum utilization of IMT-2030 would not be significantly improved from NR in our view unless unwanted emission requirement is significantly relaxed. For 100 MHz, we can assume option 1, i.e., 273 RB with 30 kHz SCS (1 RB=12 SC) with 98.28% spectrum utilization specified in the NR FR1 bands. For wider channel bandwidth than that, at least the same spectrum utilization should be achievable with higher SCS. i.e., 273 RB with 60 kHz SCS for 200 MHz. It is proposed to assume 273 RB for 30/60 kHz for 100/200 MHz channel bandwidth as tentative agreement for 7125-8400 MHz. In the specification phase (i.e., WI phase later), we can further discuss the spectrum utilization parameter for wider channel bandwidths than NR FR1.

Proposal 3: Typical signal bandwidth for 100 MHz channel bandwidth is assumed 273 RB with 30 kHz SCS for 7125 – 8400 MHz. For wider channel bandwidth such as 200 MHz, 273 RB with 60 kHz SCS can be assumed as tentative agreement.

UE Noise Figure

RAN4 earlier informed [9-13] dB as the IMT parameters of the frequency range 6.425-10.5 GHz for the sharing study toward WRC-23 [8]. We understand that the conclusion without square bracket was later summarized in TR 38.921 [6], where Typical UE NF value for 10 GHz band is 9 dB.

TR 38.921 Table 6.2.1-1: Typical noise figure for 7 – 24 GHz example frequencies

| Example frequency (GHz) | Typical NF values for NR BS (dB) | Typical NF values for NR UE (dB) |
| --- | --- | --- |
| 10 | 7 | 9 |
| 15 | 8 | 10 |
| 20 | 9 | 10 |

Issue 3-11: UE noise factor

Agreement:

11dB for simulations. Actual noise factor for reply will be decided based on feasibility.

In the agreed WF [9] in RAN4#111, it was agreed to use NF=11 dB for the coexistence simulation for 14800 – 15350 MHz.

This is 1 dB worse than the typical NF value agreed earlier. We believe this additional margin is not really needed. There may be some confusions about typical NF and the worst NF including all industrial margins (such as in the extreme condition with all production variations). We are not sure if such the worst number should be used for the coexistence simulation analysis.

For the sake of progress, we propose to use it for 14800-15350 MHz also in the reply LS to WP5D. We still propose to keep 10 dB for 7125 – 8400 MHz and 11 dB.

Proposal 4: It is proposed that NF=10 dB for 7125 – 8400 MHz and NF=11 dB for 14800 – 15350 MHz in the reply LS to WP5D.

UE ACLR

Two options were considered for UE ACLR for Frequency 7125 – 8400 MHz in RAN4#111 [10].

  - Option 1:  26dB, 27dB (study) for PC3

  - Option 2:  30dB (n104) for PC3, 31dB (n104) for PC2

Option 2 is already implementation feasible for n104. UE supporting the frequency range 7125-8400 MHz is expected to support n104 and its performance should be compatible. So even if coexistence is possible with Option 1, it is recommended not to have unnecessary relaxation from n104 for 7125-8400 MHz.

For 14800 – 15350 MHz, the coexistence study is going to be performed and ACLR can be discussed once the study is completed.

Proposal 5: ACLR 30dB is proposed for PC3 and 31dB (n104) for PC2 for 7125 – 8400 MHz. It is TBD for 14800-15350 MHz.

Issue 2-15: Maximum output power

Tentative agreement:

- Capture 23dBm as typical maximum output power in the LS to ITU.

  - Other output powers are not precluded and refer to TR

- Capture 20dBm, 23dBm, 26dBm, and higher power classes in TR

Maximum UE output power

In the WF [9] in RAN4#111, maximum output power is agreed in the following for 7125 - 8400 MHz.

In TR 38.921 [6], both 23 dBm and 20 dBm were studied for 6425 - 7125 MHz and 10.0 - 10.5 GHz. Band n104 (6425 - 7125 MHz) has been specified with UE power class 3 (23 dBm) and class 2 (26 dBm) for NR. The same power class as n104 should be implementation feasible for 7125 – 8400 MHz. We have growing trends to support the high-power UE in many NR bands and therefore all different UE high power classes should be captured in TR. However, majority of UE is still expected power class 3 (23 dBm) which should be used for the typical maximum output power in coexistence study. Therefore, we propose to capture 23 dBm as typical maximum output power in the LS response to WP 5D and capture other classes in TR.

Proposal 6: It is proposed to formally agree the tentative agreement in RAN4#111 to capture 23 dBm as typical maximum output power in the LS to ITU and refer to TR for other power classes.

UE architecture FR1 vs FR2 for 14800 – 15350 MHz

Issue 3-8: UE type

Agreement:

For simulations, consider both options.

    - FR1 like

      - RX diversity gain: [5] dB assuming 4RX

      - TX: 0dBi omnidirectional

      - Power: 23 dBm, 26dBm

    - FR2 like

      - Two panels (one in each direction) 2x2 antenna

        - 5dBi element gain. Array gain comes on top

      - Power: 23 dBm TRP

In recent discussion in RAN4, FR1-like architecture was preferred by 5 companies while FR2-like is also preferred by three companies. There are also a few companies suggesting more discussion [10]. For the coexistence simulation, both options were considered in RAN4#111 [9].

FR2 band such as band n257/n258 is already commercially implemented in UE. Such architecture would be also feasible for 14800-15300 MHz unless the size of antenna array is not a big concern such as in CPE type or vehicular mounted devices. Since the handheld device has a limited space for antenna array, it may be still preferred to consider FR1-like architecture.

More investigation would be needed to decide which architecture is assumed in the final specifications for IMT-2030. If FR1 is of only interest, we may only specify FR1 requirement and include it in the LS reply to WP5D. If both FR1 and FR2 are of interest, both requirements may need to be specified like in the BS specifications where conducted, hybrid and OTA requirement are all specified as 1-C, 1-H, and 1-O requirements. We may take such approach also for UE specifications if both architectures are of interest for specifications. In that case both architectures may better be informed in the LS reply to WP5D.

Observation 1: FR1-like architecture may be more feasible for handheld devices for 14800-15300 MHz.. FR2 may not be excluded for other UE types. It may be premature to single out the UE architecture for 14800-15300 MHz.

Proposal 7: Unless there is no clear consensus, both FR1 and FR2 architecture assumptions are informed to WP5D in the LS reply.

# Conclusion

Proposal 1: In addition to 100 MHz, 200 MHz is also mentioned as a typical maximum channel bandwidth for 7125 – 8400 MHz in LS.

Proposal 2: 200-400 MHz is considered as typical maximum channel bandwidth for 14800 – 15350 MHz.

Proposal 3: Typical signal bandwidth for 100 MHz channel bandwidth is assumed 273 RB with 30 kHz SCS for 7125 – 8400 MHz. For wider channel bandwidth such as 200 MHz, 273 RB with 60 kHz SCS can be assumed as tentative agreement.

Proposal 4: It is proposed that NF=10 dB for 7125 – 8400 MHz and NF=11 dB for 14800 – 15350 MHz in the reply LS to WP5D.

Proposal 5: ACLR 30dB is proposed for PC3 and 31dB (n104) for PC2 for 7125 – 8400 MHz. It is TBD for 14800-15350 MHz.

Proposal 6: It is proposed to formally agree the tentative agreement in RAN4#111 to capture 23 dBm as typical maximum output power in the LS to ITU and refer to TR for other power classes.

Observation 1: FR1-like architecture may be more feasible for handheld devices for 14800-15300 MHz.. FR2 may not be excluded for other UE types. It may be premature to single out the UE architecture for 14800-15300 MHz.

Proposal 7: Unless there is no clear consensus, both FR1 and FR2 architecture assumptions are informed to WP5D in the LS reply.

# Reference

[1] R4-2400333 Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz, ITU-R Working Party 5D

[2] RP-240787 New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz

[3] M.2160 Framework and overall objectives of the future development of IMT for 2030 and beyond

[4] M.2410 Minimum requirements related to technical performance for IMT-2020 radio interface(s)

[5] R4-1700305 LS on Characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz

[6] TR 38.921 Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz

[7] R4-2406615 WF on IMT parameters for other frequency ranges

[8] R4-2103104 LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)

[9] R4-2410741 WF on IMT parameter study, Ericsson, RAN4#111

[10] R4-2410738 Ad hoc minutes #2 for [111][127] FS_NR_IMT, Ericsson, RAN4#111

[11] R4-2410576, Draft LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, RAN4#111