3GPP TSG-RAN WG4 Meeting #112	R4-2411949

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Discussion on less than 5 MHz NTN system parameter aspects

Agenda item:	8.8.3.1

Document for:	Approval

Introduction

In this paper, we provide our views on system parameter aspects for the support of less than 5 MHz channel bandwidth for NR NTN UEs. Core parts of the objectives mentioned in the WID are copied below for ease of reading [1].

Objectives:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

Discussion

3 MHz channel bandwidth has been already introduced to NR bands n26, n28, n31, n72, n85, n100 and n106 in TS 38.101-1. The system parameters designed for NR FR1 bands such as SCS, spectrum utilization and minimum guard band can be reused for FR1-NTN bands n254, n255, and n256.

Proposal 1: Only 15 kHz SCS is considered for 3 MHz channel bandwidth.

Proposal 2: Maximum transmission bandwidth configuration for 3 MHz is 15 PRBs.

Proposal 3: Minimum guardband for 3 MHz channel bandwidth is 142.5 kHz.

For NR FR1 bands for TN, additional sync raster for 3 MHz channel bandwidth has been introduced. The same GSCN design can be reused for FR1-NTN bands.

Proposal 4: Sync raster design for 3 MHz channel bandwidth in TN bands is reused for FR1-NTN bands.

GSCN parameters for 3 MHz channel bandwidth in TS 38.101-1 for TN bands are defined in the following table. This sync raster design guarantee the support 3 MHz channel bandwidth position at any NR-ARFCN with the puncutred SSB for the size of 12 PRBs, which suffice the purpose of NTN as weel.

TS 38.101-1: Table 5.4.3.1-2: GSCN parameters for the global frequency for 3 MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 1000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 31634 |
| NOTE 1: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |  |  |  |

3 MHz channel bandwidth was only considered for the bands below 1 GHz in TN bands. The range of GSCN needs to be further extended at least up to 2.5 GHz for NTN band n254. Although we do not know yet if there is any NTN band above 2.5 GHz in future, it is proposed to define GSCN up to 3 GHz where NR-ARFCN is defined with 5 kHz granularity to support 100 kHz channel raster as well as Enhanced channel raster 10 kHz.

Proposal 5: The range of GSCN for 3 MHz channel bandwidth is defined up to 3 GHz for FR1-NTN bands.

Therefore, the above table needs to extended in the following in the NTN specifications.

GSCN parameters for the global frequency for 3 MHz channel bandwidth (for NTN)

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:4999, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41636 |

As we only consider 15 kHz SCS for 3 MHz channel bandwidth, SS block shall be also 15 kHz and SS block pattern Case A should be only considered. The range of GSCN for bands n256, n255 and n254 are calculated in the following so that PSS/SSS (12 PRBs) is contained in the band with the minium guardband 142.5 kHz.

Proposal 6: The following SS raster entries for 3 MHz channel bandwidth are proposed for bands n256, n255 and n254.

Applicable SS raster entries per operating band for 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 37492 – <1> – 37629 |
| n255 | 15 kHz | Case A | 34267 – <1> – 34424 |
| n254 | 15 kHz | Case A | 39060 – <1> – 39129 |

Conclusion

The following Observations and Proposals were made:

Proposal 1: Only 15 kHz SCS is considered for 3 MHz channel bandwidth.

Proposal 2: Maximum transmission bandwidth configuration for 3 MHz is 15 PRBs.

Proposal 3: Minimum guardband for 3 MHz channel bandwidth is 142.5 kHz.

Proposal 4: Sync raster design for 3 MHz channel bandwidth in TN bands is reused for FR1-NTN bands.

Proposal 5: The range of GSCN for 3 MHz channel bandwidth is defined up to 3 GHz for FR1-NTN bands.

GSCN parameters for the global frequency for 3 MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:4999, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41636 |

Proposal 6: The following SS raster entries for 3 MHz channel bandwidth are proposed for bands n256, n255 and n254.

Applicable SS raster entries per operating band for 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 37492 – <1> – 37629 |
| n255 | 15 kHz | Case A | 34267 – <1> – 34424 |
| n254 | 15 kHz | Case A | 39060 – <1> – 39129 |

- References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103, Maastricht, Netherlands, 18-21 March 2024.