3GPP TSG-RAN WG4 Meeting #112	R4-2411952

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	(NR_newRAT) NR-E-UTRAN HO requirement maintenance

Agenda item:	4.5

Document for:	Discussion

Introduction

In RAN4 meeting 110 in Athens, Greece, RAN4 agreed on CR 4018 in Tdoc R4-2403410 [1] which was a revision of a CR submitted in R4-2400470 [2]. The issue had been discussed for several meetings. The agreed change was as originally proposed in [2] however with bracket ([]) around the added wording. The added wording allowed UE additional time for an NR to E-UTRAN HO under certain conditions, while the additional delay was left open and for further discussion:

[If PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is not less than 153600⋅Ts, the longer handover delay is expected.]

Common understanding was to return the agreement and clarify the value of the expected longer handover delay to make the requirement clear.

However, since Athen, RAN4 has been busy finalizing Rel-18 work, and this aspect has been left unattended. Hence, we would now like to return to this matter to clarify the requirements.

Discussion

Discussion on NR – E-UTRAN Handover Delay

The reason for adding the additional delay was according to the CR [1]:

In TS38.133 section 6.1.2, the NR – E-UTRAN Handover requirement was specified as same as E-UTRAN – E-UTRAN Handover requirement in section 5.1 of TS36.133, where the SFN acquisition time delay for target cell was not included.

For E-UTRAN – E-UTRAN Handover, RAN1 has clear assumption for the syncrhonization between serving cell and target cell as followings (TS36.211 section 5.7.1). That’s the justification to skip the SFN acquisition during the E-UTRAN – E-UTRAN Handover when the RO is configured on even system frame.

For PRACH configurations 0, 1, 2, 15, 16, 17, 18, 31, 32, 33, 34, 47, 48, 49, 50 and 63 the UE may for handover purposes assume an absolute value of the relative time difference between radio frame i in the current cell and the target cell of less than 153600⋅Ts .

However, it’s unclear whether such assumption can be also applied for NR – E-UTRAN Handover scenario, as intuitionally different RAT may have different synchronizaiton performance compared with Intra-RAT HO case. Thus, if the NR – E-UTRAN Handover scenario can have different assumption of SFN synchronization condition, it would be very desirable to clarify the applicability of the corresponding RAN4 requirement when the RO is configured on even system frame, i.e., clarify current NR – E-UTRAN HO requirement (no SFN acquisition delay) can be applied under a specific SFN synchronization condition as in TS36.211 section 5.7.1.

Based on the discussion it was agreed to include following line in the beginning of section 6.1.2.1.1 which is the introduction:

[If PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is not less than 153600⋅Ts, the longer handover delay is expected.]

We would like to address two aspects:

- RAN4 need to decide the actual value of ‘longer handover delay’. Hence, what additional delay need to be added under the identified conditions.

- Add the delay as a clear requirement when the conditions are fulfilled.

Regarding the actual delay our understanding is that the additional delay is needed to allow the UE to acquire the SFN of the target cell. Hence, the UE would need to be able to read the LTE MIB. Under normal conditions we can expect that the HO candidate conditions will be rather good. Hence, we believe we should not assume worst case conditions in this case. Instead, we can assume at least reasonable conditions and under which the UE would be able to receive the LTE MIB in a single attempt.

Allow UE an additional HO delay of one MIB reception attempt when the given conditions are fulfilled.

As the delay under discussion and added is related to PRACH, we suggest clarifying the requirements by moving the agreed delay addition to Tinterrupt delay under TIU, which relates to the delay uncertainty in acquiring the first available PRACH occasion in the new cell.

Define the delay requirement as part of PRACH acquisition delay in Tinterrrupt under TIU.

Additionally, we suggest a minor wording update as follows:

[If PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is not less than larger than or equal to 153600⋅Ts, the longer handover delay is expected.]

Based on the proposal 1 and 2 and the proposed wording update, a TP would look as follows:

### 6.1.2	NR Handover to other RATs

#### 6.1.2.1	NR – E-UTRAN Handover

##### 6.1.2.1.1	Introduction

The purpose of inter-RAT handover from NR to E-UTRAN is to change the radio access mode of PCell from NR to E-UTRAN. The handover procedure is initiated from NR with a RRC message that implies a handover as described in TS 38.331 [2]. The requirements in this clause are applicable to SA NR, NE-DC and NR-DC.

[If PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is not less than 153600⋅Ts, the longer handover delay is expected.]

##### 6.1.2.1.2	Handover delay

When the UE receives a RRC message implying handover to E-UTRAN the UE shall be ready to start the transmission of the uplink PRACH channel in E-UTRA within Dhandover ms from the end of the last TTI containing the RRC command. Dhandover is defined as

Dhandover = TRRC_procedure_delay + Tinterrupt

Where:

TRRC_procedure_delay: it is the RRC procedure delay, which is 50ms

Tinterrupt: it is the time between end of the last TTI containing the RRC command on the NR PDSCH and the time the UE starts transmission of the PRACH in E-UTRAN, excluding TRRC_procedure_delay. Tinterrupt is defined in clause 6.1.2.1.3.

##### 6.1.2.1.3	Interruption time

When the inter-RAT handover to E-UTRAN is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt =Tsearch + TIU + 20 ms

Where:

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms. If PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is larger than or equal to 153600⋅Ts, the UE is allowed an additional MIB reception delay of [10]ms.

NOTE:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant E-UTRAN cell identification requirements are described in clause 9.4.1.

In CR [3] we provide a CR capturing the proposed CR.

Conclusion

The paper we shortly discuss some remaining open aspects related to a CR agreed in the Athens meeting. At that time, it was agreed to define a specific value for the ‘longer handover delay’. In this paper we suggest a value for the delay and some additional updates.

- Allow UE an additional HO delay of one MIB reception attempt when the given conditions are fulfilled.

- Define the delay requirement as part of PRACH acquisition delay in Tinterrrupt under TIU.

- References

- R4-2403410, (NR_newRAT) CR on NR-E-UTRAN HO requirement maintenance R15, Apple

- R4-2400470, (NR_newRAT) CR on NR-E-UTRAN HO requirement maintenance R15, Apple

- R4-2411953, (NR_newRAT) CR on NR-E-UTRAN HO requirement, Nokia