3GPP TSG-RAN WG4 Meeting #112                 	R4-2411964

Maastricht, Netherlands, Aug 19th –23rd, 2024

Source:	Nokia

Title:	Discussion on FR1-FR1 SCG activation

Agenda item:	5.14.1

Document for:	Approval

- Introduction

This paper is a slightly revised version of R4-21408161 [20]. Technical content is pretty much unchanged except continued discussion on RACH-less based activation has been removed.

RAN plenary has agreed Rel-18 WI for Even Further RRM requirement for NR and MR-DC [1], to define RRM requirements for FR1+FR1 NR-DC scenarios is one of the objectives in this WI. The discussion started in RAN4#104e. Till RAN4#108 meeting, most of the impacted RRM requirements for FR1+FR1 NR-DC scenarios have conclusion and big CR is endorsed in RAN4#107 meeting [7].

In RAN4#109 the BigCr was endorsed and agreed afterwards. Additionally, it was agreed that the core part of WI could be closed. However, it was also agreed that there are still a few open issues in SCG activation requirements. These open issues are related to similar ongoing discussion in Rel-17 and is also discussed in Rel-17 (agreed WF in Rel-17 [5]).

In RAN4#110 (Athens) and RAN4#119bis (Changsha) agreements were reached on some of the aspect of known and unknown PSCell under discussion in Rel-17 in [15, 16] and the related CR for mirroring the agreement to this WI [17] was endorsed [18].

However, there are still a few remaining open issues in SCG activation requirements for FR1+FR1 NR-DC scenarios.

- Discussion on SCG Activation/deactivation delay

2.1	Introduction

For the discussion related to SCG activation and deactivation 2 main issues have been discussed in RAN4:

- Tsearch in RACH-less based PSCell activation delay.

- Tsearch in RACH-based PSCell activation delay.

Issues related to RACH-less based PSCell activation seems to be having clear requirements in the current version of the specification. We will now look at the Tsearch part of the PSCell activation delay for RACH-based PSCell activation and discuss:

- Tsearch delay for a UE being configured with RLM-and-bfd=true.

  - Tsearch for RACH-based PSCell activation

When the UE is configured ‘bdf-and-RLM’ with value ‘true’ for the deactivated PSCell, the UE shall perform BFD (according to section 8.5) and RLM (according to section 8.1) on the deactivated PSCell.

- When ‘bdf-and-RLM’ with value ‘true’ is configured for the deactivated PSCell the UE shall perform BFD and RLM on the deactivated PSCell.

For the UE to perform these measurements and evaluate RLM and BFD, the UE is required to measure the associated RSs according to the requirements defined in sections 8.1 and 8.5. The measurements and evaluation are performed in a regular manner as defined in those sections.

The procedure for performing BFD and RLM on the deactivated PSCell and on the PCell or activated PSCell in DRX is the same. Hence, just like when the PSCell is in activated state, the UE shall measure the deactivated PSCell regularly to evaluate the downlink radio link quality for the cell (RLF) and beam (BFD).

- Evaluation of RLM and BFD on a deactivated PSCell requires the UE to measure the deactivated PSCell regularly to evaluate the downlink radio link quality.

Basic principle is illustrated in the following figure.

While the PSCell is deactivated and UE monitors bfd-and-RLM on the deactivated PSCell, following may occur:

- No BF occur and no RLF occur.

- BF occur but no RLF occur.

- Both BF and RLF occurs.

When the UE is required to perform bfd-and-RLM and has not experienced either beam failure or RLF on the deactivated PSCell, the UE has not experienced link quality problems severe enough to declare radio link or beam failure.

When the UE has not experienced link quality problems severe enough to declare radio link or beam failure, the link is considered good enough for continuing the operation in the PSCell when the PSCell is activated. Therefore, the same can be expected for the deactivated PSCell when it is being activated:

If the UE is required to perform bfd-and-RLM and has not experienced either beam failure or RLF on the deactivated PSCell prior to activation, there is no need for cell detection, measurements, or beam recovery time upon PSCell activation.

- If no link problems have been detected, the link is currently regarded good enough for continuing the operation in a cell.

- When UE is configured with bfd-and-RLM on the deactivated PSCell, and the UE no link problems (beam failure or RLF) have been detected on the deactivated PSCell, link is regarded good enough for continuing operation.

- When UE is configured with bfd-and-RLM = true and UE has not detected link problems, there is no need for additional measurements at PSCell activation.

Figure 1 Illustration of deactivated SCG (FR2) with UE configured with bfd-and-RLM  with value True.

If we consider an illustrative example as in figure 1, where we initially consider the FR2 scenario but the same applies to FR1. If the gNB1 is the deactivated PSCell with bfd-and-RLM  = true configured, the UE is required to measure and evaluate RLM and BFD according to the defined requirements for a deactivated PSCell (defined in Tables 8.1.2.2-5 and 8.5.2.2-5).

If neither RLM nor BFD has been detected while PSCell was deactivated, for example because the UE has not moved, the need for additional measurements at PSCell activation will be greatly reduced if needed at all.

If the UE has experienced link quality problems leading to a failure (either beam failure or RLF) on the PSCell while PSCell was in deactivated state, for example due to changed conditions, some additional search time would be needed when PSCell is activated.

In general, a UE which has not detected neither BFD nor RLF on the deactivated PSCell while deactivated, need no additional search time at PSCell activation (Tsearch = 0ms).

A UE which has detected either BFD or RLF on the deactivated PSCell is allowed search time at PSCell activation.

2.2.2 Tsearch in RACH-based PSCell activation delay

The latest version of 38.133, section 8.17.2 states following concerning RACH-based PSCell activation:

- For RACH based PSCell activation, if the FR1 or FR2 PSCell is known, Tsearch = 0 ms. If the FR2 PSCell is unknown and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms. If the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms.

Based on the discussion in section 2.2 above, we propose that for the UE which is configured with bfd-and-RLM =true for the deactivated PSCell and which has not experienced either beam failure or RLF on the deactivated PSCell while being deactivated, Tsearch can be reduced.

If the UE is configured with bfd-and-RLM on the PSCell and no RLF has been detected, it means that the deactivated PSCell is available, has been measured recently (according to the UE measurement requirements for a deactivated PSCell) and is evaluated having a good enough link quality. Hence, for this scenario, there seems reasonable to discuss reduced Tsearch at PSCell activation.

For RACH based PSCell activation for unknown PSCell, RAN4 to agree on conditions for reduced Tsearch at PSCell activation, when the UE is configured with bfd-and-RLM.

We suggest that for the RACH-based PSCell activation of an unknown PSCell, the condition when Tsearch can be reduced from 24*Trs or 3*Trs, is when no RLM has occurred.

Tsearch for PSCell activation of an unknown PSCell, can be reduced from 24*Trs or 3*Trs, is when no RLM has occurred.

As discussed, because the UE has continuously measured and evaluated the PSCell being activated, we believe a single sweep should allow UE enough margin during the PSCell activation.

For example:

- For RACH based PSCell activation, if the FR1 or FR2 PSCell is known, Tsearch = 0 ms. If the PSCell is an unknown FR1 or FR2 PSCell, configured with bfd-and-RLM with value true, provided no RLM has occurred, if Es/Iot ≥ -2 dB then for FR2 Tsearch = [8]* Trs ms and Tsearch = [1]* Trs ms for FR1. Otherwise, if the FR2 PSCell is unknown and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms.

For RACH based PSCell activation, if the FR1 or FR2 PSCell is known, Tsearch = 0 ms. If the PSCell is an unknown FR1 or FR2 PSCell, configured with bfd-and-RLM with value true, provided no RLM has occurred, if Es/Iot ≥ -2 dB then for FR2 Tsearch = [8]* Trs ms and Tsearch = [1]* Trs ms for FR1. Otherwise, if the FR2 PSCell is unknown and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms.

2.3 PSCell activation delay and PSCell DRX

Finally, we would like to clarify the UE behavior at PSCell activation regarding monitoring for DL scheduling.

RAN4 has introduced PSCell activation delay requirements. One set of requirements are related to RACH-less based delay requirements and another set is for RACH-based requirements. RAN4 has currently defined following:

TIU: When RACH based PSCell activation is configured, it is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TIU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion [or SR on PUCCH].

However, it is not clear from network perspective when the UE is required to be monitoring DL on the activated PSCell for possible scheduling.

For example, in RACH-less based access, the network would have to know when it is possible to initiate UE scheduling e.g. for allocating the PUSCH. However, as the PSCell have own DRX cycle this becomes ambiguous. The PSCell activation delay may end during a PSCell on-duration or not. If the delay ends during an on-duration the UE behavior is clear. However, if this is not the case the UE behavior is not clear.

- UE PDCCH monitoring requirements in the newly activated PSCell is ambiguous.

One purpose of activating the PSCell is due to immediate need for the PSCell resources. Therefore, it essential for the network to know when UE can be scheduled. It seems reasonable to assume that the UE will start monitoring the PDCCH for scheduling on the activated PSCell immediately after being activated.

UE shall start monitoring PDCCH on the activated PSCell immediately after the SCG activation delay.

For the RAN4 requirements this would mean:

TIU: When RACH based PSCell activation is configured, it is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TIU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion [or SR on PUCCH]. The UE shall apply no DRX immediately after Tactivation_time.

We have earlier suggested sending LS to RAN2 clarifying this with RAN2. We think this is still needed and have provided a draft LS in Appendix.

Send LS to RAN2 clarifying PDCCH monitoring assumption with RAN2.

- Conclusion

In this paper we address the remaining open issues related to SCG activation/deactivation delay in FR1+FR1 NR-DC. Based on the discussion in this and former meetings we propose:

Tsearch for RACH-based PSCell activation:

- When ‘bdf-and-RLM’ with value ‘true’ is configured for the deactivated PSCell the UE shall perform BFD and RLM on the deactivated PSCell.

- Evaluation of RLM and BFD on a deactivated PSCell requires the UE to measure the deactivated PSCell regularly to evaluate the downlink radio link quality.

- If no link problems have been detected, the link is currently regarded good enough for continuing the operation in a cell.

- When UE is configured with bfd-and-RLM on the deactivated PSCell, and the UE no link problems (beam failure or RLF) have been detected on the deactivated PSCell, link is regarded good enough for continuing operation.

- When UE is configured with bfd-and-RLM = true and UE has not detected link problems, there is no need for additional measurements at PSCell activation.

- In general, a UE which has not detected neither BFD nor RLF on the deactivated PSCell while deactivated, need no additional search time at PSCell activation (Tsearch = 0ms).

A UE which has detected either BFD or RLF on the deactivated PSCell is allowed search time at PSCell activation.

Tsearch in RACH-based PSCell activation delay:

For RACH based PSCell activation for unknown PSCell, RAN4 to agree on conditions for reduced Tsearch at PSCell activation, when the UE is configured with bfd-and-RLM.

Tsearch for PSCell activation of an unknown PSCell, can be reduced from 24*Trs or 3*Trs, is when no RLM has occurred.

For RACH based PSCell activation, if the FR1 or FR2 PSCell is known, Tsearch = 0 ms. If the PSCell is an unknown FR1 or FR2 PSCell configured with bfd-and-RLM with value true, provided no RLM has occurred, if Es/Iot ≥ -2 dB then for FR2 Tsearch = [12]* Trs ms and Tsearch = [1]* Trs ms for FR1. Otherwise, if the FR2 PSCell is unknown and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms.

PSCell activation delay and PSCell DRX:

- UE PDCCH monitoring requirement in the newly activated PSCell is ambiguous.

UE shall start monitoring PDCCH on the activated PSCell immediately after the SCG activation delay.

Send LS to RAN2 clarifying PDCCH monitoring assumption with RAN2.

We have captured the proposed changes in our CR [21] and provided draft LS in Appendix. The CR currently only addresses necessary change from Release 18. It was discussed in last meeting whether agreements should be applicable from Release 17. Such an approach would of course be beneficial as we would then have aligned requirement from release 17. Hence, we would support such an approach.

- References

- RP-221696	Revised WID on Rel-18 even Further RRM enhancement for NR and MRDC, Apple, Oppo, RAN#96

- TS38.133, i40

- R4-2317360	WF on NR eFeRRM (part2), Oppo, RAN4#108bis

- R4-2314337	WF on NR FeRRM (part2), Oppo, RAN4#108

- R4-2314395	WF on LTE_NR_DC_enh2 maintenance - PSCell activation, Nokia, RAN4#108

- R4-2310167 WF on FeRRM part2, Oppo, RAN4#107

- R4-2310080	Big CR for R18 RRM enhancement- FR1+FR1 NR-DC, Oppo, RAN4#107

- R4-2306322 WF on R18 RRM enhancement - FR1+FR1 NR-DC, Oppo, RAN4#106bis-e

- R4-2303304 Way forward on R18 RRM enhancement (part 2), Oppo, RAN4#106

- R4-2220736 WF on FR1-FR1 NR-DC in eFeRRM, Oppo, RAN4#105

- R4-2217250 WF on R18 eFeRRM – RRM requirements for FR1-FR1 NRDC, Oppo, RAN4#104bis-e

- R4-2214345	WF on R18 eFeRRM, Apple, RAN4#104e

- R4-2401733 DraftCR on SCG activation delay for FR1+FR1 NR-DC, Nokia, Nokia Shanghai Bell

- R4-2401729 (LTE_NR_DC_enh2-Core) Discussion concerning SCG Activation and Deactivation Delay, Nokia, Nokia Shanghai Bell

- R4-2403552, (LTE_NR_DC_enh2-Core) CR corrections for SCG Activation and Deactivation Delay, Nokia, Nokia Shanghai Bell

- R4-2401731, (LTE_NR_DC_enh2-Core) CR corrections for SCG Activation and Deactivation Delay, Nokia, Nokia Shanghai Bell

- R4-240xxxx, DraftCR on SCG activation delay for FR1+FR1 NR-DC, Nokia

- R4-2406426, FR1-FR1 SCG activation, Nokia

- R4-2408162, FR1-FR1 SCG activation, Nokia

- R4-24108161, Discussion on FR1-FR1 SCG activation, Nokia

- R4-2411965, CR on FR1-FR1 SCG activation, Nokia

- Appendix

3GPP TSG RAN WG 4 Meeting #112		TDoc R4-24xxxxx

Maastricht, Netherlands, Aug 19th – Aug 23rd, 2024

Title:	LS on PSCell activation and PSCell DRX

Response to:

Release:	Rel-18

Work Item:	NR_RRM_enh3-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Lars Dalsgaard

lars.dalsgaard@nokia.com

Attachments:	None

# 1	Overall description

RAN4 is introducing PSCell activation delay requirements. One set of requirements are related to RACH-less based delay requirements and another set is for RACH-based requirements. However, at PSCell activation it is not clear from network perspective when the network can expect the UE monitoring the DL for possible scheduling.

Currently it is unclear whether the UE will follow the DRX monitoring rules immediately after PSCell activation or whether the network can schedule the UE immediately after the PSCell activation.

RAN4 would like to ask RAN2 how the UE will apply the PSCell DRX monitoring rules after PSCell activation?

# 2	Actions

To RAN2

ACTION: 	RAN4 would like to ask RAN2 how the UE will apply the PSCell DRX monitoring rules after PSCell activation?

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN4 Meeting #112bis	14 Oct – 18 Oct, 2024		Hefei, China

TSG-RAN4 Meeting #113	18 Nov – 22 Nov, 2024		Orlando, USA