3GPP TSG-RAN WG4 Meeting #112    	R4-2412007

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Title: 	UE RF Enh 4: On necessity of additional MSD requirement for HPUE

Agenda Item:	8.1.1.1.2

Source: 	Nokia

Document for:	Approval

# Introduction

In previous RAN4 meeting #111 the discussion on HPUE MSD continued like it has continued for quite some time now. In RAN4#111 there were in fact parallel R18 and R19 discussions on the topic.

# Discussion

Under R18 WF [3] was approved

WF

As background information, up until now RAN4 must handle up to 6 different types of MSDs:

- Type 1: 1UL Intra-band MSD,

- Type 2: 2UL Intra-band MSD

- Type 3: Inter-band CA UL harmonic MSD,

- Type 4: Inter-band CA Rx harmonic mixing MSD,

- Type 5: Inter-band CA MSD due to dual-UL IMD interference, including triple beat,

- Type 6: Inter-band CA MSD due to cross-band isolation interference.

With the introduction of single PC2 FDD operation, several inter-band CA MSD test points need to be re-evaluated. Workload is heavy on RAN4.

<Way forward on simplifying HPUE MSD Analyses>

Interested companies are invited to share their views on the following options at the next meeting:

For all types of MSDs:

- Option 1: RAN 4 defines only the default power class inter-band CA and intra-band CA MSD requirements, and the new HPUE MSD requirements are no longer specified in the TS. The legacy agreed HPUE MSD test points are not impacted and are kept in the TS.

- Option 2: RAN 4 defines the default power class inter-band CA and intra-band CA MSD requirements, and RAN4 specifies equations or simplified guidelines (like in [3,4]) or lookup tables “LUT” (like in [5]) that define the relationship between the default power class MSD and the HPUE MSD requirements:

  - Option 2a: Equations/LUT are captured in the RAN4 TS,

  - Option 2b: Equations/LUT are not captured in the RAN4 TS and are provided to RAN5.

- Option 3:  Keep the existing way of working, i.e. RAN4 analyses case by case all HPUE MSD requirements and experts flag TP for TRs for which MSD needs corrections/re-evaluations.

- Other options are not precluded.

Under R19 UE RF enhancements for NR FR1/FR2 and EN-DC Phase 4 WI, MSD was discussed with more limited scope as per WI objectives. This WI has two HPUE objectives where MSD was debated

- Power class 1.5 (PC1.5) UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx and PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA, and PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA

- Investigate and if feasible, support increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band uplink CA and EN-DC HPUE with the different existing power classes which have already been specified

For the second R19 HPUE objective increasing UE transmission power limit an agreement was reached [4]

## MSD and SAR compliance

Way forward:

  - If higherPowerLimit-R17/higherPowerLimtMRDC-R17 capability is adopted for increasing UE transmission power, then taking same methodology as for higherPowerLimit-R17/ higherPowerLimtMRDC-R17 for SAR compliance and MSD. More specific,

- For a given band combination, no additional MSD requirements are specified in addition to the MSD requirements of its legacy power class(es)

- No new duty-cycle solution/mechanism for SAR compliance is considered

- P-MPR can be used for SAR mitigation

Hence for R19 WI what is open is how to handle PC1.5 operation MSD. For this in RAN#104 there was a proposal [1] to revise the WI: Proposal 1: Revise the UE RF WID to include objective for investing new MSD framework for HPUE operation. Proposal 1 was not approved. We value the proposal in [1] but in our view current WI[2] does not need modification to allow RAN4 to discuss MSD. Let us explain, here is the WIDs relevant objective

High power UE (HPUE) for CA in terrestrial network (TN)

- Specify the generic requirements of high power UE (HPUE) for NR uplink (UL) CA in FR1 and EN-DC with NR FR1 bands

  - Power class 1.5 (PC1.5) UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx

    - Specify the requirements for intra-band UL contiguous CA with or without UL-MIMO

      - Example band combinations:

        - CA_n41C, CA_n78C, CA_n77C, CA_n79C for intra-band uplink contiguous CA configurations

      - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution

    - Specify the requirements for intra-band UL non-contiguous CA without UL-MIMO

      - Example band combinations:

        - CA_n78(2A), CA_n77(2A) for intra-band uplink non-contiguous CA configurations

      - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution

    - NOTE: leave the other band combination specific requirements to the corresponding Rel-19 basket WIs

  - PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA, and PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA

    - Focus on the SAR solution

    - Enable power class 2 (PC2) and PC1.5 of two band inter-band uplink CA and EN-DC with 3Tx for handheld UE

      - Identify and update the requirements if necessary

    - Only PC3 is considered for LTE FDD in EN-DC

    - NOTE: leave the band combination specific requirements, e.g., MSD to the corresponding Rel-19 basket WIs

.

Firstly WID says what are the issues where the focus is ((MOP), MPR/A-MPR requirements, SAR solution) but MSD s not listed. By definition Focus is point of concertation and direction of attention. Focus does not preclude anything i.e. MSD also can be discussed. Secondly it is mentioned that band combination specific MSD is not discussed. We agree with this. We do not think that band combination specific MSD should be discussed at all.

Observation 1: WID objectives preclude band combination specific MSD discussions but allow generic MSD discussion applicable to all band combinations.

Building on this we propose that

Proposal 1: When new PC1.5 NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx configuration is introduced no additional MSD test points are specified.

Proposal 2: When new PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA, and PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA is introduced no additional MSD test points are specified.

There were multiple HPUE WI [5][6][7][8] approved in RAN#104 which have new configuration proposals where the necessity of creating new MSD requirement is open even in the case the proposals in this paper are approved. Framework for those new configurations can be discussed in relevant basket WI separately unless there is a consensus to have common WF in this meeting for all HPUE power classes no matter if it is single band, CA or DC.

Observation 2: Framework for those new configurations which are not covered by proposals in this paper can be discussed in relevant basket WI separately unless there is a consensus to have common WF in this meeting for all HPUE power classes no matter if it is single band, CA or DC.

# 3	Conclusion

In this contribution we have discussed HPUE MSD and made following observation and two proposals

Observation 1: WID objectives preclude band combination specific MSD discussions but allow generic MSD discussion applicable to all band combinations.

Proposal 1: When new PC1.5 NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx configuration is introduced no additional MSD test points are specified.

Proposal 2: When new PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA, and PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA is introduced no additional MSD test points are specified.

Observation 2: Framework for those new configurations which are not covered by proposals in this paper can be discussed in relevant basket WI separately unless there is a consensus to have common WF in this meeting for all HPUE power classes no matter if it is single band, CA or DC.

# 4	References

[1] RP-241042	Discussion on the MSD framework for HPUE and SRS antenna switching for 6Rx, RAN#104  	vivo

[2] RP-241656	Revised WID: UE RF enhancements for NR FR1/FR2 and EN-DC 	Phase 4, RAN#104  Huawei, HiSilicon

[3] R4-2410605 WF on HPUE MSD simplification, RAN4#111, Skyworks

[4] R4-2410750 WF on higher power UE, RAN4#111, Samsung

[5] RP-241680 - New WID: Rel-19 High power UE (power class 1.5 and 2) for NR FR1 TDD/FDD single band, and high power UE operation (power class 1) for fixed-wireless/vehicle-mounted use cases in a single NR band; RF from RAN4#112

[6] RP-241678 - New WID: Rel-19 High power UE (power class 1.5 or 2) for Dual Connectivity (DC) combinations of LTE band(s) and NR band(s); RF from RAN4#112

[7] RP-241679 - New WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR SUL (supplementary uplink); RF from RAN4#112

[8] RP-241681 - New WID: Rel-19 High power UE (power class 2) and high power operation (power class 1) for fixed-wireless/vehicle-mounted use cases in a single LTE band; RF from RAN4#112