3GPP TSG-RAN WG4 Meeting #112    	R4-2412008

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Title: 	UE RF Enh 4: Increasing UE transmission power

Agenda Item:	8.1.1.1.3

Source: 	Nokia

Document for:	Approval

# Introduction

In [1] UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 there is one objective which is discussed in this contribution.

Secondly, the feature of increasing high power limit was introduced since Rel-17. After Rel-18, now it can only support the cases of one FDD UL carrier with PC3 plus one TDD UL carrier with PC2, and one FDD UL carrier with PC3 plus one UL carrier with PC5. The current specification structure is to define the total maximum output power over the whole band combination. Under that structure, the different power class combinations should be introduced separately for a given power class of band combination, which is the sum of component UL CC powers. Such approach causes the inefficiency and high workload or delay to introduce the band combinations with high power.

Besides, with the introduction of 3Tx across 2 bands, the additional dimension for consideration appears. In the future, the combinations of bands, power classes and transmit antenna (Tx) numbers might need be considered. So in order to optimize the standardization of high power band combinations, the general framework of supporting increasing UE power limit would be needed.

# Discussion

In previous RAN4 meeting the HPUE WF [1] also addressed increasing high power limit topic and a good progress was made with a help of a strong leadership of the moderator!

Only one open issue remains, see below.

## The scenarios to be considered in Rel-19

Proposals:

  - Proposal 1: (Samsung)

- For 2Tx, the following scenario can be considered for NR-CA/EN-DC.

| Indicated PC for A-B (2Tx in total) | PC for band A of A-B | PC for band B of A-B | From which release increasing high power limit feature supported | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3(FDD or TDD) | PC2 (FDD or TDD) | Support from Rel-19 | One CC on band A, 2CC on band B |
| PC2 | PC3(FDD or TDD) | PC2(FDD) | Support from Rel-19 | One CC per band |

- For 3Tx, the following scenarios may could be considered for NR-CA/EN-DC.

(Note the analysis is based on the implementation feasibility)

| Indicated PC for A-B (3Tx in total) | PC for band A of A-B (1Tx) | PC for band B of A-B (2Tx) | The total power (dBm) | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | 27.8 | One CC per band One CC in band A, 2CC on band B (if this scenario would be introduced for 3Tx) |
| PC1.5 | PC3 | PC1.5 | 30.0 | One CC per band; For FWA only One CC in band A, 2CC on band B (if this scenario would be introduced for 3Tx); For FWA only |

  - Proposal 2: Consider PC3+PC2 and PC3+PC1.5 for 3Tx as scenarios for increasing power limit Rel-19. (vivo)

  - Proposal 3: (Qualcomm)

- higherPowerLimit-r17  is enabled for any standardized inter-band band combination. Cases where there is concern about exceeding local regulation are treated separately as exceptions.

- RAN4 to rely on network operators (carriers) to identify if local regulatory limits are exceeded when higherPowerLimit-r17 is enabled for their band combination.

- RAN4 to evaluate best method to identify corner case UL CA configuration (band combination + power class aggregation) where higherPowerLimit-r17 may NOT be used due to local regulation. Options:

    - Set up an NS case

    - Rely on PEMAX,CA, the value indicated by p-NR-FR1 or by p-UE-FR1

    - Maintain an exception list in 38.101-x for UL CA configurations where higherPowerLimit-r17 may NOT be used

- Remove references to power class or TR duplexing type from sections of the standard that enable use of higherPowerLimit-r17

  - Proposal 4: (Skyworks)

- Regardless of the down selected increased power cases for Release 19, to be future proof, the work should address PCmax equations in order to support:

- Power increase for 3 levels (0.5/1/1.8dB).

- Power boosting on one or two bands.

- Increased power with power boosting additionally.

- Increased power of 3dB should not be allowed and a higher band combination power class signalled instead.

  - FFS on how to deal with cases with 3dB increases above PC1.5

  - FFS if this rule applies to cases where the 3dB increase is the result of power boosting.

  - Proposal 5: The following power class configuration could be considered in Rel 19 for UE increasing high power limit. (Xiaomi)

- PC3 (TDD/FDD) +PC1.5 indicating PC1.5 with 3Tx

- PC2 (TDD with TxD) +PC3 (TDD/FDD) indicating PC2 with 3Tx

  - Proposal 6: Consider Option 3 as the scenarios of increasing higher power limit in Rel-19. (LGE)

- For PC2 2Tx inter-band NR CA and ENDC:

- Inter-band with intra-band UL CA in one of the bands

- For HPUE 3Tx inter-band NR CA and ENDC:

- PC2 band combination of PC3+PC2 with single carrier in each band.

- PC1.5 band combination of PC3+PC1.5 with single carrier in each band.

- PC1.5 band combination of PC2+PC1.5 with single carrier in each band.

- Note: Only PC3 is considered for LTE FDD in EN-DC

  - Proposal 7: Both FWA and handheld UE can support PC3+PC1.5 with increasing UE high power limit feature in Rel-19 if technical issues are not provided. (DCM)

  - Proposal 8: (Huawei)

- Consider the following list of scenarios for increasing UE transmission power limit as shown in Table 1 below. And Increasing the total Tx power limit beyond PC1.5 is only for FWA UEs, not for handheld UEs.

Table 1: A list of applicable scenarios for increasing UE transmission power limit

| CA power class | Power class configuration Band A + Band B |  | 2Tx | 3Tx |
| --- | --- | --- | --- | --- |
| PC3 | PC3 | PC5 | Completed in R18 | N/A |
| PC2 | PC2 | PC3 | Completed in R17 | R19 |
|  |  | PC5 | R19 | R19 |
| PC1.5 | PC1.5 | PC2 | N/A | R19 |
|  |  | PC3 | N/A | R19 |
|  |  | PC5 | N/A | R19 |

  - Proposal 9: (ZTE)

- To consider the following additional eligible PC2 2Tx inter-band NR CA and ENDC to enable increasing higher power limit in Rel-19:

- Inter-band with intra-band UL CA in one of the NR band

- To consider the following additional eligible HPUE 3Tx inter-band NR CA and ENDC with up to 3CC in UL bands to enable increasing higher power limit in Rel-19:

- PC2 band combination of PC3+PC2

- PC1.5 band combination of PC3+PC1.5

- PC1.5 band combination of PC2+PC1.5

Note: Only PC3 is considered for LTE FDD in EN-DC

  - Proposal 10: (CATT)

    - For 2Tx transmission, there is no new scenarios for increasing UE transmission power

    - For 3Tx transmission, there are three new scenarios for increasing UE transmission power as shown in the table below:

| Indicated PC for A-B (3Tx in total) | PC for band A of A-B (1Tx) | PC for band B of A-B (2Tx) | The total power (dBm) | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | 27.8 | One CC per band |
| PC1.5 | PC3 | PC1.5 | 30.0 | One CC per band; For FWA only |
| PC1.5 | PC2 | PC1.5 | 30.8 | One CC per band; For FWA only |

Way forward: FFS in future meetings

As can be seen quite many proposals was done, ten in fact. Most of these proposals are likely inclined towards a certain existing implementation which is natural but then again problematic for future proof standardization work. If RAN4 chooses a proposal which is very restrictive then we will have yet another WI in REL20 and so on.

Therefore or preference is Proposal 3 which is most future proof and produces biggest number of band combinations that can benefit from increasing Tx power.

Proposal. Choose Proposal 3 [1] as a WF

- higherPowerLimit-r17  is enabled for any standardized inter-band band combination. Cases where there is concern about exceeding local regulation are treated separately as exceptions.

- RAN4 to rely on network operators (carriers) to identify if local regulatory limits are exceeded when higherPowerLimit-r17 is enabled for their band combination.

- RAN4 to evaluate best method to identify corner case UL CA configuration (band combination + power class aggregation) where higherPowerLimit-r17 may NOT be used due to local regulation. Options:

    - Set up an NS case

    - Rely on PEMAX,CA, the value indicated by p-NR-FR1 or by p-UE-FR1

    - Maintain an exception list in 38.101-x for UL CA configurations where higherPowerLimit-r17 may NOT be used

- Remove references to power class or TR duplexing type from sections of the standard that enable use of higherPowerLimit-r17

# Conclusion

In this contribution we have evaluated different alternatives for scenarios to be considered in Rel-19 in increasing high power limit topic [1]. Our preference is Proposal 3.

Proposal. Choose Proposal 3 [1] as a WF

- higherPowerLimit-r17  is enabled for any standardized inter-band band combination. Cases where there is concern about exceeding local regulation are treated separately as exceptions.

- RAN4 to rely on network operators (carriers) to identify if local regulatory limits are exceeded when higherPowerLimit-r17 is enabled for their band combination.

- RAN4 to evaluate best method to identify corner case UL CA configuration (band combination + power class aggregation) where higherPowerLimit-r17 may NOT be used due to local regulation. Options:

    - Set up an NS case

    - Rely on PEMAX,CA, the value indicated by p-NR-FR1 or by p-UE-FR1

    - Maintain an exception list in 38.101-x for UL CA configurations where higherPowerLimit-r17 may NOT be used

- Remove references to power class or TR duplexing type from sections of the standard that enable use of higherPowerLimit-r17

# 4	References

[1] RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#104

[2] R4-2410750, WF on higher power UE, RAN4#111, Samsung