3GPP TSG-RAN WG4 Meeting #112														R4-2412009

Maastricht, Netherlands, 19-23 August 2024

Agenda item:			8.1.1.2.1

Source:	Nokia

Title:	Power domain enhancements for single carrier

Document for:	Approval

# Introduction

The following MPR reduction work has been approved as part of the RAN4 Rel-19 [1]. One motivation for this work is generally relaxing requirements for UEs that have such frequency allocation that makes meeting the requirements less important e.g. inner allocation, UE channel bandwidth within BS bandwidth. Based on RAN4 discussions another motivation is lower complexity UEs such as RedCap UEs that may not be able to meet all the current requirements. Such UEs would need to be operated so that they always have allocation allowing relaxed requirements e.g. the UE channel bandwidth is within BS channel bandwidth.

# Discussion

In RAN4#111, the following WF was proposed in [2]:

- Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects - The edge of the extended UE BW, i.e. the UE CBW plus the shifted frequency symmetrically at each side of the UE CBW, should be inside BS CBW or at least aligned with the BS CBW edge - ① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW - ② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW - ③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB - ④ FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW Other mechanisms are not precluded

Figure 1: Proposed approach to convert full RB allocation in UE CBW to “inner RB allocation”

Relaxing the Adjacent Channel Leakage Ratio (ACLR) requirement, which is currently based on the same bandwidth as the User Equipment (UE), can significantly increase interference across a wider frequency range.

Similarly, relaxing requirements on spurious emissions and out-of-band emissions can also lead to increased interference over a wider frequency range and may not be possible from regulation perspective.

In summary, relaxing ACLR and other emission limits can significantly increase interference levels, potentially impacting both intra-cell and inter-cell performance. This requires careful analysis and mitigation strategies to ensure efficient and reliable network operation.

Observation 1: Relaxation of ACLR/SEM/spurious emissions limits will lead to higher interference spread over a wide bandwidth (e.g., intra-cell interference within BS channel bandwidth, intra/inter operator BW interference).

Proposal in [2] suggests to introduce “extended UE BW” so that on each side of the UE BW there is additional added band width equal to ½ of the UE BW to mitigate the effects of emissions.

One option to introduce the “extended UE BW” could be utilizing the guard band approach already existing in the specifications. Guard band would just need to be larger e.g. equal or smaller to ½ of the UE BW. In this case, the UE channel BW may be the same as UE initial CBW (i.e., additional guard-band <½ UE BW), or a larger CBW defined in the specs (e.g., UE CBW=2* UE initial CBW with the example of guard band equal to ½ of the UE initial BW). This could be obtained e.g. by specifying new configuration of number of PRBs for maximum transmission bandwidth in the UE channel bandwidth or for the additional guard band. Hence, new definition of the extended UE CBW would not be necessary.

Observation 2: The new definition of the extended or aggregated UE CBW may not be necessary, and RAN4 could use UE CBW and the additional guard band terminology similar to the specification.

For instance, BS indication for an additional guard band configuration (wider configured guard band), as shown in the example of Figure 2, provides MPR enhancement with all identified scenarios and restricts the additional leakages/interferences with higher transmit power to be well confined within a small pre-defined bandwidth (i.e., avoid leakage spreading over a wide bandwidth). In such case, ACLR measurement bandwidth is equal to the maximum configured transmission BW (i.e., UE CBW excluding the total configured guard bands), and ACLR/SEM can be applicable from the outer edge of the guard bands as the legacy case.

Such MPR enhancement technique maintains the same system performance with all identified scenarios even for others with co-existence, avoids gNB scheduling constraints or system performance impact, and allows MPR and UL coverage enhancement even with current ACLR/SEM/spurious emissions limits and UE RF.

Figure 2: Additional configurable guard-band for MPR enhancement

Extra Guard band

Figure 3: In the left legacy spectrum occupation, on the right 10% additional extra guard band

Figure 3 shows that for the same spectral occupation with 10% of additional guard band for 10 MHz UE CBW example, there is 3dB additional margin for the ACLR in the previous transmission spectrum example compared to the legacy, and this would provide additional room to enhance MPR and boost the power.

Observation 3: Defining an additional guard band configuration allows MPR reduction and/or additional power boost capability even with the current ACLR/SEM/spurious emissions limits and UE RF/hardware.

Proposal 1: RAN4 could specify at least one guard band configuration larger than the minimum guard band (i.e., additional guard band) or a maximum transmission bandwidth smaller than the legacy maximum transmission bandwidth as shown in Figure 2, and the associated enhanced MPR/power boost.

The UE CBW and the additional guard band should be strictly within BS CBW, so it is not feasible to have any unrestricted emissions beyond BS CBW at least from regulatory, interference and co-existence perspectives. Hence, the additional bands at both edges should be well confined within BS CBW, and this can be achieved by non-symmetric guard band at both ends, or by using a symmetrical additional band respecting this constraint.

Proposal 2: It is not feasible to allow any unrestricted emissions or consider any additional guard band size beyond BS CBW at least from regulatory, interference and co-existence perspectives. RAN4 should restrict the additional band size and UE CBW to strictly lies within BS channel BW.

Perhaps the most promising and easiest way of achieving the goal of this work would be to define the boundary for OOB to be same as edges of basestation channel bandwidth, no matter what is the UE channel bandwidth with in, see Figure 4 option 2.

Figure 4: OOB style in Rel 18 versus OOB candidate for Rel 19

Observation 4: Defining UE requirements according to the BS channel bandwidth would allow reduced MPR as only IBErequirement would be valid within BS channel bandwidth.

Proposal 3: For MPR reduction the out of boundary  is defined according to BS CBW as shown in option 2 of the figure 2. ACLR, SEM and spurious emissions would be define as of today but based on  BS channel bandwidth.

# Conclusion

In this paper we discuss our view on the study MPR reduction. Observations and proposals are given as follow:

Observation 1: Relaxation of ACLR/SEM/spurious emissions limits will lead to higher interference spread over a wide bandwidth (e.g., intra-cell interference within BS channel bandwidth, intra/inter operator BW interference).

Observation 2: The new definition of the extended or aggregated UE CBW may not be necessary, and RAN4 could use UE CBW and the additional guard band terminology similar to the specification.

Observation 3: Defining an additional guard band configuration allows MPR reduction and/or additional power boost capability even with the current ACLR/SEM/spurious emissions limits and UE RF/hardware.

Proposal 1: RAN4 could specify at least one guard band configuration larger than the minimum guard band (i.e., additional guard band) or a maximum transmission bandwidth smaller than the legacy maximum transmission bandwidth as shown in Figure 2, and the associated enhanced MPR/power boost.

Proposal 2: It is not feasible to allow any unrestricted emissions or consider any additional guard band size beyond BS CBW at least from regulatory, interference and co-existence perspectives. RAN4 should restrict the additional band size and UE CBW to strictly lies within BS channel BW.

Observation 4: Defining UE requirements according to the BS channel bandwidth would allow reduced MPR as only IBErequirement would be valid within BS channel bandwidth.

Proposal 3: For MPR reduction the out of boundary  is defined according to BS CBW as shown in option 2 of the figure 2. ACLR, SEM and spurious emissions would be define as of today but based on  BS channel bandwidth.

# 4	Reference

[1] RP-241656  “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, 3GPP TSG RAN Meeting #104

[2] R4-2410749 “WF on power domain enhancements”, TSG-RAN WG4 Meeting #111, May, 2024

[3] 3GPP, 38.101 v	NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, TS 38.101-1 V18.1.0 (2023-03)