3GPP TSG-RAN WG4 Meeting #112    	R4-2412010

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Title: 	UE RF Enh 4 MPR applicability for FR1 intra-band UL CA

Agenda Item:	8.1.1.1.3

Source: 	Nokia

Document for:	Approval

# Introduction

In [1] UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 there is objective for MPR applicability for FR1 intra-band contiguous UL CA which is discussed in this contribution.

# Discussion

There are three FFS in current WF [2] on which MPR table to use when only 1 UL CC is activated. In our view these FSSs can be removed. Concerning the fourth FFS spurious emissions/ACLR/SEM requirements need to be kept outside BS Channel bandwidth. More discussion is needed on fourth bullet as it is different case when frequencies outside aggregated BW are with in BS channel BW or outside BS BW.

## 2.1. MPR applicability for FR1 intra-band contiguous UL CA

Way forward:

- For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated.

  - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C.

  - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated.

  - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

  - FFS whether spurious emissions/ACLR/SEM are kept for aggregated CBW for CA

# Conclusion

In this contribution we have expressed our view on some of the open issues on MPR applicability for FR1 intra-band contiguous UL CA.

# 4	References

[1] RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#104

[2] R4-2410749, WF on power domain enhancements, RAN4#111