3GPP TSG-RAN WG4 Meeting #112    	R4-2412011

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Title: 	UE RF Enh 4: 6Rx for handheld and FWA dRib

Agenda Item:	8.1.1.3.1

Source: 	Nokia

Document for:	Approval

# 1	Introduction

In this contribution we discuss 6Rx for handheld and FWA dRib issue mentioned in WF [1].

# 2	Discussion

## General considerations for specifying ΔRIB,6R value

Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA

Way Forward: Further discuss the following options.

-	Option 1: RAN4 needs to determine whether to define different ΔRIB,6R value for handheld UE and FWA separately

-	Option 2: Same value for handheld UE and FWA

-	Option 3: Different value for handheld UE and FWA

Whether to define different value for handheld UE and FWA has multiple aspects to consider. One value ΔRIB,6R is simpler to capture in specs but harder to agree. If one value is targeted should it be so relaxed that every handheld UE can meet that or should it be such that FWA UE can easily meet that but it is more difficult for hand held UE to meet. Perhaps it is better to define separate values of ΔRIB,6R to both FWA and handheld UE. This way ΔRIB,6R would be more technically correct. As feasibility of 6Rx for handheld is still open we propose.

Proposal 1: Define separate values of ΔRIB,6R to both FWA and handheld UEs. First define value for FWA.

## Sub-topic 1-2:	ΔRIB,6R values for handheld UE and FWA

Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA

Way Forward: Further discuss the following options for the ΔRIB,6R values. Ultimately, the decision to specify separate or same requirements for handheld UE and FWA will be dependent on outcome of Issue 1-1-2.

-	Option 1: Consider range of ΔRIB,6R in brackets for each case below for further discussion.

| Operating bands | ΔRIB,6R for FWA | ΔRIB,6R for Handheld |
| --- | --- | --- |
| n77, n78, n79, [n104] | [-3.0 ~ -3.3] dB | [-3.0 ~ -3.2] dB |
| n41 | [-3.0 ~ -4.0] dB | [-3.0 ~ -4.0] dB |

-	Option 2: Consider average value of ΔRIB,6R from company proposals for further discussion.

-	Option 3: Other options are not precluded.

In previous meeting we proposed [2] for n41 FWA that ΔRIB,6R is 4 dB.

In this paper we provide similar analysis for 3.5 GHz.

If we take current relaxations into account and apply curve fitting as in Figure 1 we get

ΔRIB,6R = -4.77 (ideal ΔRIB,6R) + 1.4 = - 3.378 = 3.4 dB

Figure 1: Relaxation vs number of receivers.

Hence our proposal for FWA ΔRIB,6R is

Proposal 2: FWA ΔRIB,6R is as defined in Table 1

| Operating bands | ΔRIB,6R for FWA |
| --- | --- |
| n77, n78, n79, [n104] | -3.4 dB |
| n41 | -4.0 dB |

Table 1: ΔRIB,6R for FWA

# 3	Conclusion

In this contribution, we have made the following proposals.

Proposal 1: Define separate values of ΔRIB,6R to both FWA and handheld UEs. First define value for FWA.

Proposal 2: FWA ΔRIB,6R is as defined in Table 1

| Operating bands | ΔRIB,6R for FWA |
| --- | --- |
| n77, n78, n79, [n104] | -3.4 dB |
| n41 | -4.0 dB |

Table 1: ΔRIB,6R for FWA

# 4 References

[1] R4-2410751, WF on requirements for 6Rx, RAN4#111

[2] R4-2408725, 6Rx for handheld and FWA UE Reference sensitivity requirements, RAN4#111, Nokia