3GPP TSG-RAN WG4 Meeting #112    	R4-2412012

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Title: 	UE RF Enh 4: 6Rx for handheld and FWA UE SRS imbalance

Agenda Item:	8.1.1.3.4

Source: 	Nokia

Document for:	Approval

# 1	Introduction

In this contribution we discuss 6Rx for handheld and FWA UE SRS imbalance issue also mentioned in WF [1].

# 2	Discussion

## 2.1	SRS imbalance reporting

There have been a lot of discussions on SRS imbalance coming from the UE hardware limitations and its impact on network performance already in Rel-18 both in RAN4 and RAN1 as stated in [3]. Some solutions proposed in Rel-18 were UE self-compensation of SRS imbalance when not in power limited scenario and also some signaling to assist NW to overcome the imbalance were proposed. Furthermore [3] raised a valid point that even if UE can compensate SRS imbalance perfectly at UE conductive antenna port it does not guarantee that that when signal is at gNodeB receiver such a balance is preserved. There are so many factors that are not controllable which cause imbalance that solution is deemed to fail. Furthermore in [4] it was shown that if there would be a signaling solution to indicate gNodeB on UE SRS imbalance with a conclusion of “there is no DL performance benefit in the UE reporting SRS offset values and gNB performing compensation of IL compared to the UE performing IL compensation alone”.

In previous RAN4 meeting we proposed not continue the discussion on how to solve the SRS imbalance issue. We propose it again.

Proposal 1: RAN4 should not continue the discussion on how to solve the SRS imbalance issue.

If WF[1] two issues were mentioned for this topic

## Sub-topic 4-1:	General considerations for SRS IL imbalance issue

Issue 4-1-2: Initial Considerations for SRS IL imbalance issue

Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL.

-	Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL.

-	Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation.

## Sub-topic 4-2:	SRS IL imbalance issue solutions

Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue

Way forward: After determining the set of initial considerations for the study, discuss candidate solutions considering the UE behaviour in terms of the IL imbalance compensation and whether the UE can compensate the IL imbalance for AS-SRS for all power levels and whether NW and UE have consistent understanding for the possible compensation.

WF discusses a initial considerations phase and subsequent study to discuss candidate solutions. If some progress in this topic is desired which in our opinion is not necessary, a deadline should be set to the initial considerations phase. For example, RAN4#112bis.

Observation 1: a deadline should be set to the initial considerations phase. For example, RAN4#112bis.

## 2.2	Release independence

If WF[1] agreement was to postpone release independence discussions until later.

Issue 1-1-3: Release independence of ΔRIB,6R value

Way Forward: Follow RAN4 Chair guidance and defer the release independence discussions until the requirements are stable.

Hence below observations can be ignored in this meeting.

The 6-layer MIMO UE capability was discussed under Rel-18 TEI in RAN1 in 2023, and the conclusion was that this is a bigger item requiring RAN4 work and could be revisited with a separate WI in Rel-19. The new UE capability and configuration signalling needs to be introduced in 38.331, and the new UE capability needs to be introduced in 38.306, and RAN1 should be consulted for possible RAN1 specification modifications; conducting this work would need to be Rel-19 work as Rel-18 is already closed.

Observation 2: If RAN4 decides to specify requirements for 6-layer MIMO then it may have to be release independent from Rel-19.

RAN4 has defined 8Rx to be release independent from Rel-17 and 4Rx from Rel-15.There is not technical reason why 6Rx without 6-layer MIMO cannot be release independent from Rel-15.

Observation 3: 6Rx REFSENS without 6-layer MIMO can be release independent from Rel-15.

# 3	Conclusion

In this contribution, we have made the following proposals.

Proposal 1: RAN4 should not continue the discussion on how to solve the SRS imbalance issue.

If proposal 1 is not acceptable then

Observation 1: a deadline should be set to the initial considerations phase. For example, RAN4#112bis.

Observation 2: If RAN4 decides to specify requirements for 6-layer MIMO then it may have to be release independent from Rel-19.

Observation 3: 6Rx REFSENS without 6-layer MIMO can be release independent from Rel-15.

# 4 References

[1] R4-2410751, WF on requirements for 6Rx, RAN4#111

[2] R4-2404793, Discussion for 6 Rx, RAN4#110bis, LG Electronics France

[3] R4-2404935, Views on 6Rx for handheld and FWA UE, RAN4#110bis, Samsung

[4] R4-2405083, UE SRS IL imbalance for 6Rx UE, RAN4#110bis, Shanghai Chen Si Electronics