3GPP TSG-RAN WG4 Meeting #112    	R4-2412013

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Title: 	Discussion intra-band non-collocated deployment Phase2 UE Capability/UE behavior and network signaling

Agenda Item:	8.5.2.2

Source: 	Nokia, NSB

Document for:	Approval

# Introduction

In previous RAN4 meeting #110bis a WF was approved [1]. In this contribution we express our views for open issues on UE Capability/UE behavior and network signaling issues.

# 2	Discussion

# Topic #2:  Rel-19 Type-4

## Sub-topic 2-2: UE Capability(s)/UE behavior and network signalling for Type 4 EN-DC/NR-CA

< Issue 2-2-1:  New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) >

Way Forward:

- Continue further discussion on the following two options and conclude it in the next meeting.

  - Option 1. Add new UE capabilities for Type 4a and Type 4b support indication.

  - Option 2. Add one new UE capability for Type 4 (no distinction between Type 4a and 4b) support indication.

Nokia view: In EN-DC operation Type 4a UE has 4 receivers for NR and 2 receivers for LTE. EN-DC operation Type 4b UE has 4 receivers for NR and 4 receivers for LTE. Four receivers for LTE is an optional capability for band 42 and is signalled separately, therefore in case of non-collocated operation it is enough UE to just indicate support for Type 4 as number of LTE receivers is indicated with other existing signalling.

Existing UE capability to inform NW that UE has 4 antenna ports is captured below from 36.331 .

fourLayer-TM3-TM4

Indicates UE supports 4-layer MIMO and 2-bit RI indications with TM3 and TM4 for any band in the band combination.

Furthermore from 36.101 we can find

Additional requirements apply for UE(s) equipped with four Rx ports. These additional requirements also apply for supported band combinations for which the UE can operate using up to four Rx ports while configured with carrier aggregation.

Proposal: it is enough UE to just indicate support for Type 4 as number of LTE receivers is indicated with fourLayer-TM3-TM4 signalling.

< Issue 2-2-2:  Whether to support Type 2 capability by UE having Type 4 capabilities >

Agreement:

- If UE reports the Type 4 capability, Type 2 capability shall be deemed as support by default regardless of whether UE indicates Type-2 capability or not.

  - If UE reports the Type 4 capability, it is not necessary for UE to report Type 2 capabilities.

  - If the issue is identified for these bullets, RAN4 will revisit them.

Nokia view: No issues identified for Type 4 UE to support Type 2 by default hence that agreement is valid. Type 4 UE needs to support type 2. But if only Type 4 capability is indicated, it cannot be recognized by legacy gNB. Indication of both type 4 and type 2 is necessary due to legacy BS issue however this is up to RAN2 to decide.

< Issue 2-2-3:  New BS Signalling to switch between Type 4a/4b and Type 1(collocated) >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

Nokia view: BS signalling to switch between Type 4a/4b and Type 1(collocated) is needed.

< Issue 2-2-4:  New BS Signalling to switch between Type 4a/4b and Type 2 >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

Nokia view: Can potentially use exiting maxMIMO-layer signaling to switch between Type 4a/4b and Type 2

< Issue 2-2-5: When to inform RAN2 the demand on new UE capability(s) and new BS signalling(s) >

Way Forward:

- Given the discussions on UE capabilities and network signalling, it is not hurry to send this.

Nokia view: RAN4 needs to decide what need to be signalled between network and UE and UE capability related aspects before sending an LS to RAN2.

< Issue 2-2-6:  UE behavior between Type 1, Type 2 and Type 4a/4b with new BS signalling >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) and BS signalling for Type 4.

- And then, continue further discussion on BS signalling/UE behavior in the next meeting.

Nokia view:

The switching between Type 1 and Type 2 has been specified in R18.

The switching between Type 2 and Type 4 is in Issue 2-2-4. We believe existing maxMIMO-layer signaling can be used.

The switching between Type 1 and Type 4 requires new BS signalling as in Issue 2-2-3. On top of that, UE may operate with different MIMO layers when switching from Type 4 to Type 1:

- Type 1 with up to 8-layer MIMO. This is a new Type 1 operation for Type 4 capable UE, but not supported by Type 2 capable UE. And this requires the UE to support maximum 8 MIMO layer.

- Type 1 with up to 4-layer MIMO. This can be supported by both Type 4 capable UE and Type 2 capable UEs. As 4-layer is the working assumption for NR, this will be the baseline Type 1 operation. (but we can discuss if 8-layer Type 1 or 4-layer Type 1 is the default Type 1 when switching from Type 4)

- Type 1 with up to 2-layer MIMO. This operation is available only for legacy R16/17 Type 2 UEs in EN-DC. For Type 4 UE, a UE shall not operate in Type 1 with up to 2-layer MIMO i.e. this option is not valid for switching from Type 4 to Type 1.

We should discuss the UE behaviour when switching from Type 4 to Type 1 for above Type 1 operations.

# 3	Conclusion

In this contribution we have expressed our views for intra-band non-collocated deployment Phase2 Capability/UE behavior and network signaling open issues from WF.

# 4	References

[1] R4-2406627	WF on NonCol_intraB_ENDC_NR_CA, RAN4#110bis