3GPP TSG-RAN WG4 Meeting # 112						R4-2412014

Maastricht, Netherlands, August 19 – 23, 2024

Source:	LG Electronics

Title:	NR side link requirements and European regulation for band n47

Agenda item:	8.3.2.1 System parameters

Document for:	Approval

# Introduction

Since the NR side link requirements were defined for Rel-17 the regulation in Europe for band n47 has been updated. This contribution discussed these updates and the impacts to NR side link minimum performance requirements.

# Discussion

European regulation for band n47 is defined in the ETSI EN 302 571, Intelligent Transport Systems (ITS); Radiocommunications equipment operating in the 5 855 MHz to 5 925 MHz frequency band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU [1] and the latest version is V2.1.1.

This version of the Harmonized standard includes following requirements that need to be further investigated for NR side link when operated in EU when new requirements are introduced in Rel-19..

## Transmitter frequency stability

Observation 1: it is required that UE operates on channels that have maximum channel bandwidth of 10MHz

Observation 2: it is required that UE operates on channels with centre frequency listed above with +/-20ppm frequency error

## Transmitter unwanted emissions

Observation 3: The out-of-band domain is defined as ±250 % of the channel bandwidth, and then the out-of-band (OOB) domain for the 5 GHz ITS frequency band is 5 835 MHz to 5 855 MHz at the lower part and 5 925 MHz to 5 945 MHz at the higher part of the frequency band.

Observation 4: Maximum EIRP power density for UE emissions into the out-of-band domain is -30dBm/MHz.

## Transmitter spectrum mask

Observation 5: Spectrum mask is defined for 10MHz channel bandwidth only

Observation 6: Spectrum mask is only applicable within 5855 to 5925MHz.

Figure below is combining the observations 3, 4, 5, 6.

Figure 1. European OOB and SEM requirements.

Observation 7: Considering the observations 3, 4, 5, 6 there is only one 10+10MHz UL NC-CA combination, as shown in Figure 2, where IM3-products fall inside the band n47. In all other cases -30dBm/MHz OOB or spurious emission requirement needs to be met.

Figure 2. European regulation and UL non-contiguous carrier aggregation.

# Conclusion

Following observations are made:

Observation 1: it is required that UE operates on channels that have maximum channel bandwidth of 10MHz

Observation 2: it is required that UE operates on channels with centre frequency listed above with +/-20ppm frequency error

Observation 3: The out-of-band domain is defined as ±250 % of the channel bandwidth, and then the out-of-band (OOB) domain for the 5 GHz ITS frequency band is 5 835 MHz to 5 855 MHz at the lower part and 5 925 MHz to 5 945 MHz at the higher part of the frequency band.

Observation 4: Maximum EIRP power density for UE emissions into the out-of-band domain is -30dBm/MHz.

Observation 5: Spectrum mask is defined for 10MHz channel bandwidth only

Observation 6: Spectrum mask is only applicable within 5855 to 5925MHz.

Observation 7: Considering the observations 3, 4, 5, 6 there is only one 10+10MHz UL NC-CA combination where IM3-products fall inside the band n47. In all other cases -30dBm/MHz OOB or spurious emission requirement needs to be met.

Proposal 1: It’s proposed that above mentioned observations are taken into account when Rel-19 side link RF requirements are discussed in RAN4-

# References

[1] ETSI EN 302 571, Intelligent Transport Systems (ITS); Radiocommunications equipment operating in the 5 855 MHz to 5 925 MHz frequency band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU