3GPP TSG-RAN WG4 Meeting #112	R4-2412015

# Maastricht , Netherlands, 19th – 23th Aug, 2024

Source:	China Telecom

Title:	Discussion on deployment scenarios and spectrum usage for AIoT

Agenda item:	8.20.2.1

Document for:	Discussion

1	Introduction

In the last RAN4#111 meeting, some issues for AIoT deployment scenarios and spectrum usage were discussed, and preliminary conclusions were reached[1]. In this contribution, we discuss our consideration on the remaining issues.

2	Discussion

2.1 Deployment scenario

Issue 2-1-1: deployment scenarios for D1T1

Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors.

Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB

Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware

Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting)

Agreement:

- Consider only adjacent channel co-existence evaluation for in-band deployment scenario for NR and AIOT

- Encourage companies to provide the simulation results for option 1-1 and 1-2

  - FFS on co-site scenario (option 2-1 and 2-2)

According to RAN1's latest conclusion[2], 900MHz is mandatory frequency band, and 2GHz is optional considering the strict sensitivity requirement of AIoT devices and weaker network coverage performance than 900MHz. However, 2GHz is a practical method to achieve co-location of Legacy NR gNB and AIoT reader and CW indoor for option 2-1 and 2-2, especially its useful for some indoor short-distance communication application scenarios.

Observation 1   Co-site scenario in option2-1 and 2-2 is realizable, such as the use of 2GHz frequency band indoors.

From the perspective of deploying AIoT networks in the future, for some scenarios which have deployed legacy NR gNB indoor, on the one hand, the low-cost transformation and upgrading of existing network equipment to support AIoT capability should be considered, on the other hand, there may be no spare space to redeploy AIoT equipment such as factories, production lines. Therefore, co-site of legacy NR and AIoT infrastructure (option 2-1 and 2-2) is very important and useful. Besides, some hardware sharing may be cost friendly, especially probably be a good choice for device2b because of its active communication mode which is similar to legacy NR, but new hardware would lead to better performance. So both options 2-1 and 2-2 should be further considered and discussed in the same priority.

Observation 2   Co-site scenario is important and useful in some cases.

Proposal 1  Both option 2-1 and 2-2 should be further studied in the same priority.

In addition, coexistence between AIoT systems deserves attention and discussion, and is as important as coexistence between legacy NR and AIoT systems. The former involves the interference between multiple tags and multiple AIoT readers/CW, and these signals have the same power level, while the latter is the coexisting between strong NR signals and weak AIoT signals. Also there will be more types of AIoT devices. From the frequency band perspective, including 900MHz and 2GHz at least, and may involve different operators' frequency band. From the types of AIoT devices perspective, it involves device1, device 2a/2b and possible different types of AIoT readers/CW. Consider the above factors, the coexistence between A-IoT systems including necessity, feasibility and details need to be studied and evaluated.

Proposal 2  The coexistence between AIoT systems including necessity, feasibility and details need to be studied and evaluated.

2.2 Spectrum usage

Issue 2-2-3: Inside and outside topology for D1T1

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied.

Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum

Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Agreement:

- Use inside topology as starting point for co-existence evaluation (case 1-1, case 1-2) for calibration.

- Further discuss the difference of outside topology (case 1-4) from co-existence study perspective.

Inside topology including case 1-1 and case 1-2 is the starting point. Different from inside topology, the case of CW outside topology shows uncertainty and flexibility. CW can be a new device, a cellular legacy UE, or others, which may bring different co-existence situation compared to the CW inside topology.

When CW is a new type device, the specific capability, function and complexity are unknown, and its location is relatively flexible depending on the specific scenario requirements, so the interference between new CW device and legacy NR system including legacy gNB and UE seems difficult to control, which requires future discussion. When CW is an outside topology legacy UE, it interferes with the legacy UEs and it is also necessary to consider whether the outside UE will communicate and collaborate with the inside topology BS to solve the  interference.Therefore, the impact of the CW outside topology needs to be further studied.

Observation 3  The CW outside topology presents different coexistence situation compared to the CW inside topology.

Proposal 3  The case of the CW outside topology needs to be further studied.

3	Conclusion

In this paper we provided our views on the deployment scenarios and spectrum usage for AIoT.

Observation 1   Co-site scenario in option2-1 and 2-2 is realizable, such as the use of 2GHz frequency band indoors.

Observation 2   Co-site scenario is important and useful in some cases.

Observation 3  The CW outside topology presents different coexistence situation compared to the CW inside topology.

Proposal 1  Both option 2-1 and 2-2 should be further studied in the same priority.

Proposal 2  The coexistence between AIoT systems including necessity, feasibility and details need to be studied and evaluated.

Proposal 3  The case of the CW outside topology needs to be further studied.

Reference

- R4-2410567	WF on co-existence study for ambient IoT and NR/LTE.

- R1-2405696	Session notes for 9.4 (Study on solutions for Ambient IoT in NR)