3GPP TSG-RAN WG4 Meeting #112	R4-2412021

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	On General Aspects of AI/ML for Air Interface

Agenda item:	8.17.1

Document for:	Discussion

Introduction

Even though more and more of the open issues in the working item on AIML for the Air interface (NR_AIML_Air) are discussed in the use-case-specific manner, there are several general aspects that needs further discussion, as follows:

- Clarification of the testing goals of the general RAN4 testing framework

- Approaches and feasibility of post-deployment model testing and verification

- Support for AI/ML model and dataset logistics for the RAN4 study

In this contribution, we share our deeper view on the issues listed above.

Discussion

Testing goals and post-deployment testing

The TR 38.843 still captures the following two options for the testing goals. In our view, these two definitions are still missing clarity and could be even merged into a single one.

In this section, the study of requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements, if applicable, are documented. … For testing goals, Option 1 and/or Option 2 below will be selected depending on the test - Option 1: The testing goal is to verify whether a specific AI/ML model (if model identification is possible)/functionality can be conducted in a proper way. - FFS how to define the specific AI/ML model (e.g., a model captured in RAN4 spec as baseline) - FFS how to define that the model is properly conducted (e.g., by defining AI/ML dedicated performance/core requirements associated with model outputs) - Option 2: The testing goal is to verify whether the minimum performance gain of AI/ML model (if model identification is possible) /functionality/feature can be achieved for a static scenario/configuration. - FFS how to define a static scenario/configuration (e.g., by defining a related testing dataset based on channel models in TR 38.901) - FFS whether and how to define non-static specific scenarios/configurations

Additionally, at the previous meetings the definition of the “Static scenario” was further clarified [2] and consideration of the non-static scenarios is left per use-case:

From RAN4#110bis: Agreement: - Both static and non-static scenarios/configurations could be needed for AI testing - RAN4 will further discuss how to use them case by case - FFS whether to use static scenarios/configurations as baseline. - Refine the definitions of static and non-static scenarios/configurations based on two bullets below - Static: channel model and SNR settings are fixed and do not change over the test, specific channel realizations may be dynamic - Non-static: Non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other conditions. From RAN4#111: Chair: It is not expected that companies continue discussing this topic under the general agenda for AI/ML.

Finally, the definition of the testing goals is closely related to the guarantees of minimum performance when the change/update/new AI/ML models are possible in post-deployment, i.e., after conformance testing of the device has passed and the UEs are already in the field:

Issue 1-1: Post deployment testing (from RAN4#111) - Interested companies are invited to bring analysis on the feasibility of the options proposed so far. Issue 1-2: Post deployment handling (from RAN4#110, for reference) Agreement: - To ensure the AI performance after device deployment, discuss the following options further - Option 1: Conduct the conformance testing for AI model/functionality before deployment - FFS on the feasibility - Option 2: Design the test to verify the performance monitoring - Depend on the other WG progress - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable - Other options are not precluded

Clarification of the testing goals

In our understanding, the main particularity of testing goal Option 1 is the use of specific AI/ML model, because testing of “specific functionality” would be equivalent to testing of any given functionality.

Even if the test is specified for a given/specific model, we cannot expect that the minimum requirements will be defined for every model that can be identified. The requirements can be in different test configurations, but they shall be defined on the functionality level and not per AI/ML model basis. According to current RAN1 and RAN2 agreements UE-side AI/ML Models might be identified with Model ID. However, the availability of this information would still not allow RAN4 test requirements per each AI/ML model not explicitly exposed by the UE.

For both Options of the testing goals, the criterion for passing the test is achieving of minimal performance/core requirements defined for the functionality, i.e., it is not feasible to define requirements per each identifiable model.

For now, RAN1/2 do not have any conclusion about the identification of AI/ML models. Even if such a mechanism for model identification will be available, then it is still unclear how model ID will be used for conformance testing. The issue is that it is unclear if every model that can be identified needs to be tested:

- If every specific (i.e., identified) AI/ML model needs to be tested, then post-deployment testing mechanism for any new/updated identified models should be specified as well, i.e., after the devices are already in the field.

  - In this case, reference model can be used just to agree the value of a particular requirement, but testing will be performed with any of the implementation specific models available in, and selected by, the UE.

- If conformance testing is performed only for a single/specific/reference model, then

  - Can it be concluded that the whole functionality is tested based on the test only for this specific model?

  - Can the vendors use any other model in the test, e.g., different from the reference one, if it also demonstrates necessary level of performance?

To avoid the issues listed above, we propose to have the test requirements defined on functionality level.

RAN4 needs to clarify whether the requirement/test defined for a specific model (Option 1 in testing goals) is applicable for the functionality.

RAN4 to agree that the requirements shall be defined on functionality level (possibly, different requirements in different configurations/parameters) even when a specific/reference model is used to derive the requirements.

One way to understand the specific model, is to consider a reference model for this purpose. Based on the discussion of the reference encoder for two-sided models, such a model, indeed, can be used to derive the requirements:

- The reference decoder is used in RAN4 discussions at least for simulation alignment/requirement derivation and/or verification of the decoder implemented by the TE. It could be documented (in TR, WF, etc) or captured in the specifications as necessary.

However, it still does not mean that this reference model shall be used in the actual test. For example, it could be that the model implemented in the device during the test can demonstrate better performance than the reference model. Why will a vendor need to use a reference model in the test then?

Regarding FFS on the specific model: companies can select any specific models to pass the test (it can be reference model or not) but this model should be available at the device after deployment (e.g., for the fallback).

The clarifications made based on the Proposal 1-3 above will help to understand how the companies see that approach for the formulation of the requirements and executing tests for AI/ML related features in RAN4. In any case, we think that the testing goals can made more straightforward.

Testing goals and derivation of RAN4 requirements are two different aspects. Formulation of the requirements can be based on a certain model that is used to derive minimal level of performance for the whole functionality. Whereas, testing of each individual model will be much more challenging or nearly impossible for Release 19 due to lack of support from RAN1 and RAN2 mechanisms.

RAN4 to consider the following unified definition for the testing goal: The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved in different test configurations/parameters.

Post-deployment testing

Post-deployment testing discussion strongly depends on the understanding of the testing goals above. As we elaborated, if updates to ML-based functionalities can be identified then, this could trigger a need to verify that the device with such updates can still provide a minimum level of expected performance according to RAN4 requirements. On the other hand, if operations with AI/ML models are transparent to the 3GPP interfaces, information about such updates will never leave the device. However, the updates can cause the change of the performance and violation of minimal performance requirements.

The ongoing discussion of post-deployment testing already includes several options to handle the issue of the use of untested functionalities in the field:

Option 1 necessitates the introduction of the new procedure so that the UE vendors could indicate that the updated version of the AI/ML-enabled functionality has passed RAN4 test cases in the same way as the original model/functionality. Such approach seems to be feasible for the offline retraining of the model, e.g., when there is a major update, and a new version is distributed to the devices in a centralized manner. On the other hand, if in the future releases, more dynamic approach is considered when the models are fine-tuned or updated in the device itself, then the approach with post-deployment conformance testing of such updates is less feasible. In our understanding, online training is not in the scope of the Rel-19 WI, hence, Option 1 is feasible approach only for infrequent centralized updates.

Some of the main challenges with Option 1 are:

- Frequent updates to the AI/ML models: The companies can come up with very frequent model updates that needs to be tested. This will increase the test times proportionally.

- High turnaround time for the conformance tests: For the conformance tests to be done across different models before deployment, if we assume that an abridged set of conformance tests impacting the functionality alone is decided for this purpose, it could increase the turnaround time for the test significantly as the testing is done per device.

- Pre-deployment testing of the updated/new AI/ML-enabled functionality before it is allowed in the field (Post-deployment Option 1: conformance testing before deployment of the model) can cause a significant overhead, especially for frequent or online updates. Moreover, a standardized mechanism for such post-deployment testing will some support from the other RAN WGs (e.g., with RAN2), e.g., to allow and indicate the use only of tested models in the life network.

In Option 2, updated functionality is tested/validated as a part of performance monitoring mechanism, which is a more flexible and dynamic solution. However, there are concerns with this option as well:

- Performance monitoring of active model/functionality is a reactive procedure: The performance issues can be identified only after the model/functionality is used in the live network.

- Ambiguity about fallback model/functionality: It is not clear which model/functionality/feature should be used if the fallback or a switch of the model is executed due to the performance issue.

Current Option 2 based on monitoring does not completely address the issue of the use of the models/functionalities in the live network, that have not passed conformance testing but can demonstrate the performance below minimal requirements.

If the malfunction of functionality is identified during performance monitoring (e.g., due to the update in the underlaying models, processing blocks, etc.), then the fallback/switch should be allowed only to the functionality or legacy (non-AI/ML) feature that has passed conformance testing. In particular, this would mean that at least one functionality (for a AI/ML-enabled Feature) that has passed conformance testing should be always available at the device. This aspect may not be considered in the other WGs and should be emphasized from RAN4.

We note further that the updated/new functionality does not automatically indicate an updated/new ML model, because the other parts of the functionality implementation might also be updated, which impact the overall performance. For example, the input data pre-processing and/or output data post-processing algorithm might be updated without making any changes to the ML model, and both can have impact on the performance of the functionality.

RAN4 to ensure (and to clarify in Option2: performance monitoring-based post-deployment handling) that at least one functionality (for an AI/ML-enabled Feature) that has passed conformance tests shall be always present in the device for the fall-back.

Another alternative solution to the issues identified in Option 2 is to use a proactive verification/monitoring in the field, i.e., to verify updated/new functionality before it can be used enabled/activated. To achieve that, the device needs to support inference for a several models or versions of functionality available in the device. In this case one functionality should be used for inference and the other(s) should be in an inactive mode. These two functionalities might also require different configurations, at least in terms of their report to the NW.

We foresee that that it could be different ways how inference for the currently used and inactive functionality can be arranged.

For example, (1) the use of the underlying ML models can be split in time in two parts:

- Regular inference: The major percentage of total time is used/allocated to make inference of the currently active functionality (to support the respective use-case)

- Validation Inference: The remaining/smaller percentage of the total time is allocated to make inference of the updated/new functionality in the device for the purpose of testing/validation in inactive state/mode before that updated/new functionality can be activated and replace the currently active functionality.

Note that in this case the model switch time is not negligible, and it is necessary to allocate model switching slot between the regular inference slot and validation inference slot.

If (2) UE is capable of parallel inference with several underlying ML models provisioning the same functionality, the inference made by the currently used functionality will continue, whereas the inference made by the updated functionality can be used for the testing/validation of the updated functionality in inactive state/mode.

RAN4 to consider handling post-deployment verification of updated/new functionality in inactive/standby state/mode before it is allowed to be used along with the inference of the currently used functionality.

As it can be seen from the discussion above, handling of post-deployment issue is hardly possible without involvement and clarifications from the other WGs. Therefore, we see it important to share potential issues of post-deployment conformance and testing of AI/ML enabled functionalities with RAN1/2 so they could be aware about and consider those in the discussions and designs. At least the following aspects should be highlighted:

- RAN4 has a concern that modified/updated/new AI/ML-enabled functionalities can be used in the field without passing conformance testing explicitly.

- The value of conformance testing might be compromised if the AI/ML enabled functionalities (or functionality configurations) that passed RAN4 conformance testing are not actually used or available in the devices in the real networks (i.e., after deployment).

- RAN4 is discussing the issues above (with the list of potential options) but they require consideration of the other WGs.

RAN4 to send an LS to RAN2 and RAN1 WGs on a need to consider conformance testability of AI/ML enabled functionalities when designing such LCM operations as monitoring/activation/update/fallback for the devices already deployed in the field.

AI/ML logistics in 3GPP for RAN4

At the previous RAN#104 plenary meeting, a question about the way how AI/ML related datasets and (specific/reference) models could be shared within 3GPP was raised. This discussion is primarily triggered by the ongoing RAN4 study of two-sided models in AI/ML CSI compression use-case. However, the topic itself is rather generic, that is why we place the discussion below in the general paper.

The following conclusion was captured after the plenary:

"conclusion: RAN4 email reflector can be used to continue discussion on this matter, please report back to next RAN meeting".

and the following WF was captured in [4]:

- RAN4 AI/ML experts to identify requirements and capabilities for sharing data like - an initial list of data to be shared (e.g. data type, possible formats, size, etc) - needed capabilities for the tool ( e.g., versioning of the datasets/models, possibility to tag, etc) - List to be shared with MCC to discuss if any of the existing tools (e.g. FORGE) can be used or there is a need for another tool (provided/administered by MCC) - Follow-up depending on MCC feedback and RAN4 needs - Progress to be reported to RAN plenary

Additionally, some offline discussions continued over the RAN4 reflector and an information exchange call took place with MCC.

Firstly, we think that it is necessary to distinguish at this stage what is the current goal of AI/ML related data/object sharing within 3GPP. One primary goal is to provide to the companies a single place to exchange the data that is used in the RAN4 study. Another more distant goal could be specification of the reference models or corresponding datasets, i.e., whether reference models and/or datasets may become a part of the standard. The latter goal will require concrete agreements and more clarity about the procedure on how such reference models/datasets are derived. Therefore, for now, we think that it will make sense to focus only on the first goal above.

The means/tools for reference AI/ML models and/or datasets logistics could serve either ongoing RAN4 study where companies need to exchange own data or, in a longer term, to store standardized reference models/datasets, if those are needed and agreed for a specific use case.

RAN4 to focus only on the discussion of AI/ML logistics for the ongoing RAN4 study/experiment, i.e., do not consider any reference models/datasets that could be a part of the standard.

There are two different data objects that might need to be shared in between the companies:

- Trained reference ML models, specific to use cases, and

- Datasets, that for example can be used for further aggregation, training, or verification of the reference models.

Logistics for reference ML models and datasets could be different due to their difference in size.

Both data objects are of the binary format, however they can be different in size. Some evaluations of the model sizes were provided over reflector by a few companies: in the ongoing RAN4 study, the total size of CNN-based encoder and decoder is on the level of 7-8 MB, whereas more advanced transformer-based models could be of 100 MB or even more. Keeping the versioning of the models (e.g. in between the companies/meeting) might be not that critical at this point and the size seems to be reasonable to be shared over the current FTP server because some of the TDoc can be of the same size already now.

Reference AI/ML model sharing could be arranged even using exiting FTP due to their reasonable size, e.g., using Other TDoc type.

Another aspect of reference model sharing, that was also raised during the discussion with MCC, is the format of the models. Since the reference models are expected to be executed by different companies, on different SW/HW platforms, with or without specific optimization, the format of the reference models should not be proprietary or hard for conversion into any other formats. Hence, we think that the reference model format used for sharing should be limited to a list of the most popular formats or to the formats that can be easily converted in between each other.

RAN4 to limit the possible formats of the shared AI/ML models to models developed in popular frameworks (e.g., TensorFlow and PyTorch) with support for intermediary sharing platforms (e.g., ONNX).

Dataset logistics could be different from the current 3GPP practice of TDoc sharing primarily due to the its larger size. As it was evaluated during the reflector discussion, the training dataset for two-sided model training could be on the level of 1 GB per company. This evaluation could be even high if time-frequency-space (TFS) compression of CSI feedback is considered because dataset shall include considerably more time samples. As it was also emphasized by the MCC, such dataset size is a challenge from the point of bandwidth and storage. Additionally, if the datasets are always publicly available this could also create additional load due to massive downloads of the datasets. Finally, with such volumes of datasets the question of intermediate/historical dataset storage becomes actual. Therefore, it seems for us that for the datasets a new dedicated solution would be needed for the dataset sharing, and such a solution should be elaborated together with MCC.

For dataset sharing, RAN4 needs to discuss further a new dedicated solution together with MCC considering the following aspects:

  - The size of the dataset on the level of 10GB per company per meeting,

  - Access to the datasets, e.g., only based on ETSI account,

  - Retention/history/optimization of the dataset storage, e.g., it can be limited to certain time horizon (1 year) or only to certain limited versions of the datasets,

  - Format of the dataset, e.g., limited to some of the popular open formats, such as mat or parquet.

Conclusion

This paper is devoted to the discussion of general aspects of aspects of introduction RAN4 requirements on AI/ML-enabled features.

The following Observations and proposals are made:

On Testing goals:

- For both Options of the testing goals, the criterion for passing the test is achieving of minimal performance/core requirements defined for the functionality, i.e., it is not feasible to define requirements per each identifiable model.

- RAN4 needs to clarify whether the requirement/test defined for a specific model (Option 1 in testing goals) is applicable for the functionality.

RAN4 to agree that the requirements shall be defined on functionality level (possibly, different requirements in different configurations/parameters) even when a specific/reference model is used to derive the requirements.

Regarding FFS on the specific model: companies can select any specific models to pass the test (it can be reference model or not) but this model should be available at the device after deployment (e.g., for the fallback).

Testing goals and derivation of RAN4 requirements are two different aspects. Formulation of the requirements can be based on a certain model that is used to derive minimal level of performance for the whole functionality. Whereas, testing of each individual model will be much more challenging or nearly impossible for Release 19 due to lack of support from RAN1 and RAN2 mechanisms.

RAN4 to consider the following unified definition for the testing goal: The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved in different test configurations/parameters.

On Post-deployment handling:

Current Option 2 based on monitoring does not completely address the issue of the use of the models/functionalities in the live network, that have not passed conformance testing but can demonstrate the performance below minimal requirements.

RAN4 to ensure (and to clarify in Option2: performance monitoring-based post-deployment handling) that at least one functionality (for an AI/ML-enabled Feature) that has passed conformance tests shall be always present in the device for the fall-back.

RAN4 to consider handling post-deployment verification of updated/new functionality in inactive/standby state/mode before it is allowed to be used along with the inference of the currently used functionality.

RAN4 to send an LS to RAN2 and RAN1 WGs on a need to consider conformance testability of AI/ML enabled functionalities when designing such LCM operations as monitoring/activation/update/fallback for the devices already deployed in the field.

On AI/ML logistics:

The means/tools for reference AI/ML models and/or datasets logistics could serve either ongoing RAN4 study where companies need to exchange own data or, in a longer term, to store standardized reference models/datasets, if those are needed and agreed for a specific use case.

RAN4 to focus only on the discussion of AI/ML logistics for the ongoing RAN4 study/experiment, i.e., do not consider any reference models/datasets that could be a part of the standard.

Reference AI/ML model sharing could be arranged even using exiting FTP due to their reasonable size, e.g., using Other TDoc type.

RAN4 to limit the possible formats of the shared AI/ML models to models developed in popular frameworks (e.g., TensorFlow and PyTorch) with support for intermediary sharing platforms (e.g., ONNX).

For dataset sharing, RAN4 needs to discuss further a new dedicated solution together with MCC considering the following aspects:

  - The size of the dataset on the level of 10GB per company per meeting,

  - Access to the datasets, e.g., only based on ETSI account,

  - Retention/history/optimization of the dataset storage, e.g., it can be limited to certain time horizon (1 year) or only to certain limited versions of the datasets,

  - Format of the dataset, e.g., limited to some of the popular open formats, such as mat or parquet.

- References

- R4-2403712, WF on AI/ML, Qualcomm, RAN4#110, Athens, Greece, February 26th – March 1st, 2024.

- R4-2406617, WF on AI/ML, Qualcomm, RAN4#110-bis, China, Changsha, April 15th – 19th, 2024.

- R4-2401570, WF on AI/ML, Qualcomm, RAN4#111, Fukuoka, Japan, May 20th – 24th, 2024.

- RP-241615, AI/ML Logistics, Qualcomm Incorporated, RAN#104, Shanghai, China, June 17-20, 2024.