#3GPP TSG-RAN WG4 Meeting #112	R4-2412022

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	On AI/ML Based CSI Feedback

Agenda item:	8.17.4

Document for:	Discussion

Introduction

This paper is devoted to the discussion of requirements, testability, and interoperability aspects of AI/ML enabled CSI reporting use-cases.

During and after the previous RAN4#111 meeting an active discussion took place bout the ways to define and derive reference and test decoder for two-sided CSI compression models. In particular, the companies proposed several ways to derive the test decoder either by selecting a single decoder or training a decoder based on the aggregated dataset. Then at the post-meeting phase, model and scenario parameters for training of experimental encoder and decoder were agreed offline. Correspondingly, in the paper we share the results of encoder and decoder training based on the agreed parameters.

Furthermore, we elaborate on the following related topics:

- Derivation/selection of the test decoder and reference encoder following Option 3 and Option 4

- Verification of the test decoder

- Metrics that should be used in RAN4 requirements and tests

- Consideration of non-static scenarios

- Generalization and test coverage

- Potential LCM requirements

Discussion

Results for test decoder derivation study/experiment

Description of the encoder and decoder models

According to the agreements from AIML model email discussion after RAN4#111, the following CNN based encoder and decoder with detailed parameters settings as shown in Figure 1 and Figure 2 are agreed:

Figure 1. Detailed model diagrams of encoder.

Reference Encoder for CSI compression:

- Input Layer: The CNN-based encoder takes the input size of (N, 2, 13, 32) where, N is the batch size (# of samples per batch), 2 corresponds to the real and image parts of the complex value and used as the “channels” of convolutional neural networks, 13 is the number of frequency subbands, and 32 corresponds to the number of gNB antennas.

- First Convolutional Layer: The first convolutional layer is with kernel size of 3 x 3, stride 1, input channels of 2 and output channels of 32. The output size of the first convolutional layer is (N, 32, 13, 32), followed by a Batchnorm2D normalization layer and ReLU activation function, without change the size of output.

- Resnet Blocks: There are six Resnet blocks used in the reference CNN encoder, the input and output sizes of each Resnet Block are (N, 32, 13, 32), each with:

  - Convolutional layer 1 with kernel size 3 x 3, stride 1, 32 channels

  - BatchNorm 2D + ReLU

  - Convolutional layer 2 with kernel size 3 x 3, stride 1, 32 channels

  - BatchNorm 2D

  - Residual connection + ReLU

- Second Convolutional Layer: The second convolutional layer is with kernel size of 1 x 1, stride 1, input channels of 32 and output channels of 1, and the output size is (N, 1, 13, 32)

- Reshape Layer: Reshape from (N, 1, 13, 32) to (N, 13*32)

- Fully Connected Layer: fully connected layer with input size of 13*32 and output size of 32, which compresses the input to the desired latent dimension.

- Scalar Quantization: 2-bit uniform scalar quantization is applied, converting each float value of a neuron to 2 binary bits

- Output Layer: the output size is 64 binary bits

Figure 2. Detailed model diagrams of decoder.

Reference Decoder for CSI compression:

- Input Layer: The input size is 64 binary bits

- Scalar Dequantization: Converting 2 binary bits to 1 float value

- Fully Connected Layer: A fully connected layer with input size of 32 and output size of 2*13*32, which expend the input latent vector to the original CSI dimension.

- Reshape Layer: Reshape from (N, 2*13*32) to (N, 2, 13, 32)

- First Convolutional Layer: The first convolutional layer is with kernel size of 3 x 3, stride 1, input channels of 2 and output channels of 128. The output size of the first convolutional layer is (N, 128, 13, 32), followed by a Batchnorm2D normalization layer and ReLU activation function, without change the size of output.

- Resnet Blocks: There are six Resnet blocks used in the reference CNN decoder, the input and output sizes of each Resnet Block are (N, 128, 13, 32), each with:

  - Convolutional layer 1 with kernel size 3 x 3, stride 1, 128 channels

  - BatchNorm 2D + ReLU

  - Convolutional layer 2 with kernel size 3 x 3, stride 1, 128 channels

  - BatchNorm 2D

  - Residual connection + ReLU

- Second Convolutional Layer: The second convolutional layer is with kernel size of 1 x 1, stride 1, input channels of 128 and output channels of 2, and the output size is (N, 2, 13, 32)

- Output Layer: the output size is original CSI size of (N, 2, 13, 32)

Optimizations and hyperparameters

In addition to the agreed parameters, L2 regularization (also known as Ridge regularization), dropout, and early stopping are the regularization techniques utilized during the model training process. The purpose of L2 regularization is to prevent overfitting in machine learning models. It works by adding a penalty to the loss function proportional to the sum of the squared values of the model parameters (weights). In a standard machine learning, the objective is to minimize a loss function. L2 regularization modifies this loss function by adding a regularization term. The regularization term is the sum of the squared values of all the model weights, multiplied by a regularization parameter (λ). The modified loss function with L2 regularization is

Loss=Original Loss+λinwi2

where:

- Loss: Loss is the regularized loss function.

- Original Loss: Original Loss is the minus SGCS as it was agreed in the email discussion.

- 𝜆: λ (lambda) is the regularization parameter that controls the strength of the penalty on large weights.

- inwi2 is the sum of the squares of all the weights in the model.

The modified loss function encourages the model to keep the weights small, thereby reducing the model's complexity and improving its generalization to new data.

During training, dropout regularization randomly sets a fraction of the neurons in the network to zero or "drops them out," at each update. This technique helps prevent the model from becoming overly reliant on a few specific neurons and encourages it to learn more robust features.

Early stopping involves monitoring the model's performance on a validation set during training and stopping the training process once the performance on the validation set starts to deteriorate. This helps to ensure that the model does not continue to train to the point where it overfits the training data and performs poorly on unseen data. Validation patience is a hyperparameter used in early stopping. It specifically defines the number of epochs to continue training after the last observed improvement in the validation performance.

In addition to the optimization techniques mentioned above, we also experimented with adding a tanh activation function before the quantization layer (after the fully connected layer) on the encoder side. This function constrains the input values to the quantization layer within the range of -1 to 1. Alternatively, without the tanh function, applying maximum absolute value normalization within the quantization layer can also serve as the same purpose. We observed that adding the tanh function could help the model training coverage faster and achieve slightly better SGCS results (increase from 0.644 to 0.660).

tanh activation function before the quantization layer was used for normalization and the following optimised hyperparameters were used during the training process as shown in Table 1 below.

Table 1:Optimized hyperparameters used during training.

| Hyperparameter | Value |
| --- | --- |
| Optimizer | RMSProp (Root Mean Square Propagation) |
| No of Epochs | 200 |
| Batch Size | 1024 |
| Learning rate | 10-4 |
| Dropout rate | 0.4 |
| L2 regularization coefficient (λ) | 10-2 |
| Validation patience | 10 |
| Loss function | SGCS |

Performance results

The encoder and decoder were trained following Type 1 procedure (i.e., joint training) on a 630K samples data set with 70% used for training, 15% for validation and 15% for testing. The total trainable parameters are about 1.93M, with achieved training and testing SGCS performance of 0.644 and 0.643, respectively. Parameter combinations 1 and 2 of Rel16 eTypeII codebook feedback method contain overhead of 59 and 88 bits with the corresponding SGCS of 0.691 and 0.732. The interpolated SGCS for eTypeII CSI feedback is 0.698 at 64 bits payload (linear interpolation with parameter combination 1 and 2 cases).

RAN4 to consider the obtained SGCS metrics for the trained AI/ML encoder-decoder pair and eTypeII CSI feedback as follows:

  - Rel16 eTypeII baseline SGCS of Rank 1 eigenvectors at 2 GHz:

Table 2: CGSC performance of eTypeII CSI feedback

| Parameter Combination | Overhead Bits | Mean SGCS |
| --- | --- | --- |
| 1 | 59 | 0.691 |
| 2 | 88 | 0.732 |
| Interpolation | 64 | 0.698 |

  - Trained AI/ML encoder-decoder pair SGCS performance:

Table 3: Trained AI/ML encoder-decoder pair SGCS performance

| Model | Overhead bits | Training SGCS | Testing SGCS | Parameters |
| --- | --- | --- | --- | --- |
| CNN (NOKIA) | 64 | 0.644 | 0.643 | 1.93M |

Derivation of test decoder and reference encoder for Option 3

Test decoder derivation

At the previous RAN4#111 meeting the discussion of test decoder derivation procedure continued, mainly focusing on the Option 3 (i.e., when the test decoder is expected to be fully specified in RAN4 including the architecture/backbone/parameters and the values of the weights). The following two approaches to derive such a test decoder were summarized in the Way Forward [1]:

- Test decoder derivation procedure: - Option 1 - companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria) - Option 2 - companies bring training data for encoder + decoder pair, interested companies train an encoder + decoder pair based on the aggregated dataset from all companies to check decoder derivation feasibility and performance/complexity (or other evaluation criteria) - RAN4 agrees to work on option 1. - Interested companies are invited to bring further feasibility analysis for Option 2

One of the main questions that was not answered in Option 1 above is How RAN4 chooses one of the decoders? Performance and complexity were listed as potential evaluation criteria. However, since the architecture/parameters of the encoder + decoder for the trial were defined during the email discussion, the complexity of all the models submitted by the companies will be the same. Thus, complexity cannot be used as criteria for now.

RAN4 have not agreed on concrete criteria for the selection of the test decoder out of the whole set shared by the companies. Since the decoder + encoder architecture was fixed during the email discussion, complexity cannot be used as a selection criterion.

The performance of the models will be compared based on the results reported by the companies. SGCS (Squared Generalized Cosine Similarity) of AI/ML-based and eTypeII CSI feedback, both relative to ideal CSI, was selected as metric.

Since the model, channel, and scenario parameters are well aligned, no large difference in the performance should be expected, also considering that CSI estimation is ideal. However, some variations in the performance could be still observed, for example, due to different hyperparameters selection/tuning. In this case, the selection of a single test decoder could not be straightforward. For example, the encoder demonstrating the best performance could also be an outlier. In this case, closeness to the average/mean performance value across all the companies could be a more reasonable selection criterion. Additionally, the relative gain in comparison to Type II CSI feedback could be an additional selection condition. For example, if AI/ML CSI feedback performance by one of the companies is higher than for others, but the corresponding eTypeII performance is also higher, then it demonstrates the systematic difference in the simulator implementation rather than better performance of AI/ML model. Hence, additional conditions are needed to pick up a single decoder with the most representative performance across the companies.

If Option 1 for test decoder derivation procedure is followed, additional criteria for the selection of a single decoder out of contributed are needed.

If fully specified test decoder (Option 3) needs to be selected from the models contributed by the companies (Option 1), then RAN4 to consider not only the best performance (in terms of SGCS) but also mean performance across all the companies and the performance of corresponding eType II CSI feedback as criteria.

The problem of training the test decoder by each of the companies individually is that it is harder to justify and explain the potential differences in performance. Datasets used by the companies for training may not be disclosed. Moreover, quite probably, companies will use Type 1 training (i.e., joint training as a single entity) that could provide too optimistic performance results in comparison to Type 3 training (i.e., separate or sequential training) that could be more practical.

Dataset aggregation can provide additional means of comparison and validation for the test decoder. The aggregated dataset can be used for objective comparison of encoder/decoder pair that was trained by different companies.

Such verification dataset can contain only eigenvectors {V} that are coming to the input of the encoder. The same V from the dataset can be also used for the calculation of the SGCS after decoding.

Aggregated dataset {V} of channel eigenvectors (or channel realizations) can be used for more objective comparison/validation of encoders/decoders contributed by the companies.

Note that this approach does not necessitate training of encoder/decoder based on the dataset as it is assumed by Option 2.

RAN4 to use an aggregated dataset of channel realizations/eigenvectors {V} for comparison of the performance of encoders/decoders provided by the companies if it is necessary to select a single test decoder (can be considered as Option 1a).

Option 2 does not contain an explicit description of the aggregated dataset. As we understand, the aggregated dataset would contain not only channel/eigenvector realizations V but also the latent messages c, i.e., {V, c}. Such dataset can be then used not only for verification/selection of the test decoder (as we describe above in Option 1a), but specifically as training dataset for the test decoder. This is the main difference in between Option 1a and Option 2.

In the general case, when the architecture of the encoder and decoder are not fixed such aggregated dataset could cause contradictory results because potentially the same feedback V could be encoded into very different messages c and c’ by different encoders. Then, it could be hard to train a decoder that will be able to map c and c’ back to V that are close to the original V. However, in the ongoing RAN4 experiment, the architectures of the encoder and decoder are agreed. Hence, we can expect that the encodings of the same eigenvector provided by different companies are close enough, and an aggregated dataset {V, c} can be used for training of a single test decoder.

To proceed with Option 2 an aggregated dataset of channel eigenvectors/realizations V and encodings c, i.e., {V, c}, should be created and then used for training, performance comparison, and selection of a single test decoder.

Each of the companies can train their own test decoder realization, even if the aggregated dataset is created. Following fully specified decoder (Option 3), it will be still necessary to select only a single test decoder. Hence, some selection criteria still need to be agreed. Since training and validation datasets are the part of the same aggregated dataset, the variations in the decoder performance can be minimized, and the decoder with the best or closest to average performance can be selected.

Selection of a single test decoder trained on aggregated dataset (Option 2) can be based on performance criteria, e.g., based on the best performance or closest to mean across companies.

Reference encoder derivation

The discussions at the previous meeting focused mostly on the test decoder derivation. However, some clarity regarding the understanding of the reference encoder was also achieved at RAN4#111 [1]:

Reference encoder: - The reference encoder is used in RAN4 discussions at least for simulation alignment/requirement derivation, test decoder derivation and/or test decoder verification. It could be documented (in TR, WF, etc) or captured in the specifications as necessary.

However, it is not agreed yet how reference encoder should be derived. We see at least two approaches for that:

- One way is to select a single reference encoder together with the test decoder, i.e., as a single pair, in the procedure that we already discussed in the previous section.

  - The issue with this approach is that the companies are using Type 1 joint training for the decoder and encoder pair. Such training could provide too optimistic performance results in comparison to the other training options (i.e., Type 3 consecutive training).

- Another approach could be to derive the test decoder first, and then to train a reference decoder. We see it as a valid approach.

  - Since reference encoder based on the test decoder can be trained by each of the companies, a reference encoder selection criterion will need to be agreed, e.g., based on the performance.

    - Similarly, to the test decoder, reference encoder can be either selected from the encoders trained by the companies or ca be trained using an aggregated dataset.

  - The only downside of this approach is that it may contradicts the agreed definition that the reference encoder can be used for the test decoder derivation.

Joint training of reference encoder with test decoder could provide too optimistic performance evaluation. It is not clear how reference encoder can be used for the derivation of the test decoder.

If a single reference decoder needs to be derived, a corresponding procedure needs to be agreed like for the test decoder, e.g.,

  - Option 1: RAN4 chooses one of the encoders trained by the companies based on the selected test encoder.

  - Option 2: A single reference decoder is trained based on the aggregated dataset.

Partially specified decoder with Option 4

In our previous contribution [3], we proposed to focus on the following sub-options for Option 4-based test decoders:

- Option 2 (Dataset based):

  - Option 2a: Freeze complete training data while leaving model architecture for implementation.

  - Option 2b: Freeze the important characteristics of training data, e.g., number of bits of latent message while leaving actual data samples and model architecture for implementation.

- Option 3: Freeze the important characteristics of training data, e.g., number of bits of latent message, and a backbone of model architecture while leaving actual data samples and architectural details for implementation.

where Option 2a was preferred to best address interoperability. In addition, proposals from other companies were summarized under Issue 4-6 in the ad-hoc minutes [2]. In addition, the way forward [1] indicated a possible need for a reference encoder and/or decoder for this option. Here we continue to address the issue of interoperability but also consider test repeatability, where repeatability means that an encoder should consistently pass or fail the test of a requirement with any test decoder.

For interoperability, we have already argued that specifying a dataset (as in our proposed Option 2a above) best addresses interoperability. In order to specify the feedback interface, the source of this dataset would be a reference encoder/decoder pair and its contents would include the eigenvectors and latent messages {V, c}. Option 4 test decoders can be trained with this dataset or, equivalently, with the eigenvectors in conjunction with the reference encoder.

A reference encoder/decoder pair can be used as the source of the dataset {V, c} that specifies the feedback interface and ensures test decoder interoperability for Option 4.

For repeatability, more is required than simply a common dataset since test decoder performance comes into play. Repeatability can be ensured by a test decoder validation procedure which includes decoder performance specifications using the specified dataset. As is common in training procedures, the dataset can be divided into training, testing, and validation parts and performance specifications for the test decoder can be specified using the testing part in conjunction with the reference encoder.  Since the specifications for an Option 4 test decoder have been relaxed from Option 3, these performance specifications are designed to provide a high degree of repeatability between test decoders developed by different test equipment vendors.

Test repeatability in Option 4 can be addressed by developing a test decoder validation procedure including decoder performance specifications using the specified dataset.

Select a reference encoder/decoder pair to create a dataset for training Option 4 test decoders.

Verification of test decoder

In the previous meeting (RAN4#111) the definition of a reference encoder and a reference decoder was generally agreed. The agreements are captured as below.

Issue 4-1: Reference encoder/decoder Agreement: Reference encoder: - The reference encoder is used in RAN4 discussions at least for simulation alignment/requirement derivation, test decoder derivation and/or test decoder verification. It could be documented (in TR, WF, etc) or captured in the specifications as necessary. Reference decoder: - The reference decoder is used in RAN4 discussions at least for simulation alignment/requirement derivation and/or verification of the decoder implemented by the TE. It could be documented (in TR, WF, etc) or captured in the specifications as necessary. For option 3, for each test, a test decoder needs to be captured in the specs, a reference encoder might be needed to derive the test decoder and/or requirements and/or to validate the test decoder implementation in the TE. the same decoder might be used in multiple tests or each test could have a difference decoder. In option 3 the Reference decoder is the test decoder. For option 4, there might be a need to have reference encoder and/or reference decoder

However, based on the option (Option 3 / Option 4) that will be the way forward for test decoder derivation, it will become necessary to verify the test decoder. This is to ensure that the test decoder is indeed interoperable and satisfies the minimum performance criteria that is specified for it for an encoder.

The following options to verify the test decoder can be considered:

- UE vendor implements the reference encoder and test decoder verification is performed with an actual UE, i.e., UE vendor’s implemented reference encoder can be used in the test setup for test decoder’s verification.

  - During the test, the UE should possess reference encoder potential together with any other decoder the UE vendor wants to use in the test.

  - The testing procedure will include, firstly, verification of the decoder with the reference encoder, and then the actual test with an encoder implemented by the UE vendor for the real network.

- Test equipment vendors can implement the reference encoder at their end, and may test only the AIML functionality, without necessarily having complete UE side operations.

  - It could be a kind of SW verification of the test decoder based on the reference encoder without using any real/test UE.

- The stakeholders will mutually agree on the use of a UE simulator/emulator which can be a 3rd party device or framework just for testing the encoder with the test decoder which is not necessarily a full-fledged UE supporting all the functionalities of a UE.

For all above listed options, the minimum performance target for test decoder’s verification should be agreed during the specification phase considering various results across different vendors.

For test decoder’s verification fully specified reference encoder should be used.

For test decoder’s verification, reference encoder may be implemented by UE vendors, TE vendors or a third-party implementation of UE sided operations including reference encoder.

For test decoder’s verification, the minimum performance target should be agreed and specified considering various results from different stakeholders.

Metric for AI/ML-enabled CSI feedback requirements

The existing PMI reporting test cases, even when PMI is reported for the Enhanced TypeII codebook, for reference the System Simulator (SS) shall transmit PDSCH with randomly selected precoding matrix from Type I Single-Panel Codebook (e.g. defined in the Table 5.2.2.2.1-6 in TS 38.214). The main reason is that it is easier to use Type I codebook to select the precoding matrices with random probabilities from the limited number of options. Whereas randomisation of Type II codebook is more complicated because it includes more indices and a linear combination of vectors. Hence, it is harder to guarantee that the resulting random precoding matrix makes sense. The same problem is expected with AI/ML-based compressed CSI feedback because randomization of the feedback (e.g. of eigenvectors) while keeping it realistic is not that straightforward like with Type I feedback.

The existing PMI/CSI reporting requirement is based on the relative throughput γ as the test metric. Type I single-panel codebook is easier to randomize to establish the reference PDSCH throughput (in the denominator of relative throughput γ) in comparison with Type II codebook or AI/ML-based compressed feedback.

Hence, it seems to us reasonable to use the same approach as in the existing requirements, i.e., to use randomized Type I single-panel codebook in the definition of relative throughput:

Random precoding matrices from Type I single-panel codebook (mode 1 with appropriate number of layers) should be used a reference in the requirement on the relative throughput γ.

Regarding the definition of the numerator in the relative throughput γ, the approach will depend on the type of AI/ML-based CSI feedback:

- In the case of AI/ML-based feedback compression, the most typical feedback format is expected to be the eigenvectors of the channel matrix. Even though the actual precoding used by the gNB is up to implementation, for performance evaluation purpose, the eigenvectors reported by the UE (after decoding) can be used as the precoding matrix directly. Correspondingly, UE throughput

- In the case of AI/ML-based CSI prediction, it is expected that the format of UE reporting stays without changed, e.g., Type II CSI feedback format will be still used even in the case when the CSI is predicted for a certain horizon in the future. In this case, the CSI predicted by the UE can be used directly to precode PDSCH like in legacy CSI feedback performance requirements.

For AI/ML-based compressed CSI feedback, decoded eigenvectors of the channel matrix reported by the UE can be used for PDSCH precoding at SS.

We are expecting that in CSI prediction use-case legacy CSI reporting format will be used.

For AI/ML-based CSI prediction, CSI feedback predicted and reported by the UE in legacy format (e.g., eType II) can be used by SS for PDSCH precoding, i.e., like in legacy requirements.

If we can assume that the same/comparable metric (i.e., relative to the PDSCH throughput with random precoding matrices from Type I single-panel codebook) is used both for AI/ML-based and legacy requirements, it becomes possible not only to derive new requirements but also to compare those to the legacy ones. That, of course, implies that the parameters of the tests are kept the same. It does not mean that all the tests should replicate existing ones, but introduction of comparable tests with the same metrics should be used to demonstrate the gains on AI/ML implementations, especially when the level of overhear is the same or higher than with the legacy solutions.

The use of the same/comparable metric (relative throughput γ) for AI/ML based and existing legacy requirements with the same parameters/tests gives the way to compare the minimal level of performance.

RAN4 to ensure that relative throughput based on AI/ML CSI feedback with the same overhead is at least not worse than in legacy tests with the same parameters.

Non-static scenarios

At the previous meeting it was decided by the RAN4 chair that further discussion of non-static scenarios in tests should continue in a use-case specific manner:

Static and non-static conditions Chair: It is not expected that companies continue discussing this topic under the general agenda for AI/ML.

The definition of the static scenarios was clarified already at RAN4#110bis, whereas the understanding of the non-static conditions requires further clarification [4]:

Agreement: - Both static and non-static scenarios/configurations could be needed for AI testing - RAN4 will further discuss how to use them case by case - FFS whether to use static scenarios/configurations as baseline. - Refine the definitions of static and non-static scenarios/configurations based on two bullets below - Static: channel model and SNR settings are fixed and do not change over the test, specific channel realizations may be dynamic - Non-static: Non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other conditions.

In our view, non-static scenarios are aimed at testing the case when UE keeps using the same functionality but might still switch models internally without any explicit signaling to the NW. Such transitions should not impact the performance of the functionality significantly and this what needs to be justified by the test.

In case of CSI feedback use case, changing radio conditions (e.g., channel model from indoor to outdoor, or Doppler related to UE speed, or delay spread, etc.) may trigger UE internal switch from one AI/ML model to another. Such switching may interrupt inference for a small duration of time that should not impact the performance (such as (relative) PDSCH throughput) above the acceptable tolerance margin.

Non-static scenarios in RAN4 are needed to verify that there is no performance degradation while UE is autonomously switching in between different AI/ML models due to the change of radio conditions.

Non-static scenarios/conditions can include the change of radio conditions during the test, such as channel model (e.g., from indoor to outdoor) and/or its’s parameters (e.g., Doppler, delay spread).

RAN4 should define performance requirements for non-static scenarios/conditions provided that the functionality is not changing/switching during the test and internal model’s transitions are up to the UE.

Generalization and test coverage

In the RAN4#110 meeting, the agreements for Generalization are noted as below.

Issue 1-1: Generalization update Agreement: - For AI/ML generalization [tests/requirements] - RAN4 should discuss it and decide the requirements/tests for each AI feature in the case-by-case manner

In RAN4#111, the concern regarding overfitting of UE models to the test environment, was raised by some companies and it was captured in the agreement and companies were encouraged to bring proposals to avoid overfitting issue.  In our view, testing generalization aspects of UE models/functionalities provide a rather realistic and feasible solution to the overfitting issue. Since, testing a UE model/functionality in different scenarios and configuration would ensure that the model/functionality is/are not overfitted to a certain test setup/environment. A UE model/functionality, if tested in different generalization scenarios, would not be overfitted to a certain test environment. However, we think that this is an important topic to be discussed. And in this section, we discuss about the different aspects of testing the generalization and scalability of the AIML enabled functionality.

Tests and Parameters that should be considered for the definition of different generalization/scalability scenarios:

- Generalization Scenarios

Generalization parameters like the overall scenario (LOS, NLOS, indoor, outdoor, etc.), SINR, UE speed, etc. are generally not known at the UE nor the gNB. And this must be configured at the TE.

Table 4: Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| UE Speed | Slow / Medium / Fast |
| SINR | Good / Bad Radio conditions |
| Outdoor / Indoor | Position of the UE |
| LOS/NLOS |  |
| Propagation Model | UMa / Umi |

- Scalability Scenarios

Scalable parameters such as the number of antenna ports, bandwidth, or carrier frequency are typically known at the UE and the gNB side, e.g., by exchange of configuration messages. Therefore, one possible option is to select or configure the UE sided ML models to the given scalability parameters. In such a case, one would have to specify a set of ML models covering the predefined scalability parameter values. This option generates some extra overhead, for example, with respect to the memory size for the multiple ML models, but otherwise might ensure best possible performance per configuration.

Scalability parameters are generally known at the UE and the gNB and, typically, do not change during the active time of a UE in a certain cell.

Table 5: Parameters for Scalability Scenarios

| Parameters | Description |
| --- | --- |
| Number of Antenna ports | (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports) |
| Carrier Frequency | FDD, TDD at sub-band level |
| Bandwidth | E.g., 10MHz, 20MHz |

For the verification/testing of generalization/scalability related aspects in RAN4 for AI/ML enabled CSI feedback enhancement, RAN4 should define different scenarios based on parameters listed in the tables above.

After the identification of different scenarios, RAN4 should further study the following aspects.

- Minimum performance for identified scenarios.

Minimum performance for identified scenarios should be comparable to the minimum RAN4 performance requirements. The functionality should be able to achieve this as baseline performance.

- Tolerance margin for other scenarios.

Tolerance margin is the margin by which the performance of the functionality is degraded with respect to the performance of the identified scenarios. The tolerance margin can be defined based on the simulation results and can be adapted for in field performance of the functionality.

AI/ML models and functionalities have their limits on the conditions where they can be used. These limitations can be imposed by the training procedure or dataset, based on the definition of the UE capabilities, and importantly based on the generalization capabilities.

The generalization capability of a functionality can impact the performance of the functionality, for example if the functionality is not generalized enough across a set of scenarios, this will result in frequent switch of the functionality resulting in the degradation of the performance.

Less generalized functionalities across a set of scenarios can result in frequent switching of model/functionality resulting in performance degradation.

To mitigate the problems caused by less generalized functionalities, it is important to ensure that the functionality is generalized enough. Ensuring the generalization aspects of functionality across different scenarios and conditions is not straightforward. At the outset we need the scenarios and conditions need to be identified and aligned. KPIs for generalization need to be defined. And then the tests need to be carried out on all the scenarios and based on the validation result it can be decided if the functionality is generalized enough. To carry out all these identified steps, it becomes imperative to have a test framework to test the generalization aspects of a functionality.

In order to address this issue, we propose a new metric to indicate the generalization capabilities of an AI/ML model/functionality, that may be referred as “generalization score”. In case of CSI feedback enhancement use case, this may be used to verify/check whether the CSI feedback enhancement functionality/model (which may be developed for a specific scenario and/or configuration) generalize well to other scenario(s) and/or configuration(s).

As an example, if it is identified that there are 3 scenarios and 3 configurations of interest, then there will be 9 different pairs (Scenario 1, Configuration 1), (Scenario 1, Configuration 2), (Scenario 1, Configuration 3), (Scenario 2, Configuration 1), …, (Scenario 3, Configuration 3). Any one pair out of these 9 pairs can be the baseline setup (depending on functionality/model implementation) and the remaining 8 pairs will then be used as other set ups to test the generalization aspects. An overall generalization score can be calculated as a function of individual pair verdicts, e.g., as simple average or weighted average of the individual generalization verdict of each Pair ‘j’ (i.e., Pair ‘1’ to Pair ‘N), where the verdict of Pair ‘j’ = 1 if the functionality/model generalize to Pair ‘j’ else it is equal to 0.

Overall Generalization Score= (VerdictPair1+…+VerdictPairN)N×100%

RAN4 needs to design a new metric, indicative of generalization capabilities of AI/ML model/functionality, to verify the generalization performance of the model/functionality in different scenarios.

LCM requirements

One of the factors that influence the performance of the AIML enabled functionality is the latency of the LCM actions.

If performance monitoring detects a performance degradation to a point where a decision to either switch this model/functionality with another model/functionality is taken or a fallback decision is taken, it means that the AI/ML functionality is degrading the system performance and if this functionality, with detected performance degradation, keeps running then the impact on system performance may result in catastrophic consequences.

Therefore, it is crucial to stop this model/functionality, either by falling back to legacy method or by switching to another model/functionality, within a specified time. For a use case like AIML enabled CSI feedback, it would be very urgent to stop/switch functionality because a wrong channel information would lead to wrong link adaptation and scheduling decisions impacting the throughput.

For UE-assisted or NW-based performance monitoring, if required LCM action is not taken in a timely manner, the performance of AI/ML-based CSI feedback may be degraded to undesirable levels.

Use-case specific core requirements should be considered to limit latency of LCM actions (e.g. activation, deactivation, fallback, switching etc.) for AI/ML-enabled CSI feedback.

Conclusion

In this paper we discuss a range of open issues related to AI/ML-enabled CSI feedback requirements, testability and operability. Primarily we focus on CSI compression use case, but some aspects of CSI predictions are also discussed.

The following Observations and Proposals were made that are arranged per topic:

On test decoder derivation study/experiment:

- tanh activation function before the quantization layer was used for normalization and the following optimised hyperparameters were used during the training process as shown in Table 1 below.

Table 1:Optimized hyperparameters used during training.

| Hyperparameter | Value |
| --- | --- |
| Optimizer | RMSProp (Root Mean Square Propagation) |
| No of Epochs | 200 |
| Batch Size | 1024 |
| Learning rate | 10-4 |
| Dropout rate | 0.4 |
| L2 regularization coefficient (λ) | 10-2 |
| Validation patience | 10 |
| Loss function | SGCS |

- RAN4 to consider the obtained SGCS metrics for the trained AI/ML encoder-decoder pair and eTypeII CSI feedback as follows:

  - Rel16 eTypeII baseline SGCS of Rank 1 eigenvectors at 2 GHz:

Table 2: CGSC performance of eTypeII CSI feedback

| Parameter Combination | Overhead Bits | Mean SGCS |
| --- | --- | --- |
| 1 | 59 | 0.691 |
| 2 | 88 | 0.732 |
| Interpolation | 64 | 0.698 |

  - Trained AI/ML encoder-decoder pair SGCS performance:

Table 3: Trained AI/ML encoder-decoder pair SGCS performance

| Model | Overhead bits | Training SGCS | Testing SGCS | Parameters |
| --- | --- | --- | --- | --- |
| CNN (NOKIA) | 64 | 0.644 | 0.643 | 1.93M |

On derivation of test decoder and reference encoder for Option 4:

RAN4 have not agreed on concrete criteria for the selection of the test decoder out of the whole set shared by the companies. Since the decoder + encoder architecture was fixed during the email discussion, complexity cannot be used as a selection criterion.

If Option 1 for test decoder derivation procedure is followed, additional criteria for the selection of a single decoder out of contributed are needed.

If fully specified test decoder (Option 3) needs to be selected from the models contributed by the companies (Option 1), then RAN4 to consider not only the best performance (in terms of SGCS) but also mean performance across all the companies and the performance of corresponding eType II CSI feedback as criteria.

Aggregated dataset {V} of channel eigenvectors (or channel realizations) can be used for more objective comparison/validation of encoders/decoders contributed by the companies.

RAN4 to use an aggregated dataset of channel realizations/eigenvectors {V} for comparison of the performance of encoders/decoders provided by the companies if it is necessary to select a single test decoder (can be considered as Option 1a).

To proceed with Option 2 an aggregated dataset of channel eigenvectors/realizations V and encodings c, i.e., {V, c}, should be created and then used for training, performance comparison, and selection of a single test decoder.

Selection of a single test decoder trained on aggregated dataset (Option 2) can be based on performance criteria, e.g., based on the best performance or closest to mean across companies.

Joint training of reference encoder with test decoder could provide too optimistic performance evaluation. It is not clear how reference encoder can be used for the derivation of the test decoder.

If a single reference decoder needs to be derived, a corresponding procedure needs to be agreed like for the test decoder, e.g.,

  - Option 1: RAN4 chooses one of the encoders trained by the companies based on the selected test encoder.

  - Option 2: A single reference decoder is trained based on the aggregated dataset.

On partially specified decoder with Option 4:

A reference encoder/decoder pair can be used as the source of the dataset {V, c} that specifies the feedback interface and ensures test decoder interoperability for Option 4.

Test repeatability in Option 4 can be addressed by developing a test decoder validation procedure including decoder performance specifications using the specified dataset.

Select a reference encoder/decoder pair to create a dataset for training Option 4 test decoders.

On test decoder verification:

For test decoder’s verification fully specified reference encoder should be used.

For test decoder’s verification, reference encoder may be implemented by UE vendors, TE vendors or a third-party implementation of UE sided operations including reference encoder.

For test decoder’s verification, the minimum performance target should be agreed and specified considering various results from different stakeholders.

On metric for AI/ML-enabled CSI feedback requirements:

The existing PMI/CSI reporting requirement is based on the relative throughput γ as the test metric. Type I single-panel codebook is easier to randomize to establish the reference PDSCH throughput (in the denominator of relative throughput γ) in comparison with Type II codebook or AI/ML-based compressed feedback.

Random precoding matrices from Type I single-panel codebook (mode 1 with appropriate number of layers) should be used a reference in the requirement on the relative throughput γ.

For AI/ML-based compressed CSI feedback, decoded eigenvectors of the channel matrix reported by the UE can be used for PDSCH precoding at SS.

We are expecting that in CSI prediction use-case legacy CSI reporting format will be used.

For AI/ML-based CSI prediction, CSI feedback predicted and reported by the UE in legacy format (e.g., eType II) can be used by SS for PDSCH precoding, i.e., like in legacy requirements.

The use of the same/comparable metric (relative throughput γ) for AI/ML based and existing legacy requirements with the same parameters/tests gives the way to compare the minimal level of performance.

RAN4 to ensure that relative throughput based on AI/ML CSI feedback with the same overhead is at least not worse than in legacy tests with the same parameters.

On non-static scenarios:

Non-static scenarios in RAN4 are needed to verify that there is no performance degradation while UE is autonomously switching in between different AI/ML models due to the change of radio conditions.

Non-static scenarios/conditions can include the change of radio conditions during the test, such as channel model (e.g., from indoor to outdoor) and/or its’s parameters (e.g., Doppler, delay spread).

RAN4 should define performance requirements for non-static scenarios/conditions provided that the functionality is not changing/switching during the test and internal model’s transitions are up to the UE.

On generalization and test coverage:

Generalization parameters like the overall scenario (LOS, NLOS, indoor, outdoor, etc.), SINR, UE speed, etc. are generally not known at the UE nor the gNB. And this must be configured at the TE.

Table 4: Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| UE Speed | Slow / Medium / Fast |
| SINR | Good / Bad Radio conditions |
| Outdoor / Indoor | Position of the UE |
| LOS/NLOS |  |
| Propagation Model | UMa / Umi |

Scalability parameters are generally known at the UE and the gNB and, typically, do not change during the active time of a UE in a certain cell.

Table 5: Parameters for Scalability Scenarios

| Parameters | Description |
| --- | --- |
| Number of Antenna ports | (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports) |
| Carrier Frequency | FDD, TDD at sub-band level |
| Bandwidth | E.g., 10MHz, 20MHz |

For the verification/testing of generalization/scalability related aspects in RAN4 for AI/ML enabled CSI feedback enhancement, RAN4 should define different scenarios based on parameters listed in the tables above.

Less generalized functionalities across a set of scenarios can result in frequent switching of model/functionality resulting in performance degradation.

RAN4 needs to design a new metric, indicative of generalization capabilities of AI/ML model/functionality, to verify the generalization performance of the model/functionality in different scenarios.

On LCM requirements:

For UE-assisted or NW-based performance monitoring, if required LCM action is not taken in a timely manner, the performance of AI/ML-based CSI feedback may be degraded to undesirable levels.

Use-case specific core requirements should be considered to limit latency of LCM actions (e.g. activation, deactivation, fallback, switching etc.) for AI/ML-enabled CSI feedback.

- References

- R4-2410570, WF on AI/ML, Qualcomm, RAN4#111, Fukuoka city, Japan, May 20th – 24th, 2024.

- R4-2410571, AI/ML ad-hoc meeting minutes, Qualcomm, RAN4#111, Fukuoka city, Japan , May 20th – 24th, 2024.

- R4-2408659, On AI/ML Based CSI Feedback, Nokia, RAN4#111, Fukuoka city, Japan, May 20th – 24th, 2024.

- R4-2406617, WF on AI/ML, Qualcomm, RAN4#110-bis, China, Changsha, April 15th – 19th, 2024.