3GPP TSG-RAN4 Meeting #112	R4-2412024

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	LG Electronics

Title:	HPUE for intra-band UL CA

Agenda item:	8.1.1.1.1

Document for:	Discussion

# Introduction

In this paper, we provide our views on HPUE for intra-band contiguous/non-contiguous UL CA.

# Discussion

In last RAN4 meeting, the following WF was approved [1]. This paper discusses the requirement of HPUE for intra-band UL CA.

# 2. PC1.5 for intra-band contiguous and non-contiguous UL CA ## 2.1 Assumed UE architecture(s)/parameters/methodology for MPR/A-MPR evaluation Online agreement: Agreement: - Evaluate MPR/A-MPR numbers for contiguous CA first and then work on non-contiguous CA MPR/A-MPR afterwards. - The intra-band PC1.5 ULCA R19 work focusses on UE implementing with two 26dBm PAs. Architectures requiring a 29dBm PA are not specified but can be implemented by fulfilling the requirements based on two 26dBm PAs. - For intra-band contiguous ULCA w/ and w/o UL MIMO, the Architecture#2 is prioritized for MPR/A-MPR evaluation. - If time is allowed, architecture #1 can be evaluated. Architecture; Description; Indicated capability; Whether can support UL MIMO; Applicable cases. #1; 2x26 dBm PA + 2 LO with 100MHz BW; dualPA-Architecture; No; Contiguous CA. #2; 2x26 dBm PA + 1 LO with 200MHz BW; TxD; Yes; Contiguous CA. - For intra-band NC ULCA w/o UL MIMO, the Architecture#1 is prioritized for MPR/A-MPR evaluation. - FFS on Architecture #2 Architecture; Description; Indicated capability; Applicable cases. #1; 2x26 dBm PA + 2 LO with 100MHz BW; dualPA-Architecture; NC CA w/o UL MIMO. #2; 2x26 dBm PA + 1 LO with 200MHz BW; TxD; NC CA w/o UL MIMO. ## 2.2 Whether to define separate MRP/A-MPR requirements for handheld UE and FWA respectively? Way forward: Yes, and assuming 20dB minimum antenna isolation for FWA and 10dB for handheld UE ## 2.3 A-MPR for n41/n77/n78/n79 Way forward: - A-MPR requirements are not needed for PC1.5 n77/n78/n79 intra-band ULCA - Evaluate A-MPR requirements for PC1.5 n41 intra-band UL CA ## PCMAX,C Proposals: - Proposal 1: Define PCMAX,c limitation for each component carrier considering PC1.5 UE architecture of intra-band UL CA. (LGE) - For dualPA-architecture - PCMAX,C limitation for each component carrier is 26dBm - For TxD (dualTx) - PCMAX,CC1 = 26+10*log10( LCRB1*SCS1LCRB1*SCS1+LCRB2*SCS2)+3 - PCMAX,CC2 = 26+10*log10( LCRB2*SCS2LCRB1*SCS1+LCRB2*SCS2)+3 - Proposal 2: Changes to TS 38.101-1 for dual-PA architecture, inset new equation for PCMAX,c (Ericsson, more details refer to R4-2407721) Way forward: FFS in future meetings ## 2.5 PCMAX Proposals: - Option 1: (Skyworks) - MOP for 2Tx architectures (TxD w/wo UL MIMO) is: 29 dBm - MOP for Dual-PA architectures (one PA/CC, 2LO) is: PCmax= 29 + 10*log(1/2*(1+Min(LCRB1*SCS1,LCRB2*SCS2)/Max(LCRB1*SCS1,LCRB2*SCS2))) dBm - Option 2: (Skyworks) MPR for Dual-PA architectures (one PA/CC, 2LO) is modified by: 10*log(1/2*(1+Min(LCRB1*SCS1,LCRB2*SCS2)/Max(LCRB1*SCS1,LCRB2*SCS2))) dB - Option 3: (Captured in last meeting’s WF) For R19 PC1.5 intra-band non-contiguous /contiguous ULCA with two 26dBm PAs and one PA per CC, the PCMAX is modified as follows to account for RB BW imbalances PCmax=10*log(10^(26/10) + 10^((26-10*log(LCRB1*SCS1/(LCRB2*SCS2)))/10)) - Option 4: For UEs indicating dualPA-Architecture (Architecture #1) the total configured output power PCMAX for the band combination is given by clause 6.2A.4.1.3 for inter-band CA in the current version of 38.101-1, the same applies for TxD (Ericsson) Way forward: FFS in future meetings ## 2.6 PCMAX tolerance Agreement: The PCMAX tolerance for uplink intra-band contiguous/non-contiguous CA are: PCMAX (dBm); Tolerance TLOW(PCMAX) (dB); Tolerance THIGH(PCMAX) (dB). 23 < PCMAX ≤ 29; 3; 2. 21 ≤ PCMAX ≤ 23; 2.0. 20 ≤ PCMAX < 21; 2.5. 19 ≤ PCMAX < 20; 3.5. 18 ≤ PCMAX < 19; 4.0. 13 ≤ PCMAX < 18; 5.0. 8 ≤ PCMAX < 13; 6.0. -40 ≤ PCMAX < 8; 7.0.

## PC1.5 UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx

Based on the agreement in WF [1], MPR/A-MPR for intra-band contiguous CA first and then MPR/A-MPR for intra-band non-contiguous CA is going to be evaluated. We have plan to provide simulation result of MPR taking both minimum antenna isolations of 20dB and 10dB into account in next meeting.

Figure 2-1 shows PC1.5 dualPA-architecture for intra-band non-contiguous UL CA.

Figure 2-1. PC1.5 dualPA-Architecture(2PA2LO)

Here, LCRB1 and LCRB2 are assumed to be allocated to each CC1 and CC2.

For example, if the following scenario is assumed,

- CBW1 = 20MHz, CBW2 = 20MHz

- SCS1 = 15MHz, SCS2 = 15kHz

- LCRB1 = 1RB, LCRB2 = 106RB

For case that different PSD is assumed,

- PCMAX,C limitation can be 26dBm for CC1 and CC2 respectively regardless of the size of LCRB1 and LCRB2.

- Total of PCMAX limitation can be 29dBm

- Difference of PSD is about 20.3dB

For case that same PSD is assumed,

- PCMAX,C limitation can be 5.7dBm for CC1

- PCMAX,C limitation can be 26dBm for CC2

- Total of PCMAX limitation cannot be 29dBm but be 26.04dBm

- Difference of PSD is 0dB

Figure 2-2 shows the limitation of PCMAX,CC1, PCMAX,CC2, and PCMAX  when LCRB1 = 1RB~106RB and LCRB2 = 106RB for dualPA-Architecture with same PSD.

Figure 2-2. Limitation of PCMAX,CC1/CC2 and PCMAX when LCRB1 = 1RB~106RB and LCRB2 = 106RB for dualPA-Architecture with same PSD

As seen in Figure 2-2, PCMAX cannot reach 29dBm with equal PSD unless LCRB1 is equal to LCRB2. While this is possible it may not be the common real field scenario.

In real field, the different PSD is assumed to be more likely use case. Therefore, the different PSD between two CCs should be considered for UE RF requirement, such as, MPR, PCMAX,C, and PCMAX.

#### Proposal 1: For dualPA-Architecture of PC1.5 intra-band non-contiguous UL CA, consider also the cases with different PSD between the CCs, especially when LCRB1 is different from LCRB2.

#### Proposal 2: For dualPA-Architecture of PC1.5 intra-band non-contiguous UL CA, consider following PCMAX,C limitation as configured output power for each component carrier, and PCMAX limitation as configured total output power.

- PCMAX,CC1 = 26 dBm

- PCMAX,CC2 = 26 dBm

- PCMAX = 29 dBm

Figure 2-3 shows PC1.5 TxD-architecture for intra-band contiguous UL CA.

Figure 2-3. PC1.5 TxD-Architecture (2PA1LO)

Here, LCRB1 and LCRB2 are assumed to be allocated to each CC1 and CC2.

For example, if the following scenario is assumed,

- CBW1 = 20MHz, CBW2 = 20MHz

- SCS1 = 15MHz, SCS2 = 15kHz

- LCRB1 = 1RB, LCRB2 = 106RB

Considering equal PSD between CC1 and CC2

- PCMAX,CC1 = 29 – 20.3 = 8.7 dBm for CC1

- PCMAX,CC2 = 29 – 0.04 = 28.96 dBm for CC2

- PCMAX is 29dBm

Figure 2-4 shows the limitation of PCMAX,CC1, PCMAX,CC2, and PCMAX  when LCRB1 = 1RB~106RB and LCRB2 = 106RB for TxD-Architecture with same PSD.

- PCMAX,CC1 = 26+10*log10( LCRB1*SCS1LCRB1*SCS1+LCRB2*SCS2)+3

- PCMAX,CC2 = 26+10*log10( LCRB2*SCS2LCRB1*SCS1+LCRB2*SCS2)+3

- PCMAX is 29dBm

Figure 2-4. Limitation of PCMAX,CC1/CC2 and PCMAX when LCRB1 = 1RB~106RB and LCRB2 = 106RB for TxD-Architecture with same PSD

As seen in Figure 2-4, PCMAX,CC2 is higher than 26dBm in all other LCRB1 than equal to LCRB2 when considering Equal PSD for TxD-Architecture.

However, in real field, the different PSD should also be assumed. Therefore, the different PSD between two CCs should be considered for UE RF requirement, such as, MPR, PCMAX,C, and PCMAX.

#### Proposal 3: For TxD-Architecture of PC1.5 intra-band contiguous UL CA, consider same PSD when LCRB1 is different from LCRB2.

#### Proposal 4: For TxD-Architecture of PC1.5 intra-band contiguous UL CA, consider following PCMAX,C limitation as configured output power for each component carrier, and PCMAX limitation  of 29dBm as configured total output power

- PCMAX,CC1 = ( 26+10*log10( LCRB1*SCS1LCRB1*SCS1+LCRB2*SCS2)+3 ) dBm

- PCMAX,CC2 = ( 26+10*log10( LCRB2*SCS2LCRB1*SCS1+LCRB2*SCS2)+3 ) dBm

- PCMAX = 29 dBm

# Use of generally known IM3 equations to extend the already existing TX CA MPR requirements for new PA combinations / power classes

For TX CA the transmitter performance and MPR have been defined in 38.101-1 for 20+20= 23dBm (PC3) and 23+23= 26dBm (PC2) and to ensure that both SEM and out-of-band performance requirements are met. The applicability of the existing requirements also for case with unequal PSD has been debated in RAN4.

We have provided our views to this in the proposals 1-4 above and see the need to further improve the coverage of the already existing requirements. However, when it comes to defining MPR for new CA PA combinations and power classes it may be possible to avoid time consuming simulations and analysis for each case by introducing a delta value to a selected reference. This delta value can be defined using the single carrier ACLR requirement as driving factor for PA and TX line-up dimensioning and linearity, in a bit similar manner as single carrier ACLR is used as calibration method in the MPR simulation.

## IM3 analysis

When two tones or carriers are transmitted with non-linear amplifier the level of spurious emissions, IM3 products, follow the well-known formula.

OIP3= Pout + 1/2*P, where P is the difference between the level two tone signal components and IM3 products.

OIP3=PTone+12∙∆P

Figure 3-1. Definition of OIP3 with equal tone or carrier power

There are also well known IM3 formulas for the case that power of the tones (or carriers) is not equal and there are shown below.

OIP3=2∙PLow+PHigh-IM3Low2     OIP3=2∙PHigh+PLow-IM3High2

Figure 3-2. Definition of OIP3 with unequal tone or carrier power

It is also known that P for two-tone signal is also a relatively good estimate for the ACLR with fully allocated (all RBs are transmitted) NR signal. Therefore, we can use the minimum ACLR performance requirement to estimate the OIP3 for the PA (or TX line up) that is required to support certain output power level with targeted linearity. Although the post PA losses are not relevant in case of relative comparison 2dB IL in RFFE between the PA and antenna is assumed here. IL higher than 2dB is often used in the analysis done for UEs supporting multiple modes and bands, but as the RFFE IL is not impacting the outcome of the relative analysis, lower IL= 2dB is used here. Block diagram of the simplified TX model that is used is shown below.

Figure 3-3. Simplified TX architecture

The 38.101-1 defines following ACLR requirement for different UE power classes:

ACLR and PA output power can be used in OIP3 equation to derive estimates for the required PA OIP3 as shown in table below.

|  | Power class 1 | Power class 2 | Power class 3 | Power class 5 |
| --- | --- | --- | --- | --- |
| Pout (dBm) | 31 | 26 | 23 | 20 |
| Post PA losses (dB) | 2 | 2 | 2 | 2 |
| PA Pout (dBm) | 33 | 28 | 25 | 22 |
| deltaP (dB) | 37 | 31 | 30 | 27 |
| PA OIP3 (dBm) | 48.5 | 40.5 | 37 | 32.5 |

The OIP3 can then be added back to same equations to estimate the level of IM3 products in case of two TX line-ups being used to generate high-output power non-contiguous ULCA signal. This simplified method does not take into account all transmitter imperfections that impact the level of emissions, but can be used as relative metric as shown below.

Figure 3-4. Examples with 23+23 and 26+26dBm PAs

Same approach can then be applied also to other cases where two transmit line-ups are used to generate UL CA signal using the formulas for both equal and un-equal power between the tones or carriers. This result indicates that there should not be a need to define MPR values individually for each non-contiguous CA power class (or PA combination), but a delta value to the already existing requirements can be used.

The results of such analysis are collected into the table below, where column MA,delta shows the additional MPR that is needed vs. the reference, which is here PC2 (23+23=26dBm) MPR values that are already defined in the 38.101-1. Total MPR would then be MA,total= MA + MA,delta.

|  | C1 | C2 | C1+C2 | MA,delta |
| --- | --- | --- | --- | --- |
| 23+23= 26dBm | 23 | 23 | 26.0 | 0 (Reference) |
| 23+26= 27.8dBm | 23 | 26 | 27.8 | 0.6 |
| 26+26= 29dBm | 26 | 26 | 29.0 | 1.0 |
| 26+31= 32.2dBm | 26 | 31 | 32.2 | 0.8 |
| 31+31= 34dBm | 31 | 31 | 34.0 | 0.5 |
| 20+20= 23dBm | 20 | 20 | 23.0 | 0.0 |
| 20+23= 24.8dBm | 20 | 23 | 24.8 | 0.0 |

#### Proposal 5: Study possibility to avoid MPR simulations and analysis for each CA output power combination through intoducing MA,delta value (MA,total= MA + MA,delta) ,which is used to offset the reference MPR requirement to new CA output power levels and PA combinations.

# Conclusion

In this contribution, we proposed our view on HPUE for intra-band UL CA. The proposals are as follows.

#### Proposal 1: For dualPA-Architecture of PC1.5 intra-band non-contiguous UL CA, consider different PSD when LCRB1 is different from LCRB2.

#### Proposal 2: For dualPA-Architecture of PC1.5 intra-band non-contiguous UL CA, consider following PCMAX,C limitation as configured output power for each component carrier, and PCMAX limitation as configured total output power.

- PCMAX,CC1 = 26 dBm

- PCMAX,CC2 = 26 dBm

- PCMAX = 29 dBm

#### Proposal 3: For TxD-Architecture of PC1.5 intra-band contiguous UL CA, consider same PSD when LCRB1 is different from LCRB2.

#### Proposal 4: For TxD-Architecture of PC1.5 intra-band contiguous UL CA, consider following PCMAX,C limitation as configured output power for each component carrier, and PCMAX limitation  of 29dBm as configured total output power

- PCMAX,CC1 = ( 26+10*log10( LCRB1*SCS1LCRB1*SCS1+LCRB2*SCS2)+3 ) dBm

- PCMAX,CC2 = ( 26+10*log10( LCRB2*SCS2LCRB1*SCS1+LCRB2*SCS2)+3 ) dBm

- PCMAX = 29 dBm

#### Proposal 5: Study possibility to avoid MPR simulations and analysis for each CA output power combination through intoducing MA,delta value (MA,total= MA + MA,delta) ,which is used to offset the reference MPR requirement to new CA output power levels and PA combinations.

# References

[1] R4-2410750, “WF on higher power UE”, Samsung

[2] RP-240828, “New WID : UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, ATT