3GPP TSG-RAN WG4 Meeting #112	R4-2412025

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	(NR_RRM_enh-Core) Discussion on Rel 16 no-gap reporting

Agenda item:	4.5

Document for:	Discussion

Introduction

This paper presents a discussion on the identified problem and proposes how to clarify the interruption behavior for “no gap” in Rel -16.

Discussion

In this paper, the following scenarios are discussed:

- Scenario 1: the UE is connected to LTE cell with NR inter-RAT measurement objects

  - In this scenario the UE:

    - supports interRAT-NeedForGapsNR-r16,

    - is connected to an LTE eNB,

    - is configured to perform NR inter-RAT measurements.

- Scenario 2: the UE is connected to NR cell with NR measurement objects

  - In this scenario a UE

    - supports nr-NeedForGap-Reporting-r16 or interFrequencyMeas-Nogap-r16

    - is connected to NR gNB

    - is configured to perform NR measurements.

In the RAN4 #110 meeting the following tentative agreement was captured in [1]:

Issue 1-1-1: Scenario 1, LTE – NR inter-RAT measurement Tentative agreements - interRAT-NeedForGaps-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of Rel-18 NeedForInterruption: - Optional since R17 - [FFS mandatory report of the UE capability R18 interRAT-NeedForIntrNR-r18 from Rel-18 UE if the UE reports interRAT-NeedForGaps-r16=FALSE] - FFS the UE behavior if the Rel-18 UE does not support interRAT-NeedForIntrNR-r18 Issue 1-1-2: Scenario 2, NR measurements without gaps Tentative agreements - “no-gap” as part of NeedForGapsInfoNR-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of Rel-18 NeedForInterruption: - Optional since R17 - FFS the UE behavior if the Rel-18 UE does not support NeedForInterruptionNR-r18

The tentative agreement was reached in RAN4 but still pending confirmation, and missing to inform RAN2 of the outcome. Therefore, we propose that RAN4 confirms the tentative agreement removing FFS points, and sends an LS to RAN2 informing of the outcome.

Confirm tentative agreement for Scenario 1, LTE – NR inter-RAT measurement:

  - interRAT-NeedForGapsNR-r16=FALSE means that the UE support measurement without gaps

    - The UE may or may not cause interruption.

  - if UE causes interruptions when performing measurements without gaps:

    - Support early implementation of interRAT-NeedForInterruptionNR-r18:

      - Optional since Rel-17

Confirm tentative agreement for Scenario 2, NR measurements without gaps

  - “no-gap” as part of NeedForGapsInfoNR-r16 means that the UE support measurement without gaps

    - The UE may or may not cause interruption.

  - if UE causes interruptions when performing measurements without gaps:

    - Support early implementation of nr-NeedForInterruptionReport-r18:

      - Optional since Rel-17

Send RAN2 LS informing of the decision and requesting to take actions related to early implementation.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Confirm tentative agreement for Scenario 1, LTE – NR inter-RAT measurement:

- interRAT-NeedForGapsNR-r16=FALSE means that the UE support measurement without gaps

  - The UE may or may not cause interruption.

- if UE causes interruptions when performing measurements without gaps:

  - Support early implementation of interRAT-NeedForInterruptionNR-r18:

  - Optional since Rel-17

Proposal 2: Confirm tentative agreement for Scenario 2, NR measurements without gaps

- “no-gap” as part of NeedForGapsInfoNR-r16 means that the UE support measurement without gaps

  - The UE may or may not cause interruption.

- if UE causes interruptions when performing measurements without gaps:

  - Support early implementation of nr-NeedForInterruptionReport-r18:

    - Optional since Rel-17

Proposal 3: Send RAN2 LS informing of the decision and requesting to take actions related to early implementation.

- References

- R4-2403559, WF on Interruption requirements for Rel-16 measurements without gaps, Nokia, Nokia Shanghai Bell.

- Draft LS to RAN2

3GPP TSG-RAN WG4 Meeting #110 R4-24xxxxx Athens, Greece, 26th of February to 1st of March, 2024 Title: LS measurements without gap Release: Rel-16 Work Item: NR_RRM_enh-Core Source: RAN4 To: RAN2 Cc: Contact Person: #### Name: Rafael Paiva E-mail: rafael.paiva@nokia.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org 1. Overall Description: During Rel-18 discussion, RAN4 identified that the behaviour relative to measurements without gaps in Rel-16 was not clearly defined. As part of this discussion RAN2 requested information on UE behaviour when UE is reporting UE capability Rel-16 “no gap” indication R2-2306919, which was replied with R4-2314457. From existing RAN4 requirements, interRAT measurements did not allow for interruptions, as described in the following from 36.133: Inter-frequency and inter-RAT measurement requirements within this clause rely on the UE being configured with one measurement gap pattern unless the UE has signaled that it is capable according to the capability interFreqNeedForGaps or interRATNeedForGaps of conducting such measurements without gaps and without interruption.. The following is agreed regarding NR measurements with NeedForGapsInfoNR-r16 and interFrequencyMeas-Nogap-r16: - LTE – NR inter-RAT measurement: - interRAT-NeedForGapsNR-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of interRAT-NeedForInterruptionNR-r18: - Optional since Rel-17 - NR measurements without gaps - “no-gap” as part of NeedForGapsInfoNR-r16 means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of nr-NeedForInterruptionReport-r18: - Optional since Rel-17. RAN4 has not defined UE requirements for interRAT NR measurements in LTE allowing interruptions on PCell, PSCell and activated SCell, if configured, when UE reports “no gap”. Additionally, no interruption requirements were defined when the UE is reporting “no-gap” as part of NeedForGapsInfoNR-r16 Considering the discussion, RAN4 believes that early implementation of nr-NeedForInterruptionReport-r18 and interRAT-NeedForInterruptionNR-r18 will help solving the ambiguity in the Rel-16 requirements. For this reason, RAN4 would like to respectfully ask RAN2 to consider early implementation of the Rel 18 features nr-NeedForInterruptionReport-r18 and interRAT-NeedForInterruptionNR-r18 starting from Rel-17. 2. Actions: To RAN2 group: ACTION: RAN4 respectfully asks RAN2 to take the above information into account in their future work. 3. Dates of next TSG-RAN WG4 meetings: RAN4#112bis 14 – 18 October 2024 Hefei, CN RAN4#113 18 – 22 November 2024 Orlando, US