3GPP TSG-RAN WG4 Meeting #111	R4-2412026

Maastricht, NL, Aug 19th – 23rd, 2024

Source:	Nokia

Title:	(NR_SmallData_INACTIVE) Discussion on SDT R17 test parameters

Agenda item:	4.5

Document for:	Discussion

Introduction

This paper presents Nokia’s views on SDT R17 test parameters for FR2 test cases.

It was identified that the current spec text has some inconsistencies regarding relationship between power levels, SDT change threshold, and RSRP measurement accuracy. It is important to fix those mistakes in order for the RAN5 work to be able to proceed.

Discussion

From R4-2407998, we have:

### Sub-topic 6-1: Power levels and thresholds in SDT TCs - Proposals - Option 1 (Nokia): - The cg-SDT-RSRP-ChangeThreshold of A.7.2.1 should be changed to 14dB. Same applies for equivalent test in RedCap. The cd-SDT-RSRP-ChangeThreshold of A.6.2.1 is suggested changed to 8dB. Same applies for equivalent test in RedCap. - The values for the power in A.7.2.1 for T1, T3 and T6 to be changed from -100 to -104dBm/SCS, power for T2 from -87 to -80dBm/SCS and the values for T4 and T5 to be changed from -75.5dBm to -78dBm. Same applies for equivalent test in RedCap, resulting in an Io range from -72 to -49 dBm/BW. - The values for the power in A.6.2.1 for T2 to be changed from -92dBm to -88dBm and the values for T4 andT5 to be changed from -82dBm to -86dBm. Same applies for equivalent test in RedCap. The range for Io is within the range of -50 to -70dBm/BW. - RAN4 to review the P0 value of the RedCap SDT test for FR2. For SDT RedCap test case, a value of -104dBm/BW is suggested. - Recommended WF - Discuss the options

Table 1: Topic summary for maintenance on Small Data from R4-2407998.

The options in Table 1 are based upon the latest Nokia discussion paper on the topic, R4-2409134, which is again based on the RAN5 analysis of the SDT test cases for R17, where the problems in lacking rules setting the parameters have not been observed, see Figure 1.

Figure 1: The parameters from the SDT FR2 test from section A.7.2.1 drawn in real proportional dimensions based on version i40 of TS 38.133. The yellow boxes illustrate the valid range of measurement results at the UE during the test. The figure is based on figure A.7.2.1.1.1-1 and the measurement relative uncertainty is taken from table 10.1.3.1.2-1, for normal condition. In this case +/-6dB.

The rules observed in R4-2409134 are summarized as:

Relations between cg-SDT-RSRP-ChangeThreshold, relative measurement uncertainty, P0, P1 and P2 are:

cg-SDT-RSRP-ChangeThreshold≥2*uncertainty

And:

P2±uncertainty-P1±uncertainty≤cg-SDT-RSRP-ChangeThreshold

And:

P2±uncertainty-P0±uncertainty>cg-SDT-RSRP-ChangeThreshold

Translated into the exact test case, this applies:

P2-uncertainty-P0+uncertainty>cg-SDT-RSRP-ChangeThreshold
⇒P2-P0-2*uncertainty>cg-SDT-RSRP-ChangeTheshold
⇒P2>g-SDT-RSRP-ChangeThreshold+P0+2*uncertainty

And:

P2+uncertainty-P1-uncertainty≤cg-SDT-RSRP-ChangeThreshold
⇒P2-P1+2*uncertainty≤cg-SDT-RSRP-ChangeTheshold
⇒P2≤cg-SDT-RSRP-ChangeThreshold+P1-2*uncertainty or
P2-cg-SDT-RSRP-ChangeThreshold+2*uncertainty≤P1

Besides the above relations, RAN5 has requirements for the Io power levels of the test cases to be within the dynamic range of -50 to -70 dBm.

As a result, during RAN4 #111 it has been suggested to adjust the number of resource blocks in the test and to remove the noise in the Io calculations to try and fix the problem of the dynamic range and P0 values.

It has been suggested to adjust the number of resource blocks in the tests to mitigate dynamic range problems for the Io values.

It has been suggested to remove noise from the Io values to mitigate the dynamic range problems.

To reduce the amount of power levels presented, this paper will focus on P0, P1 and P2 levels, instead of T1 to T6 values. P0 is the power level for T1, T3 and T6. P1 is the power level for T2. P2 is the power level for T4 and T5.

P0 is the power level for T1, T3 and T6. P1 is the power level for T2. P2 is the power level for T4 and T5.

The number of RB’s used will be as suggested, 66 vs 24, corresponding to 95.04MHz and 34.56MHz respectively.

With the above, the following table is presented:

|  | With Noise |  |  | Without Noise |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | P0 | P1 | P2 | P0 | P1 | P2 |
| FR2 SS-RSRP [dBm] | -104 | -78 | -77 | -104 | -78 | -77 |
| Es/Noc [dB] | 0 | 26 | 27 | 0 | 26 | 27 |
| BW [MHz] | 95.04 | 95.04 | 95.04 | 95.04 | 95.04 | 95.04 |
| Io [dBm/BW] | -72 | -49 | -48 | -75 | -49 | -48 |
| FR2 SS-RSRP [dBm] | -102 | -76 | -75 | -102 | -76 | -75 |
| Es/Noc [dB] | 0 | 26 | 27 | 0 | 26 | 27 |
| BW [MHz] | 34.56 | 34.56 | 34.56 | 34.56 | 34.56 | 34.56 |
| Io [dBm/BW] | -74.4 | -51.4 | -50.4 | -77.4 | -51.4 | -50.4 |

Table 2: Io values for 66 and 24 RB's, with and without noise in the calculations.

The dynamic range of RSRP is 27dB for all calculations, which is the same as the maximum value of Es/Noc for the tests.

The dynamic range of Io is 24dB with noise and 27dB without

The dynamic range of the Io does not depend on the number of RBs used in the test.

The number of RB’s have impact on the minimum RSRP value used, P0, for the test to stay within the Io range of -50 to -70dBm/BW, but does not have any impact on the Io dynamic range.

RAN4 to consider the numbers presented and present a way forward for testing of SDT in FR2 given the RSRP dynamic range forced by the large in-accuracy of FR2 measurements.

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: It has been suggested to adjust the number of resource blocks in the tests to mitigate dynamic range problems for the Io values.

Observation 2: It has been suggested to remove noise from the Io values to mitigate the dynamic range problems.

Observation 3: P0 is the power level for T1, T3 and T6. P1 is the power level for T2. P2 is the power level for T4 and T5.

Observation 4: The number of RB’s used will be as suggested, 66 vs 24, corresponding to 95.04MHz and 34.56MHz respectively.

Observation 5: The dynamic range of RSRP is 27dB for all calculations, which is the same as the maximum value of Es/Noc for the tests.

Observation 6: The dynamic range of Io is 24dB with noise and 27dB without

Observation 7: The dynamic range of the Io does not depend on the number of RBs used in the test.

Observation 8: The number of RB’s have impact on the minimum RSRP value used, P0, for the test to stay within the Io range of -50 to -70dBm/BW, but does not have any impact on the Io dynamic range.

Proposal 1: RAN4 to consider the numbers presented and present a way forward for testing of SDT in FR2 given the RSRP dynamic range forced by the large in-accuracy of FR2 measurements.