3GPP TSG-RAN WG4 Meeting #112	R4-2412027

Maastricht, Netherlands, 19-23 August 2024

Source:	Nokia

Title:	On Rel-18 Multi-Rx core part maintenance

Agenda item:	5.13.1

Document for:	Discussion

Introduction

Rel-18 multi-Rx work item was closed in the last RAN4 meeting including the performance part of the work item. However, some unfinished issues were still listed in the way forward of RAN4#111 meeting in [1]. In this contribution we discuss these issues.

Measurement restrictions

Lengthy discussions took place in the RAN4#111 meeting about the multi-Rx measurement restrictions, and the following was captured in the WF:

Issue 1-3-2: measurement restriction for sDCI and mDCI Background: As per RAN1 spec highlighted in Huawei contribution R4-2408558 - timeDurationForQCL - SCS of 60 kHz: s7, s14, s28 - SCS of 120 kHz: s14, s28. - sTRP - Moderator Observation for sTRP: - Same slot scheduling: - PDSCH always have QCL assumption of lowest CORESET. QCL source corresponding to indicated TCI state in DCI is only used for cross slot scheduling. In other words, for same slot scheduling, PDCCH and PDSCH has same QCL assumptions - Cross slot scheduling: - If the scheduling offset is more than timeDurationForQCL, it is possible to use the beam indicated in TCI state as QCL source, else it is always the same beam as PDCCH or the lowest CORESETID is used for PDSCH reception. mTRP sDCI: - Moderator Observation for mTRP sDCI: - Same slot scheduling or Scheduling offset is lower than timeDurationForQCL: - PDSCH always use QCL assumption of lowest codepoint among the TCI code points containing two different TCI states. That means the TCI state indicated in DCI is not used for same slot scheduling. - Cross slot scheduling: - If the scheduling offset is more than timeDurationForQCL, beam indicated in TCI state are used. - Moderator Observation for mTRP mDCI: - For each TRP, sTRP observation applies. After PDCCH processing time, UE knows if there is an overlapped PDSCH with CSI-RS or not. For simplicity, let’s assume PDCCH processing time as timeDurationForQCL. That means if the scheduling offset is before timeDurationForQCL, UE need to buffer the data worth of timeDurationForQCL and process it later. If the scheduling offset is larger than timeDurationForQCL, UE can know what’s the QCL assumptions for the PDSCH and it can determine whether to receive on single beam or beam pair. - Moderator recommendation - Same slot scheduling or scheduling offset less than timeDurationForQCL - Option 1: Remove the measurement restrictions (i.e., remove the following conditions) - The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol - The CSI-RS for L1-RSRP has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH - Option 2: Modify the measurement restrictions - the measurement restriction relaxation can be modified as follows: - Both the CSI-RS for L1-RSRP and the other CSI-RS are not in any CSI-RS resource set configured with repetition ON, and - Resources of the active TCI states or default QCL assumption as defined in TS38.214 for two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report, and - [The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol, and] - The CSI-RS for L1-RSRP has same QCL source as the active TCI state/or default QCL assumption of one [PDSCH], and the other CSI-RS has same QCL source as the active TCI state / or default QCL assumption of the other [PDSCH] - Cross slot scheduling when scheduling offset is larger than timeDurationForQCL - Option 1: Remove the measurement restrictions (i.e., remove the following conditions) - The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol - The CSI-RS for L1-RSRP has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH - Option 2: Modify the measurement restrictions - the measurement restriction relaxation can be modified as follows: - Both the CSI-RS for L1-RSRP and the other CSI-RS are not in any CSI-RS resource set configured with repetition ON, and - Resources of the active TCI states or default QCL assumption as defined in TS38.214 for two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report, and - [The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol, and] - The CSI-RS for L1-RSRP has same QCL source as the active TCI state/or default QCL assumption of one [PDSCH], and the other CSI-RS has same QCL source as the active TCI state / or default QCL assumption of the other [PDSCH]

Regarding measurement restriction, we try to analyse the reasons why some companies would like to keep the following condition:

- [The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol].

The current text for L1-RSRP measurement restriction contains the following conditions:

For FR2-1, when the CSI-RS for L1-RSRP measurement on the one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC, UE supporting [multi-Rx capability] according to the conditions described in clause 3.6.y shall be able to measure the CSI-RS for L1-RSRP measurement without restriction when the following conditions are met: - Both the CSI-RS for L1-RSRP and the other CSI-RS are not in any CSI-RS resource set configured with repetition ON, and - Resources of the active TCI states for two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report, and - [The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol, and] - The CSI-RS for L1-RSRP has same QCL source as the active TCI state of one [PDSCH], and the other CSI-RS has same QCL source as the active TCI state of the other [PDSCH], and Editor’s note 1: FFS the condition CSI-RS(s) and PDSCH(s) are overlapped on the same OFDM symbol(s). Editor’s note 2: FFS how to capture UE is activated with multi-Rx operation.

In this text we will refer to those conditions as

- Condition 1: The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol,

- Condition 2: The CSI-RS for L1-RSRP has same QCL source as the active TCI state of one [PDSCH], and the other CSI-RS has same QCL source as the active TCI state of the other [PDSCH],

In our view Condition 1 and Condition 2 have similar impact on the concerns that were presented over the last few meetings.

One argument in favor of Condition 1 is that the UE might be using a different beam for monitoring CSI-RS than the beam indicated for simultaneous reception if there is no PDSCH overlapping with CSI-RS. That is true if we ignore the agreed scheduling restrictions conditions. When both Conditions 1 and Condition 2 are met, there are no scheduling restrictions, and when that happens the UE is assumed to adopt N=1, otherwise the UE cannot be scheduled on those symbols.

When scheduling restrictions don’t apply, the expected beam sweeping factor is N=1 for L1 FR2 measurements.

If Condition 2 is met, the UE will have to be prepared to receive data on the symbols overlapping with the CSI-RS. In this situation, the TCI states for PDSCH are activated, and the UE has to be prepared to receive data within timedurationForQCL, so there is no chance of doing beam sweeping in that situation. Therefore, when Condition 2 is met, the UE should be using the beam pair that was identified for simultaneous reception through GBBR-r17.

When the CSI-RS for L1-RSRP has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH, the UE is expected to use Rx beam pair optimized through GBBR-r17 for simultaneous reception without further beam sweeping.

One drawback of Condition 1 is that it forces the network to always schedule the UE on top of CSI-RS to avoid the measurement restrictions. When measurement restrictions apply, if the network configures overlapping RSs the RAN4 requirements state that longer delay is expected, and no requirements are defined. For this reason, the network should always avoid meeting the measurement restriction conditions that will imply in no requirements, otherwise the UE behavior is unexpected. This is a very negative condition from the network perspective, scheduling of PDSCH is very dynamic, and it is not desirable that measurement restriction conditions are changing frequently.

When measurement restrictions apply, RAN4 requirements state that longer delay is expected and no requirements apply.

Figure 2 shows the interpretation of Condition 1. In that situation the UE reports GBBR with one pair (CSI-RS#0+CSI-RS#1). The network then activates that TCI pair and indicates it. In this figure 2 measurement cycles with 3 samples are shown. In the first measurement cycle the UE collects 3 samples of CSI-RS#0+CSI-RS#1 with PDSCH scheduled on top of it. This case follows exactly Condition 1, and we would have the respective requirements. In the measurement cycle 2, the network doesn’t schedule the UE in the second occasion where CSI-RS#0+CSI-RS#1 are transmitted simultaneously. In that case, there are measurement restrictions for 1 out of 3 CSI-RS transmissions, and the measurement requirements do not apply.

Figure 3 shows the same analysis for Condition 2. In that situation there is no problem if few measurements occasions are not overlapping with PDSCH, as long as CSI-RS#0+CSI-RS#1 is co-located with the activated TCI states used for simultaneous reception.

A condition of PDSCH scheduling for measurement restrictions would imply that requirements change for each measurement occasion depending on the scheduling.

Additionally, for Condition 2, we should remove the square brackets around PDSCH. Form the Rel-16 TCI framework, only PDSCH is received simultaneously, so we need dual PDSCH TCI in order for the Condition 2 to make sense.

Measurement restrictions due to multi Rx operation are enhanced when the following condition is met: The CSI-RS for L1-RSRP has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH.

Figure 2 Interpretation of Condition 1

Figure 3 Interpretation of Condition 2

DCI-based TCI state switch

The following issue about how to handle the intermediate state of the case when DCI-based TCI state in m-DCI happens for one TRP after the other has been FFS for multiple meetings:

Issue 2-1-1: DCI based dual TCI state switch delay for mDCI: Issue 1-4-1: DCI based dual TCI state switch delay for mDCI: - Proposals: - Proposal 1: For DCI based dual TCI state switching in mDCI, the 2 TCI state switching procedures can be treated independently. In other words, UE should be able to receive PDSCH with a target TCI state of which the switching is finished earlier. - Proposal 2: If TCI 1 and TCI 2 are in a beam pair, UE to receive on TCI 1 and TCI 2 between C and D. After D, to receive on TCI 2 and TCI 4. Between C and D, UE capable of multi-Rx can receive overlapping PDSCH 0 and PDSCH 1 simultaneously. Otherwise, UE to receive on TCI 2 alone till D. After D, UE can receive on TCI 2 and TCI 4. - Proposal 3: No additional requirements are needed for DCI based dual TCI state switch delay for m-DCI. - Proposal 4: - For mDCI based dual DCI state switch, TCI state switch on each coreset is independent without any restriction on the DCI reception. - Between point C and D, UE to receive on TCI state 0 alone, if new TCI state 0 and old TCI state 1 are not in a beam pair. - Proposal 5: - Update the DCI based dual TCI state switch for m-DCI with “UE shall be able to receive PDSCHs with target TCI states simultaneously after slot max(n1, n2) + timeDurationForQCL - the UE can receive simultaneously TCI 1 and TCI 2 between points C and D if they have been reported as a beam pair using GBBR-17 - Recommended WF: - For DCI based TCI state switch, switching happens within CP and UE can receive entire timeDurationForQCL. Based on this assumption, please discuss following. - Please further discuss whether following is agreeable - If TCI 1 and TCI 2 are a beam pair. - UE to receive on TCI 1 and TCI 2 between C and D. After D, to receive on TCI 2 and TCI 4 - If TCI 1 and TCI 2 are not a beam pair. - No requirements till point D. i.e., UE is not expected to receive on TCI 2 till point D.

In our view, if TC1 and TCI2 in the above figure are a beam pair then the UE can receive TCI 1 and TCI 2 simultaneously between points C and D. During a DCI based dual TCI state switch in m-DCI, a UE which is initially receiving on two TCI states i.e. TCI 0 and TCI 1, switches from TCI 0  to TCI 2 and is in the process of switching from TCI 1 to TCI 4, can in the interim, receive TCI 1 and TCI2 simultaneously if they form a beam pair, i.e. the RS for TCI 1 and TCI 2 have been reported as a beam pair using GBBR-r17.

For Issue 2-1-1, the UE can receive simultaneously TCI 1 and TCI 2 between points C and D if they have been reported as a beam pair using GBBR-17.

We have provided a text proposal for this in our CR [2].

Conclusion

The following observations and proposals were made in this contribution:

Observation 1: When scheduling restrictions don’t apply, the expected beam sweeping factor is N=1 for L1 FR2 measurements.

Observation 2: When the CSI-RS for L1-RSRP has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH, the UE is expected to use Rx beam pair optimized through GBBR-r17 for simultaneous reception without further beam sweeping.

Observation 3: When measurement restrictions apply, RAN4 requirements state that longer delay is expected and no requirements apply.

Observation 4: A condition of PDSCH scheduling for measurement restrictions would imply that requirements change for each measurement occasion depending on the scheduling.

Proposal 1: Measurement restrictions due to multi Rx operation are enhanced when the following condition is met: The CSI-RS for L1-RSRP has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH.

Proposal 2: For Issue 2-1-1, the UE can receive simultaneously TCI 1 and TCI 2 between points C and D if they have been reported as a beam pair using GBBR-17.

- References

[1] R4-2410356, WF on core maintenance and performance part for multi-Rx, vivo, Ericsson, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka, Japan, May 2024.

[2] R4-2412492, CR for Rel-18 multi-Rx TCI state switching delay, Nokia, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, 19-23 August 2024.