3GPP TSG-RAN WG4 Meeting #112	R4-2412028

Maastricht, Netherlands, 19-23 August 2024

Source:	Nokia

Title:	On multi-Rx RRM performance part

Agenda item:	5.13.2

Document for:	Discussion

Introduction

Most of the multi-Rx performance issues in the Rel-18 multi-Rx work item were agreed in the last RAN4 meeting. However, there are some issues which need to be resolved which we bring forth in this contribution.

Test cases

New AoA setup for multi-Rx

The following was agreed in RAN4 for 3 AoA test setup

A.3.15.X2    Setup X2: 3 AoAs for simultaneous reception with different QCL Type-D

There are 3 active probes in the test and the DL signals and noise are transmitted from the three active probes.

Out of the three AoA, one AoA [(AoA1)] is aligned to a direction which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for UE power class 3 and other 2 AoAs [(AoA2, AoA3)] are from the set of [qualified AoA pairs] according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 for power class 3 supporting simultaneous reception from multiple directions.

The relative angular offset between the directions of the AoA pair is based on the UE’s declared [AoA separation and UE] orientation as defined in clause 7.3K.3 of TS 38.101-2 and [shall not be changed for each test iteration].

The test setup now indicates that UE needs to receive from 3 AoAs, of which one AoA is a direction identified as being withing the UE’s spherical coverage during the EIS spherical coverage test while the other 2 AoAs are a set of directions identified as being within the UE’s spherical coverage during the spherical coverage test during simultaneous reception from multiple directions.

Both these spherical coverage tests will be carried out separately and the AoAs from each of these tests will also be identified separately.

Since the direction and placement of the probes in the anechoic chamber during testing is fixed, the UE can receive only those beams which are within its spherical coverage

However, during the TCI state switch test using 3 AoA setup, the UE should have all these three AoAs within its spherical coverage to carry out the test seamlessly.  For example, even in case of single to dual TCI state switch, UE has to receive from one TCI state while it measures and reports that it can receive simultaneously from two other TCI states. Hence, it can be assumed that all the three probes need to be within the UE spherical coverage during the test. This means that the UE needs to be oriented such that the spherical coverage requirements for the AoA1 as well as the AoA2, AoA3 pair are met simultaneously.

During the TCI state switch test, when the UE has to switch from two TCI states to two new TCI states, all three probes (AoA1, AoA2, AoA3) need to be within the UE’s spherical coverage so that it can receive from two TCI states initially and switch to the target two TCI states after the TCI state switch command without any change in its orientation.

The text currently is not clear that the UE should be oriented such that all the 3 AoAs shall be within the UE’s spherical coverage during the test.  Hence, to avoid any ambiguity in the performance specifications, we propose that last line of this text be modified as proposed:

The UE will be oriented such that the spherical coverage requirements for the AoA1 as well as the AoA2, AoA3 pair are met simultaneously.

Update the 3 AoA test setup with the following “The UE positioning shall be such that the UE passes both spherical coverage requirements..

Test cases for TCI state switch

During RAN4 #111 the following agreed regarding TCI switching test cases [1]

Issue 2-7: Test case(s) for dual TCI state switching for m-DCI

- Introduce one test case for DCI-based dual TCI state switching for m-DCI

- From [RS1, RS2] (Probe 1 and 2, no simultaneous transmission) to [RS3, RS4] (Probe 1 and 3) with 3 active probes.

  - Side conditions

    - Probe 1 & 3 shall fulfill the mRx spherical coverage EIS requirement (angular offset as declared by the UE). Note: Aligned with 2AoA selected as we agreed.

    - In addition, Probe 1 & 2 shall fulfill individually the legacy spherical coverage EIS requirement and angular offset from the legacy 2AoA table, i.e. selection is equivalent to a single pair from Setup 3

  - Signal mapping

    - RS1 & RS3 are mapped to Probe 1

    - RS2 is mapped to Probe 2

    - RS4 is mapped to Probe 3

It was agreed in the last meeting that test case for dual TCI state switching for m-DCI would be defined using three probes. Of the three probes, RS1 and RS3 will be mapped to probe 1 while RS2 will be mapped to probe 2 and RS4 will be mapped to probe 3.

This means that initially during the test, the UE would receive from RS1 and RS2 and would then receive TCI state switch command to receive from RS3 and RS4. For the UE to receive from RS3 and RS4 it needs to be time and frequency synchronized with these RS. Since RS1 and RS3 are transmitted from the same probe, it means the UE doesn’t need to do a time and frequency synchronization with RS3 which would provide incorrect conformance test results.

In test case for dual TCI state switch for m-DCI, three probes are used of which one probe will transmit two RS.

- Since the UE is already receiving from one of the RS transmitted by the probe before the TCI state switch, if it has to receive from another RS from the same probe, then the time/frequency synchronization which the UE needs to perform with the target RS cannot be tested accurately.

To have meaningful test results in such situations, the test probe transmitting both RS1 and RS3 should emulate different radio conditions for RS1 and RS3. For example, RS1 should be transmitted at a different time/frequency and power as compared to RS3.

When three probes are used to transmit four RS, the test probe transmitting more than one RS should emulate different DL transmit beams by transmitting different RS with different power and delay. This will ensure that the UE has done a time/frequency synchronization with the target RS before switching to the target TCI state when receiving a DCI command for a TCI state switch

Furthermore, in the m-DCI test case as it is currently written in the specification, the TE sends two DCIs to switch to the target beam pair in the same slot. Support of simultaneous PDCCH reception is not supported mandatorily in Rel-18, so all Rel-18 UES would not be able to receive two PDCCH transmissions simultaneously. Hence, the two DCIs should be sent in different slots. For the purposes of the test case, we think the two DCIs can be sent in two consecutive slots n and n+1, and the UE should be able to receive the two target TCI states simultaneously at slot n + 1 + timeDurationForQCL.

In m-DCI TCI state switching test case, because support of simultaneous PDCCH reception is not mandatory for a Rel-18 UE, the TE shall send the two DCIs to switch to the target TCI states in consecutive slots n and n+1. The UE shall be able to receive the target TCI states simultaneously at slot n + 1 + timeDurationForQCL.

Another issue with the test case is that the test switches UEs PDSCH TCI states, while the PDCCH TCI states remain as they are during the test. This does not come clear in the test setup, where the TCI states for PDCCH and PDSCH should be the following at the beginning and at the end of the test:

- TCI states at the beginning of the test:

  - PDCCH TCI states: TCI state 0 (probe 0, SSB0) and TCI state 1 (probe 1, SSB1)

  - PDSCH TCI states: TCI state 0 (probe 0, SSB0) and TCI state 1 (probe 1, SSB1)

- TCI states at the end of the test:

  - PDCCH TCI states: TCI state 0 (probe 0, SSB0) and TCI state 1 (probe 1, SSB1)

  - PDSCH TCI states: TCI state 2 (probe 2, SSB2) and TCI state 3 (probe 0, SSB3)

Distinguish PDCCH and PDSCH TCI states clearly in the m-DCI TCI state switching test case. The TCI states and associated probes and SSBs should be:

  - TCI states at the beginning of the test:

    - For CORESETPoolIndex 0

      - PDCCH TCI state: TCI state 0 (probe 0, SSB0)

      - PDSCH TCI states: TCI state 0 (probe 0, SSB0)

    - For CORESETPoolIndex 1

      - PDCCH TCI state: TCI state 1 (probe 1, SSB1)

      - PDSCH TCI states: TCI state 1 (probe 1, SSB1)

  - TCI states at the end of the test:

    - For CORESETPoolIndex 0

      - PDCCH TCI state: TCI state 0 (probe 0, SSB0)

      - PDSCH TCI states: TCI state 3 (probe 0, SSB3)

    - For CORESETPoolIndex 1

      - PDCCH TCI state: TCI state 1 (probe 1, SSB1)

      - PDSCH TCI states: TCI state 2 (probe 2, SSB2)

L1-RSRP test case

Issue 1-2: Gain accuracy in tests for verifying multi-Rx L1-RSRP accuracy requirements

- The existing G for Rx beam peak direction for PC3 is applicable for multi-Rx UEs.

- Not add additional margin on top of the legacy methodology used in the test setup

- Define test case to verify the accuracy requirements for multi-Rx.

- If the new test case is conducted, the corresponding legacy test will be skipped.

In the RAN4 #111 meeting it was agreed to define a new test case verifying the accuracy requirements for multi-Rx and if the UE passes this test then the corresponding legacy test can be skipped.

However, it is not clear which corresponding legacy test can be skipped. Hence, we propose to add the following in Section

Note: A UE which fulfils the requirements in test case A.7.7.X.1 can skip the test cases in A.7.7.4.1

Add a note in test case A.7.7.X clarifying that the UE can skip the test case in A.7.7.4 if it fulfils the requirements in test case A.7.7.X

Conclusion

The following observations and proposals were made in this contribution:

Observation 1: Since the direction and placement of the probes in the anechoic chamber during testing is fixed, the UE can receive only those beams which are within its spherical coverage

Observation 2: During the TCI state switch test, when the UE has to switch from two TCI states to two new TCI states, all three probes (AoA1, AoA2, AoA3) need to be within the UE’s spherical coverage so that it can receive from two TCI states initially and switch to the target two TCI states after the TCI state switch command without any change in its orientation.

Proposal 1: Update the 3 AoA test setup with the following “The UE positioning shall be such that the UE passes both spherical coverage requirements..

Observation 3: In test case for dual TCI state switch for m-DCI, three probes are used of which one probe will transmit two RS.

Proposal 2: When three probes are used to transmit four RS, the test probe transmitting more than one RS should emulate different DL transmit beams by transmitting different RS with different power and delay. This will ensure that the UE has done a time/frequency sychronization with the target RS before switching to the target TCI state when receiving a DCI command for a TCI state switch

Proposal 3: In m-DCI TCI state switching test case, because support of simultaneous PDCCH reception is not mandatory for a Rel-18 UE, the TE shall send the two DCIs to switch to the target TCI states in consecutive slots n and n+1. The UE shall be able to receive the target TCI states simultaneously at slot n + 1 + timeDurationForQCL.

Proposal 4: Distinguish PDCCH and PDSCH TCI states clearly in the m-DCI TCI state switching test case. The TCI states and associated probes and SSBs should be:

- TCI states at the beginning of the test:

  - For CORESETPoolIndex 0

    - PDCCH TCI state: TCI state 0 (probe 0, SSB0)

    - PDSCH TCI states: TCI state 0 (probe 0, SSB0)

  - For CORESETPoolIndex 1

    - PDCCH TCI state: TCI state 1 (probe 1, SSB1)

    - PDSCH TCI states: TCI state 1 (probe 1, SSB1)

- TCI states at the end of the test:

  - For CORESETPoolIndex 0

    - PDCCH TCI state: TCI state 0 (probe 0, SSB0)

    - PDSCH TCI states: TCI state 3 (probe 0, SSB3)

  - For CORESETPoolIndex 1

    - PDCCH TCI state: TCI state 1 (probe 1, SSB1)

    - PDSCH TCI states: TCI state 2 (probe 2, SSB2)

Proposal 5: Add a note in test case A.7.7.X clarifying that the UE can skip the test case in A.7.7.4 if it fulfils the requirements in test case A.7.7.X

- References

- R4-2410356        WF on core maintenance and performance part for multi-Rx, vivo, Ericsson,

- R4-2410364	Draft CR to 38.133 for AoA set up for multi-rx and TC for GBBR measurement accuracy, Ericsson, Samsung

- R4-2410363	Draft CR on accuracy requirements for L1-RSRP measurements with groupbasedbeamreporting, Samsung, Ericsson