3GPP TSG-RAN WG4 Meeting #112	R4-2412029

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Remaining issues for MG_enh2 RRM core requirements

Agenda item:	5.15.1

Document for:	Discussion

Introduction

This paper presents Nokia’s views on measurements without gaps in Rel 18.

Discussion for measurements without gaps

Interruptions with DRX ON

In the last RAN4 meeting, the following was open regarding DRX [1]

Issue 1-1-1: Misalignment between DRX-on duration and SMTC for NFG measurements - Proposals - Option 1: - Option 1a: Interruptions are always allowed outside DRX ON duration and it is according to Tcycle,i. - Option 1b: Interruptions are not allowed during DRX ON duration. - Option 1c: Interruptions are not allowed when DRX cycle is larger than 320ms. - Option 1d: Interruptions are not allowed in the DRX ON duration, excluding the time extended due to drx-inactivityTimer. - Option 2: Interruptions are allowed under the conditions of issues 1-1-2, 1-1-3, 1-1-4. Issue 1-1-3: Aligned DRX-on duration and SMTC for NFG measurements - Agreement - Interruption ratio requirement not based on DRX-on duration - Not define the interruption location

If the UE causes an interruption during the DRX ON period it could miss the PDCCH transmission. If an interruption, which is not a scheduling restriction period on the NW side, is colliding with a PDCCH occasion, especially during long DRX cycles, the device will lose the opportunity for UL and DL grants. If the PDCCH occasion is the only PDCCH occasion during the on-period of the long DRX period, the device is not reachable until next on-period of the DRX configuration, which will result in a long latency for retransmission of data.

The effect of missing PDSCH is not as severe as missing PDCCH due to an interruption. If the UE receives PDCCH it will be aware of the extension of the activity time and could receive the retransmission. If the UE misses the reception of PDCCH due to an interruption, it will not be aware that the DRX ON is extended due to the DRX inactivity timer and might miss data retransmissions. Additionally, since the DRX cycle can be configured as multiples of the SMTC periodicity, it is possible that in some more extreme cases the UE will systematically place an interruption on top of the DRX ON duration. That could even cause that one DL grant would be delayed for several DRX cycles.

Figure 1 Example of impact of interruptions during DRX ON with SMTC periodicity configured with same periodicity as DRX cycle.

Figure 1 shows one example of the impact of interruptions during DRX ON. In this example the SMTC and DRX have the same periodicity, and the DRX ON is located at a position where the UE is systematically causing interruptions during DRX ON. If there would be no interruptions, the UE would be able to receive data during or shortly after the first DRX ON, which means that the data latency would be about 1 DRX cycle in wort case. Here the UE loses the PDCCH scheduling the UE for DL grants in 2 consecutive DRX cycles. For each PDCCH lost on DRX ON, the network may attempt several PDSCH retransmissions during the DRX inactivity timer. As a result, the total latency is increased from less than 1 DRX cycle to between 2 and 3 DRX cycles. Additionally, the network is also wasting its resources when the UE is not available.

Interruptions on PDCCH on the DRX cycle can cause the UE to experience a further delay in UL and DL grants as long as the drx-LongCycle, which can be configured from 10 ms to 10 s.

The impact of interruption is more severe on PDCCH than for PDSCH during DRX activity time.

Considering the negative impact that interruptions have on DRX ON duration is much more damaging to the UE performance than when DRX is not configured, we believe that interruptions should not be allowed during DRX ON.

Interruptions are not allowed in the DRX ON duration.

Requirements with dual connectivity

The current requirements for interruptions due to measurements without gaps are captured in the NR Standalone clause in 38.133. It is not clear in the current state of the specification whether the interruption requirements also apply when the UE is configured with NR-DC, EN-DC, NE-DC. In the last RAN4 meeting, the following issue was open for discussion [1]:

Issue 1-2-1: Interruption requirements in 8.2.2.2.19 apply also for NR-DC, EN-DC, and NE-DC - Background: - the NFG signalling is used in NR SA only, as shown below: From 38.331: – NeedForGapsInfoNR The IE NeedForGapsInfoNR indicates whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.. - Proposals - Option 1: Yes. Interruption requirements in 8.2.2.2.19 shall also apply for NR-DC, EN-DC, and NE-DC considering that operations in one cell group do not impact operations on another cell group. - Option 1a: Same interruption requirement defined at 7.8.2.22 in 36.133 and 8.2.2.2.19 in 38.133 are applied for each cell group for UE perform NR measurement without gap with interruption. - 7.8.2.22 in 36.133 and 8.2.2.2.19 in 38.133 is applied for MCG and SCG, respectively in EN-DC, - 7.8.2.22 in 36.133 and 8.2.2.2.19 in 38.133 is applied for SCG and MCG, respectively in NE-DC, - 8.2.2.2.19 in 38.133 is applied for both MCG and SCG in NR-DC. - Option 2: NFG requirements are applicable for NR SA only. - Option 2a: deprioritize MR_DC for NFG in objective 2 of the WI.

According to the discussion of the last meeting, NeedForGaps in Rel-16 already excludes dual connectivity. Therefore, it doesn’t make sense that the Rel 18 features would apply for DC scenarios. Additionally, the same should be also considered in the RAN2 specification for the description of the Rel-18 features.

NFG requirements are applicable for NR SA only.

Send LS to RAN2 informing of the decision.

Scenarios for measurements without gaps

In the CRs agreed in the last meetings, some text was added regarding HST and FR2-2. In our view, RAN4 has never actively discussed the applicability of measurements without gaps with interruptions for HST and FR2-2. Those scenarios differ from the usual FR1 and FR2-1 discussion. For HST scenario, mobility is a very important issue, and extension of measurement delays due to the minimum measurement cycle can affect the system performance. Additionally, in FR2-2 beam sweeping factor is increased from 8 to 12. That means that measurement delays are already extended when operating in frequencies above 52.6 GHz, and RAN4 didn’t discuss whether this would be a problem to set a lower bound on measurement cycle for the scenario with extended beam sweeping scaling factor.

In the last RAN4 meeting the following issue was left for discussion [1]:

Issue 1-3-3: Cross feature support - Proposals - Proposal 1: Rel 18 measurements without gaps with interruptions do not apply for HST. - Proposal 2: Rel 18 measurements without gaps with interruptions do not apply for operation above 52.6 GHz. - Agreement - Rel-18 requirements for UE supporting NFG and inter-RAT measurements without gap do not apply for FR2-2.

Since this work item is officially closed, we think that the best solution is that all the references regarding HST in combination with measurements without gaps and interruptions are removed from 38.133 and 36.133.

RAN4 didn’t discuss whether HST should be considered for supporting Rel 18 measurements without gaps with interruptions.

Rel 18 measurements without gaps with interruptions do not apply for HST.

Discussion for concurrent gap measurements

Case 1 requirements

One remaining open issue at RAN4 #111 was how to specify the UE behavior in case dynamic collisions are not supported. We are contributing the proposed specification change in [2].

RAN4 to agree the proposed changes in R4-2413309 related to UE behavior in case dynamic collisions are not supported.

Case 2 requirements

One remaining open issue at RAN4 #111 was how to specify the UE behavior in case of deactivated SCell measurements, based on the agreement that NCSG is used. We are contributing the proposed specification change in [3].

RAN4 to agree the proposed changes in R4-2413310 related to UE behavior in case of deactivated SCell measurements with NCSG.

Conclusion

The following Observations and Proposals were made:

Observation 1: Interruptions on PDCCH on the DRX cycle can cause the UE to experience a further delay in UL and DL grants as long as the drx-LongCycle, which can be configured from 10 ms to 10 s.

Observation 2: The impact of interruption is more severe on PDCCH than for PDSCH during DRX activity time.

Proposal 1: Interruptions are not allowed in the DRX ON duration.

Proposal 2: NFG requirements are applicable for NR SA only.

Proposal 3: Send LS to RAN2 informing of the decision.

Observation 3: RAN4 didn’t discuss whether HST should be considered for supporting Rel 18 measurements without gaps with interruptions.

Proposal 4: Rel 18 measurements without gaps with interruptions do not apply for HST.

Proposal 5: RAN4 to agree the proposed changes in R4-2413309 related to UE behavior in case dynamic collisions are not supported.

Proposal 6: RAN4 to agree the proposed changes in R4-2413310 related to UE behavior in case of deactivated SCell measurements with NCSG.

- References

- R4-2410406, WF on NR_MG_enh2_part2, Intel

- R4-2413309, CR 38.133 Corrections to Case 1 requirements for NR_MG_enh2, Nokia

- R4-2413310, CR 38.133 Corrections to Case 2 requirements for NR_MG_enh2, Nokia