3GPP TSG-RAN WG4 Meeting #112	R4-2412035

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	TDCP RRM Requirements

Agenda item:	5.27.2

Document for:	Discussion

Introduction

In this paper we present Nokia’s updated simulation results for TDCP RRM requirements based on the agreements from previous meetings.

The simulation parameters used are shown in Appendix A.

Discussion

In the previous RAN4 (#110bis) meeting, the following agreement was reached regarding TDCP test cases:

Issue 2-1-1: Test metric of TDCP test cases: < Agreement> : Agreements for the test case definition: - Define the same test case for both 15kHz with FDD and and 30kHz with TDD in the same sub-clause. - Channel models: TDL-A30 - Doppler: - Two TCs with low and high Doppler - TC1: low (10) for both 15kHz and 30kHz - TC2: high - Option 1: high (300) - Option 2: 100 for 15kHz SCS, and 200 for 30kHz SCS - BW: - 10MHz for FDD and TDD - SNR: - Option 1: 20dB for TC1 - Option 2: 10dB or 20dB for TC2 - The distance between two TRSs: 1 - Report index: Bring CR for both options, make decision in the next meting - Option 1: - lower Doppler: CDP at X1 is higher than Y1 = [90] % - high Doppler: CDP at X2 is lower than Y2= [10] % - X1, X2, Y1, and Y2 can be different for TDD and FDD - Option 2: - [X1, X2] for Y2= FFS

Furthermore, in the last RAN4 (#111) meeting, the following agreement was reached regarding TDCP.

Issue 2-1-1: Test metric of TDCP test cases: < Agreement> : - Lower Doppler (10Hz, 20dB): reported TDCP index is no bigger than a certain value (e.g., 6), with at least 80% probability - TC1 for low doppler condition+15kHz SCS FDD: value 6 - TC2 for low doppler condition+30kHz SCS TDD: value 5 - Higher Doppler - TC3 for high doppler condition+15kHz SCS FDD, 300 Hz, SNR=[10]dB - Reported TDCP index is larger than 8, with at least 80% probability - TC4 for high doppler condition+30kHz SCS TDD, 300Hz, SNR=[10]dB - Reported TDCP index is [equal to or] larger than 6, with at least 80% probability

From Nokia’s previous submissions in RAN4 #110bis [1] and 111 [2], we provided the simulation results for the case of 30 KHz SCS and 15 KHz SCS respectively for the carrier frequency of 3.5 GHz. We now provide updated results with 20% and 80% CDF points for estimated TDCP.

Table 1: Results (Unquantized amplitudes).

| SCS (KHz) | Doppler | SNR | CDF points for estimated TDCP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 10% | 20% | 50% | 80% | 90% | 95% |
| 15 | 10 | 20 | 0.9476 | 0.9609 | 0.9734 | 0.9872 | 0.9936 | 0.9953 | 0.9964 |
|  | 100 | 10 | 0.5966 | 0.6747 | 0.7584 | 0.8746 | 0.9345 | 0.9522 | 0.9623 |
|  |  | 20 | 0.8652 | 0.9079 | 0.9435 | 0.9772 | 0.9897 | 0.9928 | 0.9946 |
|  | 200 | 10 | 0.4681 | 0.5699 | 0.6868 | 0.8421 | 0.9207 | 0.9425 | 0.9554 |
|  |  | 20 | 0.6511 | 0.7629 | 0.8602 | 0.9487 | 0.9797 | 0.9864 | 0.9903 |
|  | 300 | 10 | 0.338 | 0.4509 | 0.5903 | 0.7957 | 0.8998 | 0.9273 | 0.9444 |
|  |  | 20 | 0.4482 | 0.5948 | 0.737 | 0.9034 | 0.9631 | 0.9766 | 0.9835 |
| 30 | 10 | 20 | 0.9477 | 0.9609 | 0.9737 | 0.9877 | 0.9939 | 0.9956 | 0.9966 |
|  | 100 | 10 | 0.6291 | 0.7019 | 0.7811 | 0.8873 | 0.9411 | 0.9569 | 0.9663 |
|  |  | 20 | 0.9271 | 0.9474 | 0.966 | 0.9849 | 0.9929 | 0.995 | 0.9961 |
|  | 200 | 10 | 0.5941 | 0.6742 | 0.7605 | 0.8781 | 0.9375 | 0.9544 | 0.9648 |
|  |  | 20 | 0.8659 | 0.9095 | 0.9442 | 0.9778 | 0.9902 | 0.9933 | 0.9949 |
|  | 300 | 10 | 0.5386 | 0.6303 | 0.7291 | 0.8645 | 0.9317 | 0.9507 | 0.9617 |
|  |  | 20 | 0.7684 | 0.8461 | 0.9075 | 0.9661 | 0.9862 | 0.9908 | 0.9931 |

The results for 20% and 80% percentiles, follow a similar trend as the results presented in previous meetings for 10% and 90% where we deduced that 300 Hz is a better option for the TDCP verification with high doppler and that it is better to use 10 Hz Doppler with 20 dB SNR for the test case with low Doppler.

Furthermore, the agreement on using 80% pass rate gives an additional implementation margin, without allowing overlapping rates for low and high Doppler test cases.

- References

- R4-2405808, TDCP RRM Requirements, Nokia, RAN4# #110bis, Changsha, China, April 15 – April 19, 2024.

- R4-2409141, TDCP RRM Requirements, Nokia, RAN4# #111, Fukuoka, Japan, May 20 – May 24, 2024.

- Appendix A: Simulation Parameters

Table 4: Simulation parameters.

| Parameter | Value |
| --- | --- |
| Delay (between TRS symbols) | 1 slot |
| Channel model | TDL-A, delay spread=30ns |
| Doppler Spread | 10 Hz, 100 Hz, 200 Hz, 300 Hz |
| SNR | 10,20 |
| Number of averaging samples: | one shot |
| Channel BW | 10MHz |
| SCS | 15 kHz and 30 kHz |
| Reference Channel estimation | LS base CE for TRS as baseline |
| Correlation matrix and antenna configuration | 1x2 Low |
| Carrier Frequency | 3.5 GHz |