3GPP TSG-RAN WG4 Meeting # 112	R4-2412038

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for Fast SCell activation for UE supporting R18 EMR

Agenda item:	8.15.3

Document for:	Discussion

Introduction

In this contribution, we provide our views on fast SCell activation for UE supporting R18 EMR item. The objective of fast SCell activation for UE supporting Rel-18 EMR is captured below:

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1

Discussion

Reducing the SCell activation delay is very helpful to increase the UE throughput [3]. And the UE supporting Rel-18 EMR have benefit to reduce the SCell activation delay by getting the SCell information earlier. Here is the procedure of SCell activation in normal UE which does not support the Rel-18 EMR.

Figure 1. Scell activation procedure

If the UE support the Rel-18 EMR, so the UE has valid L3 measurement result before receiving the SCell activation command, then the green box part in Figure 1 can be omitted. For more detail requirements and signaling, RAN4 need to discuss.

Proposal 1: RAN4 to discuss how to get valid EMR measurement results for fast Scell activation.

To get the valid EMR for fast SCell activation, the EMR measurement have to be continued after T331 expired also. But in many cases, the UE might stay in IDLE/INACTIVE mode for long time, so in that case the UE does not necessary the EMR results. But the UE does not know whether the UE would transit to RRC_CONNECTED mode soon. For fast SCell activation with valid EMR reporting, since the EMR measurement have to be continued after T331 expired even though the UE stay in IDLE/INACTIVE mode for long time, the EMR measurement relaxation should be considered to reduce the measurement burden.

Proposal 2: For fast SCell activation with valid EMR reporting, the continuous EMR measurements are necessary after T331 is expired, and the EMR measurement relaxation should be considered to reduce the measurement burden.

Conclusion

In this contribution, we provide our views on fast SCell activation for UE supporting R18 EMR item.

Proposal 1: RAN4 to discuss how to get valid EMR measurement results for fast Scell activation.

Proposal 2: For fast SCell activation with valid EMR reporting, the continuous EMR measurements are necessary after T331 is expired, and the EMR measurement relaxation should be considered to reduce the measurement burden.

Reference

[1] R4-2406392, “WF for [110bis][230] NR_RRM_Ph5,” Moderator (Apple).

[2] RP-233209, “Views on Candidate RRM and Demodulation Topics for Rel-19”, Huawei

[3] RP-233460, “RAN4-led topics in Rel-19: RRM & Demodulation”, Nokia