3GPP TSG-RAN WG4 Meeting # 112	R4-2412039

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for NR duplex operation

Agenda item:	8.19.3

Document for:	Discussion

Introduction

In the last RAN plenary, revised WID[1] for evolution of NR duplex operation was approved. The RRM requirement related objectives are captured below:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. … - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Notes: Following are assumed for UL resource muting … - Check impact on transmit signal quality/MPR requirement, if any - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting … - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

In this contribution, we provide our high-level views on the UL resource muting for PUSCH and L1 based UE-to-UE CLI measurement and reporting.

Discussion

According to the WID[1], some features are introduced to support CLI handling. Among them 2 features are considered to be related to RAN4 work. First feature is UL resource muting for PUSCH. UE can measure CLI at muted RE. Currently RAN1 assuming comb-2 muting and up to 2 OFDM symbols in time domain. For keeping the time domain power level, the transmit power of PUSCH REs in the same OFDM symbols are considered 3dB power boosting.

RF part need to check the impact on transmit signal quality/MPR requirement. But RRM perspective, there is no spec. impact.

Proposal 1: No additional RRM requirements would be required for UL resource muting for PUSCH feature.

Regarding L1 based UE-to-UE CLI measurement and reporting, there could be various aggressor and victim scenarios.

And the RRM requirements might be depends on the scenario. So, RAN4 need to discuss the aggressor and victim scenario first. In the TR38.858[2], inter-UE CLI handling schemes considering UL sub-band UE as aggressor and DL sub-bands UE as victim in same DL BWP scenario.

Figure 1. UL sub-band UE as aggressor and DL sub-bands UE as victim in same DL BWP scenario

RAN4 can discuss RRM requirement from the UL sub-band UE as aggressor and DL sub-band UE as victim in same DL BWP scenario like figure.1. From the scenario, the victim UE should measure CLI-RSSI and SRS-RSRP within DL-sub-band. But the victim UE can’t measure SRS-RSRP of the aggressor UE at DL sub-band because the orthogonality is broken. So, RAN4 need to discuss how to measure the UL CLI leakage by UL sub-band aggressor UE at DL sub-band.

Proposal 2: RAN4 need to discuss how to measure the UL CLI leakage by UL sub-band aggressor UE at DL sub-band at the UL sub-band UE as aggressor and DL sub-band UE as victim in same DL BWP scenario.

To get the UL CLI leakage for DL sub-band, RAN1 is discussing how to use the SRS-RSRP and CLI-RSSI results from UL sub-band. So, RAN4 need to consider SRS-RSRP and CLI-RSSI measurement within UL sub-band, and Rel-16 CLI principle can be applied as baseline.

Proposal 3: RAN4 need to consider SRS-RSRP and CLI-RSSI measurement within UL sub-band, and Rel-16 CLI principle can be applied as baseline.

Summary

Proposal 1: No additional RRM requirements would be required for UL resource muting for PUSCH feature.

Proposal 2: RAN4 need to discuss how to measure the UL CLI leakage by UL sub-band aggressor UE at DL sub-band at the UL sub-band UE as aggressor and DL sub-band UE as victim in same DL BWP scenario.

Proposal 3: RAN4 need to consider SRS-RSRP and CLI-RSSI measurement within UL sub-band, and Rel-16 CLI principle can be applied as baseline.

Reference

[1] RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei

[2] TR38.858 v18.1.0, “Study on Evolution of NR Duplex Operation (Release 18)”, 3GPP