3GPP TSG-RAN WG4 Meeting # 112	R4-2412041

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for LP-WUS/WUR

Agenda item:	8.22.4.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues of RRM requirements for NR LP-WUS/WUR based on listed issues in approved WF [1].

Discussion

General aspects

Issue 1-1-2: Core requirements to be specified for RRM relaxation

In the last meeting, RAN4 agreed as below:

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

For the case #2~#4, depending on the signal quality os serving cell based on MR or LR, RRM measurement relaxation could be considered. If the signal quality is high enough, then fully offloading case can be considered as #1. if LR measurement is performed in center or middle of cell coverage, MR measurement for serving and/or neighbour cell could be relaxed comparing legacy measurement requirements as #2 and #3. The UE is in cell edge area, then the UE should perform normal measurement for neigbhor cells as legacy case (#5) because the mobility is more important than power saving. However, for case#4, it is unclear under what conditions neighboring cells are measured without measuring the serving cell based on MR.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #5 Legacy case | On | On | OFF |

From the below figure, UE#1 is expected to be “Fully offloading case” and UE#2 can be “case#2 or #3” and UE#3 should be in “Legacy case”.

Proposal 1: According to the channel quality the three cases are can be necessary.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | On |
| #2 Relaxed case a | On with relaxation measurement | Off | On |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | On |
| #5 Legacy case | On | On | Off |

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation

Based on companies’ view in the last meeting, MR RRM measurement relaxation can be considered for serving cell and/or neighbour cells. And the MR RRM measurement relaxation for serving cell and/or neigbhor cells should be considered when LP-WUR measurement is performed..

Proposal 2: Further MR based RRM measurement relaxation for serving cell and/or neighbour cells should be applied when LP-WUR measurement is performed.

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR measurement

RAN2 had agreements for entry/exit condition for LP-WUS monitoring as baseline.

Ageement in RAN2#126 - Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

Based on the agreements, RAN4 needs to discuss whether the same entry/exist condition would be applied to LP-WUR measurement. For LP-WUR measurement condition, UE mobility condition should be also considered since cell reselection based on MR measurement might be more important than power saving for high mobility.

Proposal 3: entry/exit conditions for LP-WUR measurement can be based on entry/exit condition for LP-WUS monitoring agreed in RAN2, and additionally UE mobility condition should be considered.

Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation

The discussion and decision for the criteria of entry/exit conditions for MR RRM measurement relaxation would be RAN2 work. However, RAN4 can provide input to RAN2 such as possible serving/neighbour measurement relaxation conditions or which receiver (i.e., MR or LP-WUR) to base it on for MR based RRM measurement relaxation depending on the decision for scenario in Issue 1-1-2.

Proposal 4: RAN4 to discuss and provide input to RAN2 such as possible serving/neighbour measurement relaxation conditions or which receiver (i.e., MR or LP-WUR) to base it on for MR based RRM measurement relaxation depending on the decision for MR measurement relaxation scenarios.

Issue 1-1-7: Entry/exit condition for LP-WUS monitoring

As mentioned above in Issue 1-1-5, basic entry/exit condition for LP-WUS monitoring is at least based on the serving cell measurement using LR and/or MR according to RAN2 agreements. However, RAN4 needs to consider overall entry/exit condition for LP-WUR measurement, MR measurement relaxation, and LP-WUS monitoring jointly to clear LP-WUR behavior. If there are other conditions that should be considered, RAN4 needs to provide input to RAN2.

Proposal 5: RAN4 needs to consider overall entry/exit condition for LP-WUR measurement, MR measurement relaxation, and LP-WUS monitoring jointly based on RAN2 agreements of entry/exit condition for LP-WUS monitoring. And if there are other conditions that should be considered, RAN4 needs to provide input to RAN2.

LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1: Methodology on specifying LP-WUR RRM requirements at Idle/Inactive mode

RAN1 is still discussing LP-SS design such as sequences and PRB size depending on SCS, so RAN4 needs to wait for further conclusion from RAN1 discussion. RAN4 agreed to define both LP-SS and existing PSS/SSS based measurement requirements, and since SNR/SINR target based on outcome of RAN1’s study is used as the starting point for RAN4 evaluation as below, RAN4 can discuss and evaluate exsiting PSS/SSS based LP-WUR measurement requirements.

Agreement on the target SNR for RAN4 simulation: - The outcome of RAN1’s study in Rel-19 WI on SNR/SINR target is used as the starting point for RAN4 LP-WUR requirement study. - Aspects not considered in RAN1 (if any) can be further discussed in RAN4.

Proposal 6: RAN4 needs to wait further conclusion from RAN1.

Issue 1-2-3: Accuracy for LP-WUR measurement

In the RAN4#110bis meeting, RAN4 discussed whther/how to define the accuracy requirements for LP-WUR in Idle/Inactive state and some agreements and information were made below.

RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: - RAN4 to discuss the simulation assumptions considering the target SNR, accuracy and number of samples. - RAN4 will define the delay requirement in core part, and further discuss whether/how to define the accuracy requirement. For information only: RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: Options for further discussion (the options are only for information purpose) - Option 1): Define the dedicated accuracy requirement in the performance section, FFS whether/how it can be verified in the test case. - Option 2): No dedicated accuracy requirement in the performance section, and reflect the accuracy performance as a margin in the core requirement. Option 2a): FFS accuracy requirement, and reflect the accuracy performance in delay requirement in core part. - Other options are not precluded.

Basically, in Idle/Inactive state, there is no measurement reporting, so core requirement for Idle/Inactive state is measurement delay based on the number of samples to meet measurement accuracy at certain side condition. Therefore, no dedicated accuracy requirement is needed.

Proposal 7: Do not define measurement accuracy requirements for LP-WUR in Idle/Inactive state in performance section.

MR RRM relaxation

MR RRM relaxation factor for serving cell/neighbour cell

As mentioned above section 2.1, MR based measurement relaxation for serving cell and/or nighbor cells should be considered for UE supporting LP-WUR. Detailed criterion for measurement relaxation would be further discussed based on the agreed measurement relaxation scenarios, and then MR RRM relaxation factor for serving and/or neigbhor cell.

Proposal 8: RAN4 to further discuss MR RRM relaxation factor for serving and/or neigbhor cell after scenario and detailed criterion for measurement relaxation have been clarified.

LP-WUR CONNECTED mode

LP-WUR at CONNECTED mode

Based on the objectives for LP-WUS/WUR, RAN4 needs to clarify the overall scope of RRM core requirements. LP-WUS design commonly applicable to both IDLE/INCATIVE and CONNECTED modes, but RAN4 needs to only focus on IDLE/INACTIVE mode related core requirements.

Proposal 9: Focus on RRM core requirements only for IDLE/INACTIVE mode in Rel-19.

Conclusion

In this contribution, we provide views on issues listed in WF [1], and we propose

Proposal 1: According to the channel quality the three cases are can be necessary.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | On |
| #2 Relaxed case a | On with relaxation measurement | Off | On |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | On |
| #5 Legacy case | On | On | Off |

Proposal 2: Further MR based RRM measurement relaxation for serving cell and/or neighbour cells should be applied when LP-WUR measurement is performed.

Proposal 3: entry/exit conditions for LP-WUR measurement can be based on entry/exit condition for LP-WUS monitoring agreed in RAN2, and additionally UE mobility condition should be considered.

Proposal 4: RAN4 to discuss and provide input to RAN2 such as possible serving/neighbour measurement relaxation conditions or which receiver (i.e., MR or LP-WUR) to base it on for MR based RRM measurement relaxation depending on the decision for MR measurement relaxation scenarios.

Proposal 5: RAN4 needs to consider overall entry/exit condition for LP-WUR measurement, MR measurement relaxation, and LP-WUS monitoring jointly based on RAN2 agreements of entry/exit condition for LP-WUS monitoring. And if there are other conditions that should be considered, RAN4 needs to provide input to RAN2.

Proposal 6: RAN4 needs to wait further conclusion from RAN1.

Proposal 7: Do not define measurement accuracy requirements for LP-WUR in Idle/Inactive state in performance section.

Proposal 8: RAN4 to further discuss MR RRM relaxation factor for serving and/or neigbhor cell after scenario and detailed criterion for measurement relaxation have been clarified.

Proposal 9: Focus on RRM core requirements only for IDLE/INACTIVE mode in Rel-19.

Reference

[1] R4-2410296, “WF for RRM requirements for LP-WUS/WUR,” Moderator (vivo)