3GPP TSG-RAN4 Meeting #112	R4-2412046

# Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0107 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on log formula for FR2-NTN UE RF requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Log formula is ambiguous. log(x) needs to be changed with log10(x). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix typo, - log(x) log10(x). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | log formula is remained ambiguously. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2, 9.5.3.2.3, 9.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

9.2.2	Off-axis EIRP emission density limit within the operating band

9.2.2.1	General

The Off-axis EIRP density envelope is applicable within the band to NTN VSAT transmitting towards geostationary satellite orbit.

9.2.2.2	Minimum requirement for bands n510 and n511

For co-polarized transmissions in the plane tangent to the GSO arc, the requirements specified in Table 9.2.2.2-1 apply to NTN VSAT.

Table 9.2.2.2-1: Off-axis EIRP density limits for co-polarized transmissions in the plane tangent to the GSO arc

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 62.5 – 25log10(θ) | 1 |
| 7° ≤ θ ≤ 9.2° | 41.5 | 1 |
| 9.2° ≤ θ ≤ 19.1° | 65.5 – 25log10(θ) | 1 |
| 19.1° < θ ≤ 180° | 33.5 | 1 |

For co-polarized transmissions in the plane perpendicular to the GSO arc, the requirements specified in Table 9.2.2.2-2 apply to NTN VSAT.

Table 9.2.2.2-2: Off-axis EIRP density limits for co-polarized transmissions in the plane perpendicular to the GSO arc

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 3.5° ≤ θ ≤ 7° | 65.5 – 25log10(θ) | 1 |
| 7° ≤ θ ≤ 9.2° | 44.5 | 1 |
| 9.2° ≤ θ ≤ 19.1° | 68.5 – 25log10(θ) | 1 |
| 19.1° < θ ≤ 180° | 36.5 | 1 |

The EIRP density levels specified in Table 9.2.2.2-1 and Table 9.2.2.2-2 may be exceeded by up to 3 dB, for values of θ > 7°, over 10% of the range of theta (θ) angles from 7–180° on each side of the line from the NTN VSAT to the target SAN.

For cross-polarized transmissions in the plane tangent to the GSO arc and in the plane perpendicular to the GSO arc, the requirements specified in Table 9.2.2.2-3 apply to NTN VSAT.

Table 9.2.2.2-2: Off-axis EIRP density limits for cross-polarized transmissions in the plane tangent to the GSO arc and in the plane perpendicular to the GSO arc

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 52.5 – 25log10(θ) | 1 |

9.2.2.3	Minimum requirement for band n512

9.2.2.3.1	Fixed VSAT

For co-polarized transmissions, the requirements specified in Table 9.2.2.3.1-1 apply to Fixed VSAT type 1 or 2 when transmitting towards satellites in geostationary orbit.

able 9.2.2.3.1-1: Off-axis EIRP density limits for co-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.8° ≤ θ ≤ 7° | 49 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 28 – K | 40 |
| 9.2° ≤ θ ≤ 48° | 52 – 25log10(θ) – K | 40 |
| 48° < θ | 20 – K | 40 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Fixed VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.4.2 in [18]. The manufacturer shall declare the value of N. |  |  |

For cross-polarized transmissions, the requirements specified in Table 9.2.2.3.1-2 apply to Fixed VSAT type 1 or 2 when transmitting towards satellites in geostationary orbit.

Table 9.2.2.3.1-2: Off-axis EIRP density limits for cross-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.8° ≤ θ ≤ 7° | 39 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 18 – K | 40 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Fixed VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.4.2 in [18]. The manufacturer shall declare the value of N. |  |  |

9.2.2.3.2	Mobile VSAT

For co-polarized transmissions, the requirements specified in Table 9.2.2.3.2-1 apply to Mobile VSAT.

Table 9.2.2.3.2-1: Off-axis EIRP density limits for co-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 49 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 28 – K | 40 |
| 9.2° ≤ θ ≤ 48° | 52 – 25log10(θ) – K | 40 |
| 48° < θ ≤ 180° | 20 – K | 40 |
| NOTE1: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17]. The manufacturer shall declare the value of N. NOTE2: The manufacturer shall declare the operational conditions of the system e.g. motion of the platform with 6 degrees of freedom and the duration for which the limits will not be exceeded for more than 0,01% of the time. |  |  |

Mobile VSAT with low elevation angles may exceed the limits specified in Table 9.2.2.3.2-1 by the amount specified in Table 9.2.2.3.2-2.

Table 9.2.2.3.2-2: Off-axis EIRP density limits for co-polarized transmissions

| Elevation angle to Satellite (ε) | Increase in EIRP density (dB) |
| --- | --- |
| ε < 5o | 2.5 |
| 5o < ε < 30o | 3 – 0.1 * ε |

For cross-polarized transmissions, the requirements specified in Table 9.2.2.3.2-3 apply to Mobile VSAT.

Table 9.2.2.3.2-3: Off-axis EIRP density limits for cross-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 39 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 18 – K | 40 |
| NOTE 1: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17]. The manufacturer shall declare the value of N. NOTE 2: The manufacturer shall declare the operational conditions of the system e.g. motion of the platform with 6 degrees of freedom and the duration for which the requirement will not be exceeded for more than 0,01% of the time. |  |  |

9.2.2.3.3	Additional Off-axis EIRP density requirements for protection of fixed services

For VSAT, the requirements specified in Table 9.2.2.3.3-1 apply.

Table 9.2.2.3.3-1: Off-axis EIRP density limits for protection of fixed services

| Frequency Range (GHz) | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 27.8285—28.4445 |  |  |
| 28.8365 – 28.9485 (NOTE1) | -5 | 1 |
| 28.9485 – 29.4525 |  |  |
| NOTE1: When applicable, if this frequency range is allocated to fixed service. |  |  |

< END OF CHANGE #1 >

< START OF CHANGE #2 >

9.5.3.2	On-axis spurious requirement

9.5.3.2.1	Applicability

The On-axis spurious requirement is applicable to NTN VSAT operating in band n512. The On-axis spurious emissions are measured as EIRP.

These limits are applicable to the complete NTN VSAT equipment, including cabling between the units.

9.5.3.2.2	“Emissions disabled” and “Carrier-off” states

The requirements specified in table 9.5.3.2.2-1 apply to NTN VSAT in “Emissions disabled” and “Carrier-off” states. They apply outside the transmission bandwidth.

Table 9.5.3.2.2-1: On-axis spurious limits in “Emissions disabled” and “Carrier-off” states

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 27.5 – 30.0 | 19 | 1 |

9.5.3.2.3	“Carrier-on” state

The requirements specified in Tables 9.5.3.2.3-1 and 9.5.3.2.3-2 apply to NTN VSAT in “Carrier-on”.

The requirement specified in Table 9.5.3.2.3-1 apply outside a bandwidth of 5 times the occupied bandwidth centred on the carrier centre frequency.

The requirement specified in Table 9.5.3.2.3-2 apply inside a bandwidth of 5 times the occupied bandwidth centred on the carrier centre frequency, and outside the transmission bandwidth.

NOTE:	The on-axis spurious radiations, outside the frequency range 27.5 GHz to 30.0 GHz, are indirectly limited by sub-clause 9.5.3.3.

Table 9.5.5.2.2.3-1: On-axis spurious limits in “Carrier-on” state - outside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 27.5 – 30.0 | 4, 5 | 44 - K (NOTE) | 1 |
|  | 1, 2, 3 | 4 - K (NOTE) | 1 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17] for Mobile VSAT or sub-clause 4.2.4.2 in [18] for Fixed VSAT. The manufacturer shall declare the value of N. |  |  |  |

Table 9.5.5.2.2.3-2: On-axis spurious limits in “Carrier-on” state - inside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 27.5 – 30.0 | 4, 5 | 58 - K (NOTE) | 1 |
|  | 1, 2, 3 | 48 - K (NOTE) | 1 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17] for Mobile VSAT or sub-clause 4.2.4.2 in [18] for Fixed VSAT. The manufacturer shall declare the value of N. |  |  |  |

< END OF CHANGE #2 >

< START OF CHANGE #3 >

9.6.2	Antenna performance

The following requirements are applicable to NTN VSAT type 1, type 2, type 4 or type 5 operating in band n511 or in band n510 and communicating with Geostationary Satellite Orbit (GSO) SAN.

The co-polarization gain in the plane tangent to the GSO arc shall not exceed the levels specified in Table 9.6.2-1. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2-1: Co-polarization gain limit in the plane tangent to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 2° ≤ θ ≤ 7° | 29 – 25log10(θ) |
| 7° ≤ θ ≤ 9.2° | 8 |
| 9.2° ≤ θ ≤ 19.1° | 32 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 0 |

The co-polarization gain in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2-2. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2-2: Co-polarization gain limit in the plane perpendicular to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 3.5° ≤ θ ≤ 7° | 32 – 25log10(θ) |
| 7° ≤ θ ≤ 9.2° | 10.9 |
| 9.2° ≤ θ ≤ 19.1° | 35 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 3 |

The off-axis cross-polarization gain in the plane tangent and in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2-3.

Table 9.6.2-3: Cross-polarization gain limit

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 2° ≤ θ ≤ 7° | 19 – 25log10(θ) |

< END OF CHANGE #3 >