3GPP TSG-RAN WG4 Meeting #111	R4-2412055
Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.12.2.2

Source:	vivo

Title:	Discussions on NTN OTA test method

Document for:	Approval

1	Introduction

This contribution provides our further views on NTN OTA test method and performance metric.

2	Discussion

2.1 UE type, usage scenarios and performance metric for NTN devices (NR-NTN and IoT-NTN)

Some agreements for NTN UE type in last RAN4 meeting [1], the prioritized UE type for IoT-NTN OTA discussion is FFS:

Issue 3-1-1: UE type for NR-NTN

Agreements

  - Prioritize Handheld UE and further study VSAT-like UE.

Issue 3-1-4: UE type for IoT-NTN

Agreements

  - FFS detailed UE type for IoT-NTN.

Issue 3-1-6: Usage scenarios for NR-NTN and IoT-NTN handheld UE

Agreements

  - RAN4 further discuss prioritized usage scenarios. Free space only is not considered.

    - Option 1: Prioritize the browsing mode (with hand phantom)

    - Option 2: Prioritize the browsing mode (with hand phantom) and Free Space

    - Option 3: Prioritize browsing mode (with hand phantom) and talk mode (Head+Hand)

    - Option 4: Prioritize Head+Hand talk mode, hand only browsing mode and talk mode (new positioning guideline), and Free Space

    - Option 5: Prioritize Free Space mode only

Currently, handheld IoT-NTN is widely discussed among companies including chipset vendors, OEMs and satellite operators, which can be considered as a potential popular UE type for future commercialization. Therefore, it is suggested that the handheld formfactor could be selected as 1st priority for IoT-NTN usage scenarios and performance metric discussion.

Proposal 1: For IoT-NTN test method development, RAN4 can consider handheld UE type as 1st priority.

For satellite communications, the typical smartphone usage scenario includes head+hand talk mode, hand only browsing and talk mode, free space mode.

Proposal 2: All the following usage scenarios for NTN handheld device should be considered:

- Head+hand talk mode, Handheld Hand only talk mode and browsing mode, Free space mode

The corresponding performance metric was discussed last meeting:

Issue 3-2-1: Categorized performance metric for NR-NTN handheld

Agreements

- RAN4 should not adopt direction 1, other directions are FFS

  - Direction 1: Only single point performance metric, i.e., Peak EIRP/EIS

  - Direction 2: Single point + measured full sphere (integrated or CDF), e.g.,

    - D2a, Peak EIRP/EIS+TRP/TRS,

    - D2b, Peak EIRP/EIS+ full Spherical coverage CDF

  - Direction 3: Single point + measured partial sphere (integrated or CDF), e.g.,

    - D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles),

    - D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles)

  - Direction 4: Only Full sphere (integrated TRP/TRS or Spherical coverage CDF) with/without weighting,

    - D4a, Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile

    - D4b, TRP/TRS or Weighted metric, e.g., weighted TRP/TRS (with larger weight for declared hemisphere)

  - Direction 5: Only Partial sphere (integrated or CDF) (e.g., within selected 30, 60, 90, 180 degree range of angles),

Given NTN communication is more-like a directional case, which is different from general smartphone talk mode usage, directional performance of NTN UE would play a more important role for NTN UE performance.

Proposal 3: Consider direction 3 as 1st priority and further discuss details.

2.2 Test method and parameters for NR-NTN and IoT-NTN devices

For NR NTN test parameters, RAN4 has considered the spectrum allocation status which might be fragmented in different satellite operators .

Issue 3-3-2: CBW for NR-NTN bands

Agreements

  - Due to limited spectrum resource for NTN, RAN4 further consider whether min test channel BW defined in RAN5 38.508-1 should be adopted for FR1 NTN OTA testing.

If min test channel BW is selected then 5MHz should be selected for FR1-NTN based on min CBW in TS 38.508-1.

Table 4.3.1.0B-3 in TS 38.508-1: Low Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE Low Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 5 |
| n255 | 5 |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-5 [57] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-5 [56] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. |  |

Proposal 4: Select 5MHz CBW for NR FR1 NTN OTA testing.

The corresponding detailed test parameters for these two bands can be generated based on the parameters specified in TS 38.521-5, as following:

Table 1: NR FR1 NR-NTN TRP measurement parameters

| NR Band | CBW [MHz] | SCS (kHz) | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | UL RB Allocation (LCRB @ RBstart) | DL configuration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 5 | 15 | DFT-s-OFDM QPSK | Low | 396500 | 1982.5 | 434500 | 2172.5 | 12@6 | N/A |
|  |  |  |  | Mid | 399000 | 1995 | 437000 | 2185 |  |  |
|  |  |  |  | High | 401500 | 2007.5 | 439500 | 2197.5 |  |  |
| n255 | 5 | 15 | DFT-s-OFDM QPSK | Low | 325800 | 1629 | 305500 | 1527.5 | 12@6 | N/A |
|  |  |  |  | Mid | 328700 | 1643.5 | 308400 | 1542 |  |  |
|  |  |  |  | High | 331600 | 1658 | 311300 | 1556.5 |  |  |

Table 2: NR FR1 NR-NTN TRS measurement parameters

| NR Band | CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | UL RB Allocation (LCRB @ RBstart) | DL Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 5 | 15 | CP-OFDM QPSK | DFT-s-OFDM QPSK | Low | 396500 | 1982.5 | 434500 | 2172.5 | 25@0 | 25@0 |
|  |  |  |  |  | Mid | 399000 | 1995 | 437000 | 2185 |  |  |
|  |  |  |  |  | High | 401500 | 2007.5 | 439500 | 2197.5 |  |  |
| n255 | 5 | 15 | CP-OFDM QPSK | DFT-s-OFDM QPSK | Low | 325800 | 1629 | 305500 | 1527.5 | 25@0 | 25@0 |
|  |  |  |  |  | Mid | 328700 | 1643.5 | 308400 | 1542 |  |  |
|  |  |  |  |  | High | 331600 | 1658 | 311300 | 1556.5 |  |  |

Proposal 5: Adopt the above Table 1 and Table 2 test parameters for NR-NTN OTA testing.

For NB-IoT NTN OTA testing, the detailed test parameters should also be defined. The following parameters are conducted test case in TS 36.521-4.

Table 6.2B.1.4.1-1: Test Configuration Initial Conditions for FDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 2) |  | BPSK | 1@0 | 3.75 |
| 2 (Note 3) |  | BPSK | 1@47 | 3.75 |
| 3 (Note 2) |  | QPSK | 1@0 | 15 |
| 4 (Note 3) |  | QPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 3@3 | 15 |
| NOTE 1: Applicable to UE supporting UL multi-tone transmissions. NOTE 2: only applicable for low range. NOTE 3: only applicable for high range. |  |  |  |  |

RAN4 should further discuss which set of parameters is typical to verify IoT-NTN OTA performance.

Proposal 6: RAN4 should further discuss and decide which set of parameters is a typical case to verify IoT-NTN OTA performance, e.g. QPSK 1 tone with 15kHz SCS.

3 Conclusion

In this paper, we provide our further views on NTN OTA.

Proposal 1: For IoT-NTN test method development, RAN4 can consider handheld UE type as 1st priority.

Proposal 2: All the following usage scenarios for NTN handheld device should be considered:

- Head+hand talk mode, Handheld Hand only talk mode and browsing mode, Free space mode

Proposal 3: Consider direction 3 as 1st priority and further discuss details.

Proposal 4: Select 5MHz CBW for NR FR1 NTN OTA testing.

Proposal 5: Adopt the above Table 1 and Table 2 test parameters for NR-NTN OTA testing.

Proposal 6: RAN4 should further discuss and decide which set of parameters is a typical case to verify IoT-NTN OTA performance, e.g. QPSK 1 tone with 15kHz SCS.

4	References

- R4-2409931, WF for [111][338] TRP_TRS_MIMO_OTA, vivo, RAN meeting #111, May 2024.

- TS 38.508-1

- TS 36.521-4

- TS 38.521-5