3GPP TSG-RAN WG4 Meeting #112	R4-2412058

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	 8.22.2.1

Source:	 vivo

Title:	Discussions on LP-WUS system parameters

Document for:	 Approval

- Introduction

This contribution shares our views on LP-WUS system parameters.

- LP-WUS UE RF

2.1 Channel raster

Issue 1-1-7: Channel raster for LP-WUR

Agreements:

RAN4 further check whether no channel raster is needed for LP-WUR.

It was agreed that RAN4 will further discuss LP-WUS channel raster [1].

In our understanding the WUS signal is modulated by in-band NR waveform, the channel raster of the whole channel including LP-WUS and NR should follow the defined channel raster in TS 38.101-1.

Issue 1-1-11: Metric for LP_WUR requirements

Agreements:

  - MDR is used as the metric for LP_WUR Rx RF requirements

    - FFS MDR values.

  - FAR will be defined as demodulation requirements

    - FFS FAR values.

Proposal 1: For in-band WUS operation, the channel raster of LP-WUS can be the same as NR.

Last RAN4 meeting, the MDR was selected as single metric for LP-WUS RF requirements, but the percentage value is FFS. In the agreed simulation assumption, 1% is the baseline and 5% is optional.

Based on our simulation results in [2], and the intention of alignment with RAN1 coverage link budget evaluations, the MDR of RAN4 LP-WUS RF requirements is suggested to be 1%.

Proposal 2: RAN4 select 1% MDR as performance metric for LP-WUS RF requirements.

- Conclusion

In this contribution, we share our views on LP-WUR system parameters:

Proposal 1: For in-band WUS operation, the channel raster of LP-WUS can be the same as NR.

Proposal 2: RAN4 select 1% MDR as performance metric for LP-WUS RF requirements.

# References

- R4-2406619, WF on NR LP-WUS UE requirements, vivo, RAN4#110bis

- R4-2412059, Discussions on LP-WUS REFSENS, ASCS and ACS, vivo, RAN4#112