3GPP TSG-RAN WG4 Meeting #112	R4-2412059

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	 8.22.2.2

Source:	 vivo

Title:	Discussions on LP-WUS REFSENS, ASCS and ACS

Document for:	 Approval

- Introduction

This contribution shares our simulation results for LP-WUS REFSENS, ASCS and ACS requirements.

- LP-WUS UE RF

  - Link-level simulation for LP-WUS target SNR

In LTE/NR, SNR = -1 dB is widely used to calculate REFSENS and this value is derived from 95% throughput at link-level simulation under AWGN, 1/3QPSK, no HARQ, 12,5 symbol per subframe as shown in [1][2].

Figure 1 Target LTE SNR simulation with 5, 10 and 20 MHz bandwidths (throughput in Mbps); one receiving antenna [1].

Figure 2 Target NR SNR simulation with 5, and 10 MHz bandwidths [2].

The link-level simulation assumption for LP-WUS was agreed in WF [3], similar method as LTE and NR can be used to derive target SNR value for LP-WUS RF, the following Table 1 shows our simulation results.

Table 1 SNR@1% MDR and @5%MDR with 4bit ADC under AWGN (no repetition)

| Modulation | Chip rate | Channel structure | Receiver type | SNR@5% MDR | SNR@1% MDR |
| --- | --- | --- | --- | --- | --- |
| OOK-1 | M = 1 | 8bit | Envelope | -3.9 dB | -3.0 dB |
|  |  |  | OFDM | -10.4 dB | -9.3 dB |
|  |  | 16bit | Envelope | -3.6 dB | -2.9 dB |
|  |  |  | OFDM | -10.2 dB | -9.2 dB |
| OOK-4 | M = 1 | 8bit | Envelope | -4.0 dB | -3.1 dB |
|  |  | 16bit |  | -3.7 dB | -3.0 dB |
|  | M = 2 | 8bit |  | -5.1 dB | -4.2 dB |
|  |  | 16bit |  | -4.9 dB | -4.0 dB |
|  | M = 4 | 8bit |  | -3.4 dB | -2.5 dB |
|  |  | 16bit |  | -3.1 dB | -2.3 dB |
| OOK-4 | M = 1 | 8bit | OFDM | -10.4 dB | -9.3 dB |
|  |  | 16bit |  | -10.3 dB | -9.1 dB |
|  | M = 2 | 8bit |  | -10.5 dB | -9.5 dB |
|  |  | 16bit |  | -10.3 dB | -9.2 dB |
|  | M = 4 | 8bit |  | -7.6 dB | -6.5 dB |
|  |  | 16bit |  | -7.3 dB | -6.2 dB |

Observation 1: There is about 1dB difference between 1% MDR and 5%MDR.

Observation 2: The worst case is OOK-4, M=4, payload = 16bit with envelope receiver, and the SNR at 1%MDR is about -2.3 dB.

However, the TDL-C is also agreed in the assumption, it is a problem that which channel should be the baseline for deriving the SNR. We further compare the difference between AWGN and TDL, and the results are shown in Table 2.

Table 2 SNR comparison between AWGN and TDL-C 300 (no repetition)

| Modulation | Chip rate | Channel structure | Receiver type | AWGN SNR@5% MDR | TDL SNR@5% MDR | Difference | AWGN SNR@1% MDR | TDL SNR@1% MDR | Difference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OOK-1 | M = 1 | 8bit | Envelope | -3.9 | -0.3 | -3.6 | -3 | 1.6 | -4.6 |
|  |  |  | OFDM | -10.4 | -4.5 | -5.9 | -9.3 | -1.5 | -7.8 |
|  |  | 16bit | Envelope | -3.6 | 0 | -3.6 | -2.9 | 2 | -4.9 |
|  |  |  | OFDM | -10.2 | -4.9 | -5.3 | -9.2 | -2 | -7.2 |
| OOK-4 | M = 1 | 8bit | Envelope | -4 | -0.4 | -3.6 | -3.1 | 1.6 | -4.7 |
|  |  | 16bit |  | -3.7 | 0 | -3.7 | -3 | 1.8 | -4.8 |
|  | M = 2 | 8bit |  | -5.1 | -1.2 | -3.9 | -4.2 | 0.8 | -5 |
|  |  | 16bit |  | -4.9 | -0.8 | -4.1 | -4 | 1.1 | -5.1 |
|  | M = 4 | 8bit |  | -3.4 | 0.8 | -4.2 | -2.5 | 3 | -5.5 |
|  |  | 16bit |  | -3.1 | 1.2 | -4.3 | -2.3 | 3.4 | -5.7 |
| OOK-4 | M = 1 | 8bit | OFDM | -10.4 | -4.8 | -5.6 | -9.3 | -2.2 | -7.1 |
|  |  | 16bit |  | -10.3 | -4.5 | -5.8 | -9.1 | -1.8 | -7.3 |
|  | M = 2 | 8bit |  | -10.5 | -5 | -5.5 | -9.5 | -2.4 | -7.1 |
|  |  | 16bit |  | -10.3 | -4.5 | -5.8 | -9.2 | -2 | -7.2 |
|  | M = 4 | 8bit |  | -7.6 | -2.3 | -5.3 | -6.5 | 0.5 | -7 |
|  |  | 16bit |  | -7.3 | -1.8 | -5.5 | -6.2 | 0.9 | -7.1 |

Under the TDL channel, the LP-WUS require higher SNR to achieve same MDR performance, but we found that under same receiver type, the SNR difference between AWGN and TDL-C is nearly a fixed value. For example, under envelope receiver, the SNR difference is about 5 dB at 1% MDR, and about 7 dB for OFDM receiver.

Observation 3: For same receiver type, the performance between AWGN and TDL-C has a relatively stable offset. To specify RF requirements, there is no need to consider both AWGN and TDL-C channel models.

Proposal 1: From RF requirement verification perspective, using AWGN is sufficient.

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered:

      - SNR=-3dB for NF of LR = NF of MR+ 8dB

      - SNR= -0.5dB for NF of LR = NF of MR+ 5dB

      - SNR=2dB for NF of LR = NF of MR+ 2dB

      - Note 1: The NF of MR is assumed as 7dB

Another issue here is how to ensure the SNR derived from RAN4 method can meet the RAN1 msg3 coverage target. In RAN1#117, the following relationship between SNR and NF are agreed based on link budget analysis:

In RAN1 study, TDL-C with repetition is assumed, and in the following table we provide SNR results under similar assumption as RAN1, i.e., repetition 2 times for M=1/2 and repetition 4 times for M=4.

Table 3 SNR under TDL-C channel with repetition

| Modulation | Chip rate | Channel structure | Receiver type | SNR@5% MDR | SNR@1% MDR |
| --- | --- | --- | --- | --- | --- |
| OOK-1 | M = 1 | 8bit | Envelope | -2.1 dB | -0.2 dB |
|  |  |  | OFDM | -8.0 dB | -5.0 dB |
|  |  | 16bit | Envelope | -1.8 dB | 0.2 dB |
|  |  |  | OFDM | -7.5 dB | -5.0 dB |
| OOK-4 | M = 1 | 8bit | Envelope | -2.0 dB | -0.1 dB |
|  |  | 16bit |  | -1.8 dB | -0.1 dB |
|  | M = 2 | 8bit |  | -3.0 dB | -1.0 dB |
|  |  | 16bit |  | -2.6 dB | -0.6 dB |
|  | M = 4 | 8bit |  | -2.6 dB | -0.6 dB |
|  |  | 16bit |  | -2.2 dB | -0.2 dB |
| OOK-4 | M = 1 | 8bit | OFDM | -8.0 dB | -5.0 dB |
|  |  | 16bit |  | -7.5 dB | -4.6 dB |
|  | M = 2 | 8bit |  | -8.1 dB | -5.1 dB |
|  |  | 16bit |  | -7.4 dB | -4.8 dB |
|  | M = 4 | 8bit |  | -8.4 dB | -5.4 dB |
|  |  | 16bit |  | -7.8 dB | -5.0 dB |

Observation 4: Under similar assumption in RAN1, the estimated SNR with repetition by RAN4 LLS can meet RAN1 coverage target.

Observation 5: There is a stable SNR gap between “AWGN+no repetition” case and “TDL-C+repetition” case.

In summary, following similar approach of MR, the target SNR of WUR RF in RAN4 can be defined at 1% MDR based on AWGN model with no repetition. Although this RAN4 SNR is different from the SNR value analyzed by link budget, it is the understanding that the meg3 coverage can be met with repetitions and reasonable assumption of noise figure.

Proposal 2: Follow similar approach of MR, RAN4 define the target SNR for WUR Rx requirements at 1% MDR based on AWGN model with no repetition.

Proposal 3: Define a single target SNR based on worst case of LLS outcome, i.e., Envelope receiver, [-2]dB.

  - ASCS requirements

Using the agreed simulation parameters, we also perform ASCS link level simulations, which is summarized in Table 4.

Table 4 SNR results for ASCS (similar result with/without guard RBs inside)

| modulation | Chip rate | payload | receiver | Freq | Freq offset | Filter order | ASCS SNR@ 5% MDR | target SNR@5%MDR | ASCS SNR @ 1% MDR | target SNR@1%MDR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OOK-1 | M = 1 | 16bit | Envelope | 900 MHz | The results for different offset are similar | The results for different filter are similar | -3.8 dB | -3.9dB | -2.9 dB | -3.0 dB |
|  |  |  |  | 2.6 GHz |  |  |  |  |  |  |
|  |  |  |  | 3.5 GHz |  |  |  |  |  |  |
|  |  |  | OFDM | 900 MHz | 0 ppm | 3rd | -10.0dB | -10.0 dB | -9.2 dB | -9.2 dB |
|  |  |  |  |  |  | 5th | -10.2dB | -10.2 dB | -9.0 dB | -9.0 dB |
|  |  |  |  |  | 10 ppm | 3rd | -8.8 dB | -8.8 dB | -7.7 dB | -7.7 dB |
|  |  |  |  |  |  | 5th | -9.0 dB | -9.0 dB | -8.0 dB | -8.0 dB |
|  |  |  |  |  | 20 ppm | 3rd | -7.1 dB | -7.1 dB | -6.0 dB | -6.0 dB |
|  |  |  |  |  |  | 5th | -7.2 dB | -7.2 dB | -6.2 dB | -6.2 dB |
|  |  |  |  | 2.6 GHz | 0 ppm | 3rd | -10.0dB | -10.0dB | -9.2 dB | -9.2 dB |
|  |  |  |  |  |  | 5th | -10.2dB | -10.2dB | -9.0 dB | -9.0 dB |
|  |  |  |  |  | 10 ppm | 3rd | -8.3 dB | -8.3 dB | -7.5 dB | -7.5 dB |
|  |  |  |  |  |  | 5th | -8.5 dB | -8.5 dB | -7.7 dB | -7.7 dB |
|  |  |  |  |  | 20 ppm | 3rd | -5.7 dB | -5.7 dB | -5.0 dB | -5.0 dB |
|  |  |  |  |  |  | 5th | -6.1 dB | -6.1 dB | -5.1 dB | -5.1 dB |
|  |  |  |  | 3.5 GHz | 0 ppm | 3rd | -10.0dB | -10.0dB | -9.2 dB | -9.2 dB |
|  |  |  |  |  |  | 5th | -10.2dB | -10.2dB | -9.0 dB | -9.0 dB |
|  |  |  |  |  | 10 ppm | 3rd | -8.3 dB | -8.3 dB | -7.5 dB | -7.5 dB |
|  |  |  |  |  |  | 5th | -8.5 dB | -8.5 dB | -7.5 dB | -7.5 dB |
|  |  |  |  |  | 20 ppm | 3rd | -5.7 dB | -5.7 dB | -5.0 dB | -5.0 dB |
|  |  |  |  |  |  | 5th | -5.9 dB | -5.9 dB | -5.0 dB | -5.0 dB |
| OOK-4 | M = 4 | 16bit | Envelope | 900 MHz | The results for different offset are similar | The results for different filter are similar | -3.0 dB | -3.1dB | -2.3 dB | -2.3 dB |
|  |  |  |  | 2.6 GHz |  |  |  |  |  |  |
|  |  |  |  | 3.5 GHz |  |  |  |  |  |  |
|  |  |  | OFDM | 900 MHz | 0 ppm | 3rd | -7.3 dB | -7.3 dB | -6.2 dB | -6.2 dB |
|  |  |  |  |  |  | 5th |  |  |  |  |
|  |  |  |  |  | 10 ppm | 3rd | -7.2 dB | -7.2 dB | -6.1 dB | -6.1 dB |
|  |  |  |  |  |  | 5th |  |  |  |  |
|  |  |  |  |  | 20 ppm | 3rd | -7.1 dB | -7.1 dB | -5.9 dB | -5.9 dB |
|  |  |  |  |  |  | 5th |  |  |  |  |
|  |  |  |  | 2.6 GHz | 0 ppm | 3rd | -7.3 dB | -7.3 dB | -6.2 dB | -6.2 dB |
|  |  |  |  |  |  | 5th |  |  |  |  |
|  |  |  |  |  | 10 ppm | 3rd | -6.6 dB | -6.6 dB | -5.5 dB | -5.5 dB |
|  |  |  |  |  |  | 5th |  |  |  |  |
|  |  |  |  |  | 20 ppm | 3rd | -5.0 dB | -5.0 dB | -3.8 dB | -3.8 dB |
|  |  |  |  |  |  | 5th |  |  |  |  |
|  |  |  |  | 3.5 GHz | 0 ppm | 3rd | -7.3 dB | -7.3 dB | -6.2 dB | -6.2 dB |
|  |  |  |  |  |  | 5th |  |  |  |  |
|  |  |  |  |  | 10 ppm | 3rd | -6.2 dB | -6.2 dB | -5.1 dB | -5.1 dB |
|  |  |  |  |  |  | 5th |  |  |  |  |
|  |  |  |  |  | 20 ppm | 3rd | -3.2 dB | -3.2 dB | -2.2 dB | -2.2 dB |
|  |  |  |  |  |  | 5th |  |  |  |  |

In the Table above, we don’t differentiate the performance under different guard RB and the reason here is we observe that the benefit of guard RB is not significant for ASCS.

Observation 5: The benefit of guard RB in ASCS case is minor, and the impact of adjacent NR signal on the WUS sensitivity is negligible.

It is also observed that the frequency offset has no impact on Envelope receiver but significant impact on OFDM receiver, the reason is that the frequency offset impacts a lot the coherence detection of receiver rather than the interference power level.

Proposal 4: No ASCS requirements value is needed, RAN4 just specify the test parameters.

Proposal 5: RAN4 can define the ASCS test parameters as following: Achieving 1% MDR without guard RB under the power level of REFSENS+[0.5dB]. detailed test parameters in Table 5

Table 5: Test parameters for LP-WUS ASCS

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5-100MHz |
| LP-WUS Power | dBm | REFSENS + [0.5] dB |
| In-band Pinterferer | dBm | NR signal, same as LP-WUS |
| Guard RB between LP-WUS and NR |  | 0 |
| MDR |  | 1% |

  - ACS requirements

For ACS, we also perform the full sets of simulations based on the agreed simulation assumptions [3], the LP-WUS is working under a high SNR condition, i.e., 14dB+REFSENS.

We figure out that envelop receiver can tolerate less adjacent interference, the typical outcome of envelope receiver ACS is in Figure 3 and Figure 4.

Figure 3 OOK1 under different adjacent interfernece power level when LP-WUS SNR=REFSENS+14 dB

Figure 4 OOK4 M=4 under different adjacent interfernece power level when LP-WUS SNR=REFSENS+14 dB

Observation 6: For the ACS cases, the ADC plays a critical role and only when ADC bit is enough, the guard RB can help on the MDR performance.

Observation 7: To endure same interference power level, i.e., 31.5dB as MR, then 8bit ADC is mandatory with 3rd filter, in addition, 4 guard RBs are still needed.

To further dig out the reason behind ADC impacts, we compare the waveform after ADC under different interference power level in time domain, as shown in below:

Figure 5 Mixed Waveform in time domain under different interference power level

Since in the time domain, the NR waveform and OOK waveform superimposed on each other, when the interference power level increases, the proportion of the gap between ON and OFF in the time domain will become smaller, which means more quantized orders are needed to distinguish them.

We also compare the impact of different frequency offset, as shown in Figure 6.

0-20ppm

Figure 6 MDR performance under different frequency offset

Observation 8: The frequency offset has minor impact on ACS performance.

For the OFDM receiver, we also did the full set of ACS simulations, all the waveforms can pass MR ACS requirements. We just share the worst case of the waveform in the paper, i.e., OOK4 M=4 with 4bit ADC and 3rd filter, as below:

Figure 7 OFDM receiver performance under the worst case

Observation 9: No guard RB is required for OFDM receiver to ensure ACS performance.

With above analysis and observations, we believe the ACS requirements should consider worst case of the receiver. The dynamic range of LP-WUR is not that large as MR, to ensure the coverage of sensitivity then the capability of LP-WUR to tolerate high interference may be limited. RAN4 should consider a different ACS requirement for this new type of receiver.

Proposal 6: Define WUR ACS requirement as a value within the range [20~31.5]dB. FFS number of guard RBs.

  - REFSENS requirements

The following agreements for REFSENS have been agreed last meeting [3]:

Issue 2-2-4: NF Gap between LR and MR for REFSENS (assume MR as 9dB)

Agreement:

  - RAN4 further discuss potential NF for OOK-based and OFDM-based receiver.

  - Linked to resolution of Issue 2-2-8

Issue 2-2-8: Whether a baseline architecture is needed for LP-WUS receiver

Agreement:

  - RAN4 will further discuss and decide baseline architecture(s) for OOK-based receiver and OFDM-based receiver.

Regarding noise figure of LP-WUS receiver, it was agreed the group can consider two different set of parameters for OOK-based receivers and OFDM-based receivers. For each of them, no need to further evaluate specific values for each architecture or variants, considering there would be large amount of architecture variants even for OOK-based receivers (e.g., shared RF path, shared antenna, additional RF elements considered…). Therefore, specify a NF range for OOK and further narrow down to specific value could be a way to go in RAN4.

The following gap between LR and MR can be considered.

Proposal 7: The following NF can be discussed in RAN4 for RESENS:

For OOK based WUR

- [9~16] dB

For OFDM based WUR

- [9~14] dB

Proposal 8: For the baseline architecture of LP-WUR, the following two types can be considered: IF envelop receiver for OOK-based, and no FFT based OFDM receiver for OFDM receiver.

- Conclusion

In this contribution, we share our views on LP-WUR REFSENS, ASCS and ACS:

Observation 1: There is about 1dB difference between 1% MDR and 5%MDR.

Observation 2: The worst case is OOK-4, M=4, payload = 16bit with envelope receiver, and the SNR at 1%MDR is about -2.3 dB.

Observation 3: For same receiver type, the performance between AWGN and TDL-C has a relatively stable offset. To specify RF requirements, there is no need to consider both AWGN and TDL-C channel models.

Proposal 1: From RF requirement verification perspective, using AWGN is sufficient.

Observation 4: Under similar assumption in RAN1, the estimated SNR with repetition by RAN4 LLS can meet RAN1 coverage target.

Observation 5: There is a stable SNR gap between “AWGN+no repetition” case and “TDL-C+repetition” case.

Proposal 2: Follow similar approach of MR, RAN4 define the target SNR for WUR Rx requirements at 1% MDR based on AWGN model with no repetition.

Proposal 3: Define a single target SNR based on worst case of LLS outcome, i.e., Envelope receiver, [-2]dB.

Observation 5: The benefit of guard RB in ASCS case is minor, and the impact of adjacent NR signal on the WUS sensitivity is negligible.

Proposal 4: No ASCS requirements value is needed, RAN4 just specify the test parameters.

Proposal 5: RAN4 can define the ASCS test parameters as following: Achieving 1% MDR without guard RB under the power level of REFSENS+[0.5dB]. detailed test parameters in Table 5

Table 5: Test parameters for LP-WUS ASCS

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5-100MHz |
| LP-WUS Power | dBm | REFSENS + [0.5] dB |
| In-band Pinterferer | dBm | NR signal, same as LP-WUS |
| Guard RB between LP-WUS and NR |  | 0 |
| MDR |  | 1% |

Observation 6: For the ACS cases, the ADC plays a critical role and only when ADC bit is enough, the guard RB can help on the MDR performance.

Observation 7: To endure same interference power level, i.e., 31.5dB as MR, then 8bit ADC is mandatory with 3rd filter, in addition, 4 guard RBs are still needed.

Observation 8: The frequency offset has minor impact on ACS performance.

Observation 9: No guard RB is required for OFDM receiver to ensure ACS performance.

Proposal 6: Define WUR ACS requirement as a value within the range [20~31.5]dB. FFS number of guard RBs.

Proposal 7: The following NF can be discussed in RAN4 for RESENS:

For OOK based WUR

- [9~16] dB

For OFDM based WUR

- [9~14] dB

Proposal 8: For the baseline architecture of LP-WUR, the following two types can be considered: IF envelop receiver for OOK-based, and no FFT based OFDM receiver for OFDM receiver.

# References

- R4-080125, TP Receiver Requirements: REFSENS, ACS and blocking, Ericsson, RAN4#46

- R4-1709564, Preliminary SNR values for NR UE REFSENS, Huawei, Hisilicon, NR ad-hoc #3, 2017

- R4-2410569, WF for [111][136] NR_LPWUS_UERF, vivo, RAN4#111