3GPP TSG-RAN WG4 Meeting #112		R4-2412061

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	 8.22.2.4

Source:	 vivo

Title:	Discussions on LP-WUS Testability

Document for:	 Approval

- Introduction

This contribution shares further views on LP-WUS RF requirements testability issues.

- LP-WUS UE RF testability

For LP-WUS testing, one basic issue is how to achieve BLER/MDR counting, with MR wake up each time or not. If the MR is wake up each time based on normal procedure of LR to MR, then to meet the statistic of test results, the overall testing time will be unacceptable.

Therefore, before discussion detailed test issues RAN4 should align first the general guidance that the MDR can be counted without waking up the MR each time would be enough for the LP-WUS test in terms of verifying the RF requirements.

Proposal 1: RAN4 should agree the general guidance first that the MDR can be counted without waking up the MR each time for LP-WUS RF performance testing.

The testability issue of LP-WUS BLER/MDR should be well resolved otherwise the requirements can not be properly verified, according to the LP-WUR mode, there could be two general directions to go, e.g., MR idle mode and MR connection mode.

For MR connection mode, the testability issue would be easy to resolve, the tested wake-up signal “ACK/NACK” results can be reported to gNB via MR, then the BLER/MDR can be calculated accordingly. It should be noted that MR should not receive and detect LP-WUS signal during testing, otherwise there might be a risk that the UE could cheat gNB by using MR detection results.

However, for MR idle mode, the REFSENS will be verified under a no close-loop condition, a new UE test mode is needed. Besides, each detection results as “ACK/NACK” in LR should be recorded and then MR can be waken up after LP-WUR testing period finished, then the detection results can be feedback to gNB via MR.

Proposal 2: the following two options can be considered for LP-WUS general procedure:

- Option1: Using MR connection mode, but MR should not receive and detect LP-WUS signal. The miss detection ratio can be calculated based on “ACK/NACK” results of LP-WUR which is feedback to gNB by MR.

- Option2: Using MR idle mode, there is no LP-WUS feedback to gNB during testing, a new UE test mode is needed. MR should be waken-up after testing of all LP-WUS signal and connected to gNB to feedback the LP-WUS detection results.

Proposal 3: Details test parameters can be left to RAN5, however the general procedure and metric should be defined in RAN4 first.

- Conclusion

In this contribution, we share our views on LP-WUR testability issue.

Proposal 1: RAN4 should agree the general guidance first that the MDR can be counted without waking up the MR each time for LP-WUS RF performance testing.

Proposal 2: the following two options can be considered for LP-WUS general procedure:

- Option1: Using MR connection mode, but MR should not receive and detect LP-WUS signal. The miss detection ratio can be calculated based on “ACK/NACK” results of LP-WUR which is feedback to gNB by MR.

- Option2: Using MR idle mode, there is no LP-WUS feedback to gNB during testing, a new UE test mode is needed. MR should be waken-up after testing of all LP-WUS signal and connected to gNB to feedback the LP-WUS detection results.

Proposal 3: Details test parameters can be left to RAN5, however the general procedure and metric should be defined in RAN4 first.

# References

- R4-2406619, WF on NR LP-WUS UE requirements, vivo, RAN4#110bis