3GPP TSG-RAN WG4 Meeting #111		R4-2412062

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	 10.14.3

Source:	 vivo

Title:	Discussions on LP-WUS BS RF requirements

Document for:	 Approval

- Introduction

This contribution shares our views on LP-WUS BS RF requirements.

- LP-WUS BS RF requirements

Regarding manufacture declaration on LP-WUS, there are two options in WF [1].

## 1.1 Manufacture declaration on LP-WUS

WF

FFS on the following options for next meeting:

- Option 1: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level.

- Option 2: Minimum power boosting level in core specification together with manufacturer declaration in the conformance test specification.

Similar to NB-IoT, we believe a minimum power boosting level is valuable to be specified in specification to ensure at least the min value can be achieved when this feature is optionally declared by BS. If no value will be specified, then actually RAN4 do not need to discuss this power boosting feature.

Proposal 1: RAN4 should define a min power boosting level in core specification which is based on optional manufacture declaration.

For LP-WUS dynamic range definition, the simulation assumption in RAN1 is the EPRE ratio between LP-WUS and NR signal, hence, we think the LP-WUS power boosting definition should consider both option 1 and option 2 listed in WF.

## 1.2. Concept of LP-WUS dynamic range/power boosting

WF

FFS on the following options for next meeting:

- Option 1: Stick with the one in RAN4#110bis approved WF R4-2406140, which is:

  - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

- Option 2: Define EPRE ratio between LP-WUS and NR signals instead of power dynamic range.

Proposal 2: RAN4 LP-WUS power boosting definition can reuse the NR RB power dynamic range as much as possible meanwhile consider the fact that the assumption in RAN1 of 3dB or 6dB means the EPRE ratio.

Based on companies’ analysis, the power boosting will be more difficult in a small CBW. However, if the power boosting is specified as optional feature based on BS declaration, then seems there is no need to preclude any CBW, whether small CBW can be supported or not depends on gNB implementation.

## 1.3. Whether to preclude small CBW for consideration of LP-WUS power boosting

WF

FFS on the following options for next meeting:

- Option 1: Consider a power degradation limit, e.g., 2dB, for validating a configuration for LP-WUS representing by (EPRE ratio, channel bandwidth).

- Option 2: Only consider LP-WUS power boosting for CBWs larger than 20MHz.

- Option 3: Focus on CBWs >= 10MHz and different power boosting values can be considered for different CBW.

- Option 4: Not preclude small CBW for LP-WUS power boosting.

Proposal 3: There is no need to preclude any CBW.

For min value for LP-WUS power boosting, how much power can be boosted for LP-WUS depends on analyzed impacts on NR performance. It is encouraged that companies can evaluate the NR throughput loss vs in-band LP-WUS boosting, then the min value can be decided based on a reasonable NR degradation value.

## 1.5. On minimum value for LP-WUS power boosting

WF

FFS on the following options for next meeting:

- Option 1: Use [3dB] as starting point

  - It should be considered in conjunction with the supported CBWs.

- Option 2: To consider the power degradation of RBs other than LP-WUS signal within the carrier after the number of LP-WUS RBs have been decided in RAN1.

Proposal 4: RAN4 should evaluate the NR throughput impact due to in-band LP-WUS power boosting. The min value can be decided based on reasonable threshold of NR performance degradation.

- Conclusion

In this contribution, we share our views on LP-WUR BS RF:

Proposal 1: RAN4 should define a min power boosting level in core specification which is based on optional manufacture declaration.

Proposal 2: RAN4 LP-WUS power boosting definition can reuse the NR RB power dynamic range as much as possible meanwhile consider the fact that the assumption in RAN1 of 3dB or 6dB means the EPRE ratio.

Proposal 3: There is no need to preclude any CBW.

Proposal 4: RAN4 should evaluate the NR throughput impact due to in-band LP-WUS power boosting. The min value can be decided based on reasonable threshold of NR performance degradation.

# References

- R4-2409956, Way Forward for [111][314] NR_LPWUS, Huawei, RAN4#111, May 2024.