3GPP TSG-RAN WG4 Meeting #112 	R4-2412063

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on the deployment and spectrum usage of AIoT

Agenda Item:	8.20.2.1

Document for:	Approval

- Introduction

In this contribution, we further provide our views on the deployment and spectrum usage of AIoT.

- Discussion

  - Deployment

In the last meeting, the following agreement was achieved for D1T1[1]:

- Consider only adjacent RB/channel co-existence evaluation for in-band deployment scenario for NR and AIOT

- Encourage companies to provide the simulation results for option 1-1 and 1-2

  - FFS on co-site scenario (option 2-1 and 2-2)

For the option 2-1 and 2-2, which AIoT BS share same hardware and co-located with legacy NR BS, we already have an agreement that these two options will be studied after option 1-1 and option 1-2. In the last meeting, 12 cases are approved for co-existence study for option 1-1 and option 1-2:

| deployment scenario and topology |  | Evaluation case No. | (Aggressor -> Victim) | spectrum | Priority | note |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | D1T1-A2-legacy UE only outdoor | 1 | device -> NR UL | R2D: DL CW2D and D2R: UL | High | D2R |  |
|  |  | 2 | NR UL -> reader |  |  | D2R |  |
|  |  | 3 | reader -> NR DL |  |  | R2D |  |
|  |  | 4 | NR DL -> device |  |  | R2D |  |
|  |  | 5 | device -> NR DL | R2D: DL CW2D and D2R: DL |  | D2R |  |
|  |  | 6 | NR DL -> reader |  |  | D2R |  |
| 2 | D1T1-A2-legacy UE indoor | 7 | device -> NR UL | R2D: DL CW2D and D2R: UL | High | D2R |  |
|  |  | 8 | NR UL -> reader |  |  | D2R |  |
|  |  | 9 | reader -> NR DL |  |  | R2D |  |
|  |  | 10 | NR DL -> device |  |  | R2D |  |
|  |  | 11 | device -> NR DL | R2D: DL CW2D and D2R: DL |  | D2R |  |
|  |  | 12 | NR DL -> reader |  |  | D2R |  |

It is noted that only device1 and D1T1-A2 is used for calibration purpose, and we still need more time to discuss detailed assumptions and take different device type and scenarios into consideration. Considering the target date of finishing SI is in December this year, the workload of evaluating option 1-1 and 1-2 is already quite heavy.

Observation 1: The workload of current co-existence evaluation is already heavy, and there may not be enough time to complete the evaluation of option 2-1 and 2-2 in SI.

Proposal 1:  During SI stage, focusing on finishing the co-existence platform calibration and the evaluation of option 1-1 and option 1-2.

  - Spectrum usage

In [1], we agreed that:

Issue 2-2-1: Spectrum usage for R2D in D1T1

- Use FDD DL as starting point for co-existence evaluation for R2D in D1T1

  - FFS on FDD UL spectrum.

For cases that R2D in UL spectrum, there are mainly two additional cases need to be evaluated:

- Reader(R2D)NR UL

- NR UL  device(R2D)

Based on RAN1 assumption, the output power of AIoT BS will be restricted to 23 dBm. In this stage, the R2D interferer NR UL is similar to NR UL interferer NR UL, which is no need to be evaluated. For the NR UL interferer R2D, if the UE is outdoor, the interference should be ignored considering the O2I penetration loss and pathloss, so only the indoor NR UL interferer device need to be studied.

Proposal 2: For R2D in UL spectrum, only the case that indoor NR UL interferer device need to be evaluated.

For the spectrum usage of CW, the mainly reaming issue is whether and how to study the case that CW is outside topology.

- Further discuss the difference of outside topology (case 1-4, case2-4) from co-existence study perspective.

The CW distribution mainly impacts on the signal power of D2R and corresponding interference. In our understanding, when CW outside topology, it is easier to change the location and number of CW node to provide better D2R power level while reduce the interference and how to deploy the CW node depends on the implementation. One possible problem is the self-interference. When the CW is outside topology and near to the reader, the interference of CW may hard to be cancelled when reader receive D2R.

Observation 2: When the CW is outside topology and near the reader, the interference of CW may hard to be eliminated when reader receive D2R.

To provide enough coverage and avoid severer interference, the CW network can have a grid shift with reader.

Proposal 3: Take the following CW distribution assumption for co-existence study when CW is outside topology.

Another issue is case 2-3 for D2T2

Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum

- FFS on case 2-3

For D1T1, both CW in UL spectrum and DL spectrum are evaluated, but for D2T2, only CW in UL spectrum is agreed in the co-existence simulation. In RAN1, both UL and DL CW spectrum usage are studied for D1T1 and D2T2, so if case 2-3 is not studied, there will mismatch between RAN1 and RAN4 and the study is not sufficient

Observation 3: The co-existence study is incomplete if CW in DL spectrum cases of D2T2 is not evaluated.

Proposal 4: Once the CW distribution assumption is concluded, the case 2-3 for D2T2 should be evaluated.

- Conclusion

Observation 1: The workload of current co-existence evaluation is already heavy, and there may not be enough time to complete the evaluation of option 2-1 and 2-2 in SI.

Proposal 1:  During SI stage, focusing on finishing the co-existence platform calibration and the evaluation of option 1-1 and option 1-2.

Proposal 2: For R2D in UL spectrum, only the case that indoor NR UL interferer device need to be evaluated.

Observation 2: When the CW is outside topology and near the reader, the interference of CW may hard to be eliminated when reader receive D2R.

Proposal 3: Take the following CW distribution assumption for co-existence study when CW is outside topology.

Observation 3: The co-existence study is incomplete if CW in DL spectrum cases of D2T2 is not evaluated.

Proposal 4: Once the CW distribution assumption is concluded, the case 2-3 for D2T2 should be evaluated.

- References

- R4-2410567, WF on co-existence evaluation for ambient IoT, CMCC, RAN4#111