3GPP TSG-RAN WG4 Meeting # 112 	R4-2412064

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on the co-existence of AIoT

Agenda Item:	8.20.2.2

Document for:	Approval

- Introduction

In the last meeting, the simulation assumption for co-existence calibration was approved [1], and following cases need to be evaluated:

| deployment scenario and topology |  | Evaluation case No. | (Aggressor -> Victim) | spectrum | Priority | note |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | D1T1-A2-legacy UE only outdoor | 1 | device -> NR UL | R2D: DL CW2D and D2R: UL | High | D2R |
|  |  | 2 | NR UL -> reader |  |  | D2R |
|  |  | 3 | reader -> NR DL |  |  | R2D |
|  |  | 4 | NR DL -> device |  |  | R2D |
|  |  | 5 | device -> NR DL | R2D: DL CW2D and D2R: DL |  | D2R |
|  |  | 6 | NR DL -> reader |  |  | D2R |
| 2 | D1T1-A2-legacy UE indoor | 7 | device -> NR UL | R2D: DL CW2D and D2R: UL | High | D2R |
|  |  | 8 | NR UL -> reader |  |  | D2R |
|  |  | 9 | reader -> NR DL |  |  | R2D |
|  |  | 10 | NR DL -> device |  |  | R2D |
|  |  | 11 | device -> NR DL | R2D: DL CW2D and D2R: DL |  | D2R |
|  |  | 12 | NR DL -> reader |  |  | D2R |
| 3 | D2T2-A2-legacy UE only outdoor | 13 | device -> NR UL | R2D: UL CW2D and D2R: UL | High | D2R |
|  |  | 14 | NR UL -> reader |  |  | D2R |
|  |  | 15 | reader -> NR UL |  |  | R2D |
|  |  | 16 | NR UL -> device |  |  | R2D |
| 4 | D2T2-A2-legacy UE indoor | 17 | device -> NR UL | R2D: UL CW2D and D2R: UL | High | D2R |
|  |  | 18 | NR UL -> reader |  |  | D2R |
|  |  | 19 | reader -> NR UL |  |  | R2D |
|  |  | 20 | NR UL -> device |  |  | R2D |

In this contribution, we bring some preliminary simulation results and further discuss these assumptions.

- Discussion

  - Baseline SINR in D1T1

In [1], the following layout was agreed for D1T1

| BS deployment / Intermediate UE dropping | For D1T1-A2: 18 BSs on a square lattice with spacing D, located D/2 from the walls. - L=120m x W=60m; D=20m - BS height = 8 m For D1T1-A1: - FFS on layout: one node as transmission and CW, the neighbor node as reception |
| --- | --- |

Following the traditional way in the co-existence evaluation, all 18 BS will be activated in one snapshot. However, due to the higher radiated power assumption, the intra-system interference in D1T1 is quite severe, and the baseline SINR of device and reader are shown in Figure 1.

Figure 1 Baseline SINR for reader and device in D1T1

Based on our study, to achieve 10% BLER, reader require about -15 dB SNR and device require about 15 dB. If take these two values as reference, we can find that about 15% reader’s SINR is lower than -15dB which may hard to guarantee a feasible miss-detection rate and the 95% devices cannot work. In addition, the due the large ICI power, the impact of inter-system will hard to be observed which make the co-existence evaluation become meaningless.

Observation 1: When assuming all 18 AIoT BS in D1T1 are activated, the intra-system interference is quite significant, which make the AIoT system cannot work properly and the impact of inter-system interference is also hard to be observed.

A quick idea to reduce the intra-system interference is to reduce the number of activated AIoT BS in one snapshot, but in our understanding, the main advantage of D1T1 is that it can provide better coverage and inventory efficiency, and if the activated rate is too low, such advantage seems to be doubtful.

Observation 2: Compared to D2T2, the advantage of D1T1 is better coverage and inventory efficiency, and if the activation rate of reader in a snapshot it super low, such advantage will be doubtful.

Another problem is that we never assume the coordination between BS, which means we cannot avoid different readers active simultaneously.

Observation 3: Coordination between different readers is not a baseline assumption, which means we cannot always avoid different readers active simultaneously.

Based on the analysis above, 1/3 activation rate can be the starting point in D1T1.

Proposal 1: For D1T1, at least 1/3 readers are activated in one snapshot is assumed.

  - Reduce unnecessary cases in D2T2

For D2T2-A2:

The intermediate UEs selected from the fixed positions.

Number of intermediate UE for calibration:

- Option 1: 2 UE at one drop

- Option 2: 12 UE at one drop

In [1], two options of reader activation are listed:

As analysis in section 2.1, all reader activated in one snapshot will bring severe intra-system interference, so only option 1 is the feasible assumption.

Proposal 2: For D2T2, only take option 1, i.e., 2 readers are activated in one snapshot, as the evaluation assumption.

Based on the option 1, the SINR degradation results of case 13~20 are listed below:

Based on the simulation, for the cases that NR UE outdoor, all cases do not have obvious SINR degradation since the all the interference will suffer from the O2I loss.

Observation 4: For the NR UE is outside of factory, all cases show no obvious SINR degradation.

Since in this scenario 80% NR UEs are located in the factory, the interference from NR is more significant, especially for NR interferer AIoT reader/device. Some degradation also appear in Case 19 and One of the reasons here is that a normal NR UE will follow the power control but the intermediate UE always transmit in its maximum output power no matter where it is located, which may harm on the NR BS reception but the impact seems also can be ignored.

Observation 5: For the NR UE is inside of factory, case 17 and case 19 can be considered as no SINR degradation.

Proposal 2: To save efforts, the following cases of D2T2 can be marked as no co-existence issue and no need to further evaluate

| 3 | D2T2-A2-legacy UE only outdoor | 13 | device -> NR UL | R2D: UL CW2D and D2R: UL | High | D2R |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 | NR UL -> reader |  |  | D2R |
|  |  | 15 | reader -> NR UL |  |  | R2D |
|  |  | 16 | NR UL -> device |  |  | R2D |
| 4 | D2T2-A2-legacy UE indoor | 17 | device -> NR UL | R2D: UL CW2D and D2R: UL | High | D2R |
|  |  | 19 | reader -> NR UL |  |  | R2D |

  - Update of simulation assumption

As shown in section 2.2, the performance of AIoT system suffer from significant degradation when the NR UE inside the factory, and the main reason here is that we assume 80% UE are dropped into the factory, which may not be a practical assumption. It is suggested to lower down this ratio in the evaluation.

Proposal 3: it is suggested to modify the NR UE indoor ratio from 80% to 10%

  - Impact of CW for NR

Except the co-existence between NR and AIoT signal, the impact of CW also needs to be studied. Since single tone CW is agreed in RAN1, we can take narrow band blocking as a reference.

| NR band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3 | 5 | 10 | 15 | 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n31, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n70, n71, n72, n74, n75, n76, n85, n100, n101, n106 | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |
|  |  | dB | 18 | 16 | 13 | 14 | 16 | 16 | 16 |
|  | Puw (CW) | dBm |  | -55 |  |  |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 4 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS | NA |
|  | Fuw (offset SCS= 30 kHz)4 | MHz | NA |  |  |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1a, Table 7.3.2-1b and Table 7.3.2-2 for two and four antenna ports, respectively. NOTE 4: Fuw shall be rounded to half of SCS. NOTE 5: For SDL bands, requirements shall be applied only for CA band combination cases. |  |  |  |  |  |  |  |  |  |

-55 dBm interference power is defined which means if the CW interference is higher than this value, the UE performance is not guaranteed.

Observation 6: based on the narrow band blocking requirement, if the CW interference is higher than -55dBm, the UE performance is not guaranteed.

Since we already have the output power of reader, there will be a minimum distance between reader and NR UE to avoid higher CW interference. Taking D2T2 as an example, the InH pathloss model are listed below:

| InH - Office | LOS |  |  |  |
| --- | --- | --- | --- | --- |
|  | NLOS |  |  |  |
|  |  | Optional |  |  |

Based on the 23 dBm CW power and 1.5m height of reader and using NLOS model, 41m (2D distance) can be concluded to ensure the CW interference less than -55 dBm.

For D1T1, the channel model should be replaced by inF, and based on similar calculation, 340m isolation distance is required when the antenna gain is not counted.

Observation 7: Based on the calculation, 41m and 340m isolation distance between CW node and NR can ensure the CW interference lower than -55 dBm.

Proposal 4: To avoid the impact from CW, an isolation distance between activated CW node and indoor NR UE can be defined. For D2T2, 41m can be the starting point, For D1T1, further discuss whether it is feasible to allow NR UEs locate inside factory.

- Conclusion

Observation 1: When assuming all 18 AIoT BS in D1T1 are activated, the intra-system interference is quite significant, which make the AIoT system cannot work properly and the impact of inter-system interference is also hard to be observed.

Observation 2: Compared to D2T2, the advantage of D1T1 is better coverage and inventory efficiency, and if the activation rate of reader in a snapshot it super low, such advantage will be doubtful.

Observation 3: Coordination between different readers is not a baseline assumption, which means we cannot always avoid different readers active simultaneously.

Proposal 1: For D1T1, at least 1/3 readers are activated in one snapshot is assumed.

Proposal 2: For D2T2, only take option 1, i.e., 2 readers are activated in one snapshot, as the evaluation assumption.

Observation 4: For the NR UE is outside of factory, only case 15 shows the obvious SINR degradation.

Observation 5: For the NR UE is inside of factory, only case 17 can be considered as no SINR degradation.

Proposal 2: To save efforts, the following cases of D2T2 can be marked as no co-existence issue and no need to further evaluate

| 3 | D2T2-A2-legacy UE only outdoor | 13 | device -> NR UL | R2D: UL CW2D and D2R: UL | High | D2R |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 | NR UL -> reader |  |  | D2R |
|  |  | 15 | reader -> NR UL |  |  | R2D |
|  |  | 16 | NR UL -> device |  |  | R2D |
| 4 | D2T2-A2-legacy UE indoor | 17 | device -> NR UL | R2D: UL CW2D and D2R: UL | High | D2R |
|  |  | 19 | reader -> NR UL |  |  | R2D |

Proposal 3: it is suggested to modify the NR UE indoor ratio from 80% to 10%

Observation 6: Based on the narrow band blocking requirement, if the CW interference is higher than -55dBm, the UE performance is not guaranteed.

Observation 7: Based on the calculation, 41m and 340m isolation distance between CW node and NR can ensure the CW interference lower than -55 dBm.

Proposal 4: To avoid the impact from CW, an isolation distance between activated CW node and indoor NR UE can be defined. For D2T2, 41m can be the starting point, For D1T1, further discuss whether it is feasible to allow NR UEs locate inside factory.

- References

- R4-2410567, WF on co-existence evaluation for ambient IoT, CMCC, RAN4#111