3GPP TSG-RAN WG4 Meeting #112 	R4-2412065

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on the RF requirement of AIoT BS

Agenda Item:	8.20.3.1

Document for:	Approval

- Introduction

In this contribution, we further provide our views on the RF requirements for AIoT BS.

- Discussion

  - Power boosting of AIoT BS

Issue 2-1-1: deployment scenarios for D1T1

Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors.

Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB

Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware

Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting)

Agreement:

- RAN4 to first evaluate co-existence for deployment scenario of option 1-1 and 1-2, and further study option 2-1 and 2-2.

Issue 2-3-2: Priorities of spectrum deployment mode for co-existence evaluation

Agreement:

- Prioritize the following spectrum deployment mode for RAN4 co-existence evaluation

  - A-IoT is located within a NR transmission bandwidth configuration

  - A-IoT which is operating indoor shares in-band spectrum with outdoor macro BS

In previous meeting, the following scenarios are prioritized:

The power boosting of R2D from BS side was mentioned in previous meetings, but in current prioritized scenario assumption, the AIoT BS only transmit AIoT signal and does not share hardware with legacy NR, and it is not clear what the power boosting mean, because if we need higher power, it is enough to discuss the MOP requirement of BS directly.

Proposal 1: The definition of power boosting should be clarified first if AIoT BS does not share hardware with legacy NR.

If the power boosting is needed for the deprioritized scenario, e.g., option 2-1 and 2-2, our preference is that such work can be trigger after the co-existence evaluation of these case are finish and also depends on the RAN1 coverage study.

Proposal 2: For the case that AIoT BS and legacy NR share same hardware, the power boosting related work in RAN4 can be started after the co-existence evaluation and RAN1 coverage study.

  - Spectrum usage and RF requirement

In the last meeting, the following agreement for AIoT BS requirement was achieved:

- The existing NR BS RF requirement framework can be used as starting point for A-IoT BS.

  - FFS on the detailed requirements.

Compared to existing NR BS, the main difference of AIoT BS is that such BS may transmit or receive signal in both UL and DL spectrum which depends on the spectrum usage. The RF requirements may be also impacted and some observations are listed below:

- From Tx perspective, AIoT BS can transmit higher power in DL spectrum but when BS transmit in UL spectrum, to avoid interference, the out power of BS may be restricted, e.g., 23 dBm which is similar to NR UE.

- From Rx perspective, AIoT BS need additional Rx chain when CW in DL spectrum, the receiver performance may be different than the case that receive in UL spectrum.

Observation 1: The RF requirement of AIoT BS will be impacted by different spectrum usage.

Proposal 3：It is suggested to discuss whether different sets of requirements need to be defined for AIoT BS based on different spectrum usage.

- Conclusion

Proposal 1: The definition of power boosting should be clarified first if AIoT BS does not share hardware with legacy NR.

Proposal 2: For the case that AIoT BS and legacy NR share same hardware, the power boosting related work in RAN4 can be started after the co-existence evaluation and RAN1 coverage study.

Observation 1: The RF requirement of AIoT BS will be impacted by different spectrum usage.

Proposal 3：It is suggested to discuss whether different sets of requirements need to be defined for AIoT BS based on different spectrum usage.

- References

- R4-2410597	WF on impacts of A-IoT on RF requirements	Huawei  RAN4#111