3GPP TSG-RAN WG4 Meeting #112 	R4-2412066

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on the RF requirement of AIoT device

Agenda Item:	8.20.3.2

Document for:	Approval

- Introduction

In the last meeting, several RF requirements are agreed as the starting point of RF requirement for AIoT device and it is encouraged to discuss the RF requirement for different device type separately. In this contribution, we further provide our views on which requirements need to be defined for different AIoT device.

- Discussion

  - Device 1

In RAN1, the following architecture was agreed for device 1 in SI:

Figure 1 Architecture for device 1

The device 1 is designed as very low power consumption and its target cost should be pretty low, but if there are too many requirements need to be verified, the costs will inevitably go up, so it is important to identify which requirement should be necessary rather than just reuse NR requirement framework.

Observation 1: It is important to identify which requirements are indeed necessary for device, which can help reduce the cost of device.

The Tx chain of device 1 is quite simple and we can find following observations:

Observation 2:  For device 1 from Tx requirement perspective:

- The device does not support power control, so power control related requirement is not needed, e.g., power control accuracy, minimum output power

- There is no LO in Tx chain, so LO related requirement is not needed, e.g., carrier leakage

- When the device is in OFF state, it will still reflect the incoming signal since the impedance is unmatched, so the transmit OFF power seems meaningless for such device

- The Tx intermodulation product is mainly produced by PA. Since there is no PA for device 1, such requirement seems also not needed.

For Rx requirements, we also have several observations:

Observation 2:  For device 1 from Rx requirement perspective:

- In the co-existence study, the receiver bandwidth of device 1 is assumed the same as system channel bandwidth, which means ACS/ASCS is not available.

- The RF envelope detection is used and there is no LNA and mixer are assumed, so the Rx intermodulation should not be important for such receiver.

Based on the analysis above, we propose:

Proposal 1: The following requirements are considered removed for device 1:

- Transmit OFF power

- Minimum output power

- Power control requirement

- Carrier leakage

- Tx intermodulation

- ACS

- ASCS

- Receiver intermodulation

  - Device 2a

The architecture for device 2a which is agreed in RAN1 is shown in below:

Figure 2 Architecture for device 2a

Compared to device 1, the LNA, PA, large frequency shifter (including mixer and LO) are assumed, so for this type of device, carrier leakage, Tx intermodulation, Receiver intermodulation can not be easily removed, and more studies are needed.

Proposal 2: The following requirements are considered removed for device 2a:

- Transmit OFF power

- Minimum output power

- Power control requirement

- ACS

- ASCS

  - Device 2b

In our understanding, the device 2b is more powerful than device 1/2a which can transmit signal by its own Tx chain and the requirement for NB-IoT can be the starting point.

Proposal 3: For device 2b, the RF requirement for NB-IoT can be considered as starting point.

  - Others

The is a typical metric for measuring transmits modulation accuracy and the EVM is measured in frequency domain for QAM signal in NR system. However, the OOK is a metric to show the modulation quality in time domain. From envelope detection perspective, the main factor can be the gap between ON symbol and OFF symbol, so the average power ratio between ON symbol and OFF symbol may can be a candidate. The final value may not need to be decided in SI, but it would be helpful if a clear definition can be agreed..

Observation 3: The traditional EVM define the modulation quality for QAM signal in frequency domain, but the OOK waveform need a metric in time domain.

Proposal 4: It is suggested to discuss how to define the modulation quality for OOK waveform in time domain in SI, e.g., the power ratio between ON symbol and OFF symbol.

- Conclusion

Observation 1: It is important to identify which requirements are indeed necessary for device, which can help reduce the cost of device.

Observation 2:  For device 1 from Tx requirement perspective:

- The device does not support power control, so power control related requirement is not needed, e.g., power control accuracy, minimum output power

- There is no LO in Tx chain, so LO related requirement is not needed, e.g., carrier leakage

- When the device is in OFF state, it will still reflect the incoming signal since the impedance is unmatched, so the transmit OFF power seems meaningless for such device

- The Tx intermodulation product is mainly produced by PA. Since there is no PA for device 1, such requirement seems also not needed.

Observation 2:  For device 1 from Rx requirement perspective:

- In the co-existence study, the receiver bandwidth of device 1 is assumed the same as system channel bandwidth, which means ACS/ASCS is not available.

- The RF envelope detection is used and there is no LNA and mixer are assumed, so the Rx intermodulation should not be important for such receiver.

Proposal 1: The following requirements are considered removed for device 1:

- Transmit OFF power

- Minimum output power

- Power control requirement

- Carrier leakage

- Tx intermodulation

- ACS

- ASCS

- Receiver intermodulation

Proposal 2: The following requirements are considered removed for device 2a:

- Transmit OFF power

- Minimum output power

- Power control requirement

- ACS

- ASCS

Proposal 3: For device 2b, the RF requirement for NB-IoT can be considered as starting point.

Observation 3: The traditional EVM define the modulation quality for QAM signal in frequency domain, but the OOK waveform need a metric in time domain.

Proposal 4: It is suggested to discuss how to define the modulation quality for OOK waveform in time domain in SI, e.g., the power ratio between ON symbol and OFF symbol.

- References

- R4-2410597	WF on impacts of A-IoT on RF requirements	Huawei