3GPP TSG-RAN WG4 Meeting #112 	R4-2412067

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on the RF requirement of intermediate UE

Agenda Item:	8.20.3.3

Document for:	Approval

- Introduction

In this contribution, we further provide our views on the RF requirements for intermediate UE.

- Discussion

  - Relationship between NR and AIoT requirement for intermediate UE

In the last meeting, the following table is agreed as starting point for intermediate UE:

| RF Requirement for A-IoT intermediate UE |  |  |  |
| --- | --- | --- | --- |
| TX requirement | Maximum output power | RX requirement | Reference sensitivity |
|  | Output power dynamics |  | Maximum input power |
|  | Transmit ON/OFF power |  | ICS |
|  | Transmitted signal quality |  | ACS |
|  | Transmission times |  | In-band blocking |
|  | Occupied bandwidth |  | Out-of-band blocking |
|  | Tx intermodulation |  | Receiver intermodulation |
|  | ACLR |  | Rx spurious emission |
|  | Operating band unwanted emissions |  |  |
|  | Transmitter spurious emissions |  |  |

Except ICS, the other requirements are also defined in NR system. For a UE support both NR and AIoT operation, if some requirements are already verified in NR which is already show a more powerful performance of hardware, it may not be necessary to check it again from AIoT requirement perspective.

Observation 1: The NR system require a more powerful hardware compared to AIoT system, so for a intermediate UE supports both AIoT and NR, if some NR requirements are verified and show enough performance, it may not be necessary to check it again from AIoT requirement perspective.

Proposal 1: It is suggested to identify which requirements of AIoT can be waived if they are already verified in NR for intermediate UE.

Another issue related to the proposal above is that both UL and DL spectrum are possibly used for CW. When CW is outside topology and in the DL spectrum, the UE can receive D2R signal in DL spectrum which is similar to NR and OFDM receiver may be shared between NR and AIoT, but when the CW is inside topology and in the UL spectrum, intermediate UE need an additional Rx chain in UL, and the reception performance may be different when CW in UL and DL

Observation 2: When the CW is in UL spectrum, an additional Rx chain in UL is needed which may have impact on the requirement design.

Proposal 2: It is suggested to discuss the UE architecture assumption when CW is in the UL spectrum, and whether different requirements should be defined for different spectrum usage of CW.

  - Simultaneous Tx/Rx for NR and AIoT link

An underdiscussed remaining issue is whether new requirements are needed if the intermediate UE transmit/receive NR and AIoT signal simultaneously. In our understanding, before we jump into the requirement discussion, it is still not clear whether such condition indeed exist which also depends on RAN1 discussion, so we prefer postpone the requirement discussion of such condition and wait for more information from RAN1.

Proposal 3: The requirements of intermediate UE to transmit or receive NR and AIoT signal simultaneously are not considered in SI unless there are clear feedback from RAN1.

  - CW node

In the last meeting, we agreed that:

- To further investigate output power, emission requirements for CW node

  - FFS for other requirements.

In the SI stage, we may only focus on what requirements should be defined and leave the details into WI stage. For the CW node, maximum output power and spurious emission requirement are needed, and the NR requirement can be the starting point.

Proposal 4: For the maximum output power and spurious emission of CW, the NR requirement can be the starting point.

From D2R receive perspective, the phase noise of CW may also need to be guaranteed. When the phase noise is worse, the noise will overlap with D2R spectrum, which impact on the D2R performance, so it is worth to discuss whether phase noise of CW should be restricted by requirement.

Observation 3: The phase noise of CW may overlap with D2R which impact on the D2R receive performance.

Proposal 5: It is suggested to discuss whether the phase noise of CW need to be restricted by RF requirement.

- Conclusion

Observation 1: The NR system require a more powerful hardware compared to AIoT system, so for a intermediate UE supports both AIoT and NR, if some NR requirements are verified and show enough performance, it may not be necessary to check it again from AIoT requirement perspective.

Proposal 1: It is suggested to identify which requirements of AIoT can be waived if they are already verified in NR for intermediate UE.

Observation 2: When the CW is in UL spectrum, an additional Rx chain in UL is needed which may have impact on the requirement design.

Proposal 2: It is suggested to discuss the UE architecture assumption when CW is in the UL spectrum, and whether different requirements should be defined for different spectrum usage of CW.

Proposal 3: The requirements of intermediate UE to transmit or receive NR and AIoT signal simultaneously are not considered in SI unless there are clear feedback from RAN1.

Proposal 4: For the maximum output power and spurious emission of CW, the NR requirement can be the starting point.

Observation 3: The phase noise of CW may overlap with D2R which impact on the D2R receive performance.

Proposal 5: It is suggested to discuss whether the phase noise of CW need to be restricted by RF requirement.

- References

- R4-2410597	WF on impacts of A-IoT on RF requirements	Huawei