3GPP TSG-RAN WG4 Meeting # 112 	R4-2412068

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on the UE parameter for 7125 to 8400 MHz

Agenda Item:	8.2.3

Document for:	Approval

- Introduction

In the previous meeting, most UE requirements for 7125 to 8400 MHz are discussed [1][2] but there still are several remaining issues and in this contribution, we provide our views on these issues.

- Discussion

- Typical signal bandwidth

Option 1: 273 RB, 30k SCS as 38.104

Option 2: Quote formula of RBs * SCS without number of RBs

Option 3: Directly quote the signal bandwidth in MHz

In [2], there options are recorded:

In [3], the signal bandwidth for 4400-4800MHz is written as: Signal bandwidth = NRB x SCS x 12 and the NRB and SCS is refer to the TS38.101-1. Since we already agreed that mention 100MHz as typical channel bandwidth, similar way also works for 7125-8400.

Proposal 1: For the signal bandwidth, similar format as 4400-4800 can be used, i.e., Signal bandwidth = NRB x SCS x 12 and the NRB and SCS is refer to the TS38.101-1.

- Maximum output power

In [2], a tentative agreement is listed:

- Capture 23dBm as typical maximum output power in the LS to ITU.

  - Other output powers are not precluded and refer to TR

- Capture 20dBm, 23dBm, 26dBm, and higher power classes in TR

In our understanding at least the 23 dBm need to be supported as typical value, and it doesn’t mean other powers are precluded if they are not mentioned in reply LS. Higher power or lower power can be discussed in the future release when there is a dedicated WI exist.

Proposal 2: In the reply LS, only provide 23dBm as typical value for MOP. Other powers are not precluded and can be recorded in TR.

- ACLR/ACS

In [1], there are several options for ACLR and ACS

Issue 2-6 ACLR

- UE

  - Option 1:  26dB, 27dB (study) for PC3

  - Option 2:  30dB (n104) for PC3, 31dB (n104) for PC2

Issue 2-12 ACS

- UE:

Follow n104 or follow previous studies

In [4], 26 dBc ACLR and 32 dBc ACS is decided based on the co-existence study for 7GHz. The UE can benefit from lower ACLR to reduce the MPR and achieve higher output power, which is also good for the coverage in such high frequency band, so we prefer use the results in the TR as the baseline.

Observation 1: UE can benefit from lower ACLR to reduce the MPR and a achieve better UL performance, which is also good for the coverage.

Proposal 3: Use the co-existence outcome from TR 38.921 as the ACLR/ACS value, i.e., 26 dB ACLR/32dB ACS for UE in 7125-8400MHz .

- Noise figure

In [1], the options for noise figure are listed below:

Issue 2-9 Noise Figure

- UE

  - Option 1: Follow n104 noise figure (12dB)

  - Option 2: Be consistent with information sent previously IMT-2020 28GHz, e.g. 10dB

  - Option 3: Be consistent with Previous LS to ITU-R on 6, 10GHz, NF was 9-13dB

From the co-existence evaluation perspective, the 9 dB NF was used but based on the component link analysis, 12 dB is used for n104. Since the reason behind the values are different, it is hard to further down selection from them, so provide a range can be a possible compromise.

Observation 2: There are different reasons behind each NF value, it is hard to do the down selection.

Proposal 4: Provide a range of NF in the reply LS, e.g., 9-13 dB.

- SINR operation range

In [3], a whole page is used to describe the SINR operation range which actually from co-existence assumption. Since all these contents are based on AWGN link level performance and should be frequency-agnostic, same description can be used for 7125-8400MHz

Proposal 5: The SINR operation range in 4400-4800MHz LS can be used for 7125-8400MHz.

- Sensitivity

In [2], we agreed that 100MHz is mentioned as typical channel bandwidth, to match with this, the REFSENS of 100MHz in n104 can be used.

Proposal 6: To match with 100MHz typical channel bandwidth, the REFSENS of 100MHz in n104 is used in reply LS.

- Blocking response

In current TS 38.101-1, the blocking requirement for band n104 is already defined which can be used in LS.

Proposal 7: For the UE blocking response in 7125-8400MHz, reuse the requirement of n104.

- Conclusion

In this contribution, we provide our views on the UE parameter for 7125 to 8400 MHz frequency range, and our proposals are listed below:

Proposal 1: For the signal bandwidth, similar format as 4400-4800 can be used, i.e., Signal bandwidth = NRB x SCS x 12 and the NRB and SCS is refer to the TS38.101-1.

Proposal 2: In the reply LS, only provide 23dBm as typical value for MOP. Other powers are not precluded and can be recorded in TR.

Observation 1: UE can benefit from lower ACLR to reduce the MPR and a achieve better UL performance, which is also good for the coverage.

Proposal 3: Use the co-existence outcome from TR 38.921 as the ACLR/ACS value, i.e., 26 dB ACLR/32dB ACS for UE in 7125-8400 MHz.

Observation 2: There are different reasons behind each NF value, it is hard to do the down selection.

Proposal 4: Provide a range of NF in the reply LS, e.g., 9-13 dB.

Proposal 5: The SINR operation range in 4400-4800MHz LS can be used for 7125-8400MHz.

Proposal 6: To match with 100MHz typical channel bandwidth, the REFSENS of 100MHz in n104 is used in reply LS.

Proposal 7: For the UE blocking response in 7125-8400MHz, reuse the requirement of n104.

- References

- R4-2406615, WF on IMT parameters for other frequency ranges, CATT, RAN4#110bis

- R4-2410741, WF on IMT parameter study, Ericsson, RAN4#111

- R4-2410576 Draft LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27

- 3GPP TR 38.921