3GPP TSG-RAN WG4 Meeting # 112 	R4-2412069

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on the co-existence for 14800 to 15350 MHz

Agenda Item:	8.2.4.1

Document for:	Approval

- Introduction

In the last meeting, the co-existence assumption for 14800 to 15350 MHz was agreed and, in this contribution, we provide our preliminary evaluation results.

- Discussion

In the agreed simulation assumption[1], some parameters have several options, and in this contribution, we only choose one set of them to perform the evaluation, the detailed assumptions can be found in Annex.

  - Urban macro

For the Uma, the simulation results are shown in below:

Due to spatial isolation of beamforming, the throughput loss of FR2-like UE is smaller than FR1-like UE in same scenario. The bottle neck of Uma is DL throughput loss in cell edge, which require about 18 dB ACIR for FR2-like UE and 22dB for FR1-like UE.

Observation 1: For Urban macro, 18dB ACIR for FR2-like UE and 22dB for FR1-like UE are required.

2.2 Indoor office

For Indoor office, the dominated scenario is average DL throughput loss. 15dB ACIR and 13dB ACIR are needed for FR1-like and FR2-like UE respectively.

Observation 2: For Indoor office, 13dB ACIR for FR2-like UE and 15dB for FR1-like UE are required.

Based on the simulation, the maximum required ACIR for FR2-like UE is similar to TR38.803 and for FR1-like UE, the required ACIR is similar to TR38.921, so in our view, corresponding ACLR and ACS can be reused.

Proposal: For FR1-like UE, the ACLR/ACS in TR38.921 is reused. For FR2-like UE, the ACLR/ACS in TS38.101-2 can be reused.

- Conclusion

Observation 1: For Urban macro, 18dB ACIR for FR2-like UE and 22dB for FR1-like UE are required.

Observation 2: For Indoor office, 13dB ACIR for FR2-like UE and 15dB for FR1-like UE are required.

Proposal: For FR1-like UE, the ACLR/ACS in TR38.921 is reused. For FR2-like UE, the ACLR/ACS in TS38.101-2 can be reused.

- References

- R4-2410592 TP to TR 38.922: System level simulation methodology and assumptions for coexistence study for 14800 - 15350 MHz frequency range Nokia

- Annex

| Parameters | Indoor | Urban macro |
| --- | --- | --- |
| Channel bandwidth | 200 MHz | 200 MHz |
| Scheduled channel bandwidth per UE (DL) | 200 MHz | 200 MHz |
| Scheduled channel bandwidth per UE (UL) | 200 MHz | 200 MHz |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | 1 UE per slot | 1 UE per slot |
| Traffic model | Full buffer | Full buffer |
| DL power control | NO | NO |
| UL power control | YES | YES |
| BS max TX power in dBm | 23dBm | 43dBm |
| UE max TX power in dBm | 23dBm | 23dBm |
| UE min TX power in dBm | -40dBm | -40dBm |
| BS Noise figure in dB | 16 | 8 |
| UE Noise figure in dB | 11 | 11 |
| Handover margin | 3dB | 3dB |
| Antenna sub-array configuration | 8 x 8 elements | 64 x 24 elements |
| Inter-site distance | 20 | 45 |
| Multi operators layout | coordinated | coordinated |