3GPP TSG-RAN WG4 Meeting # 112 	R4-2412070

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on the UE parameter for 14800 to 15350 MHz

Agenda Item:	8.2.4.2

Document for:	Approval

- Introduction

In the last meeting, the co-existence assumption for 14800 to 15350 MHz was agreed but the UE parameter is not decided yet, and related discussion point are recorded in [1]. In this contribution, we provide our views on these issues.

- Discussion

- Channel bandwidth

For FR2, 200MHz is mandatory and 400MHz is optional, so it is suggested that use 200MHz as a typical value.

Proposal 1: 200MHz is used as typical channel bandwidth for 14800 to 15350 MHz in the LS.

- UE form factor

Issue 3-1 Common understanding on UE antenna array options

- Option 1: “UE beamforming” (FR2 like)

  - Assumed to be FR2 like UE, with two panels similar to Rel-14 NR study 38.803

  - Analogue like beamforming

  - Option 1a: 2x2 antennas per panel

- Option 2: FR1 like

  - Assumed to be FR1 like UE with more than one TX/RX antenna

  - Isotropic antennas

  - Option 2a: 2 TX

  - Option 2b: 4 TX

  - Option 2c: 4 RX

  - Option 2d: 6 RX

  - Option 2e: 8 RX

In [1], two types of UE are agreed for co-existence evaluation:

It is quite controversial to decide the form factor of UE in this new frequency at this stage. In TR 38.820, similar discussion was happened but, in the end, both FR1-like and FR2-like are marked as possible in FR3. It is inefficient to take too much time on this topic.

Observation 1: It is inefficient to try to down-selection from different UE form factor since there no commercial UE is designed in this new frequency range at this stage.

To save effort, we prefer keep the door open and provide two set of parameters in the LS

Proposal 2: The UE parameters for both FR1-like and FR2-like are provided in the reply LS for 14800-15300MHz.

- Conclusion

Proposal 1: 200MHz is used as typical channel bandwidth for 14800 to 15350 MHz in the LS.

Observation 1: It is inefficient to try to down-selection from different UE form factor since there no commercial UE is designed in this new frequency range at this stage.

Proposal 2: The UE parameters for both FR1-like and FR2-like are provided in the reply LS for 14800-15300MHz.

- References

- R4-2410741, WF on IMT parameter study, Ericsson, RAN4#111